3GPP TSG CT WG1 Meeting #122						C1-200055

# Sophia Antipolis (France), 24-28 February 2020

# 3GPP TSG-RAN WG2 #108	R2-1916623

# Reno, Nevada, US, 18 – 22 November 2019

Title:	Reply LS on extended NAS timers for CE in 5GS

Response to:	R2-1916292/C1-199034

Release:	Release 16

Work Item:	LTE_eMTC5-Core, NB_IOTenh3-Core, 5G_CIoT

Source:	RAN2

To:	CT1

Cc:	RAN3, SA2

Contact Person:

#### Name:	Tuomas Tirronen

E-mail Address:	tuomas.tirronen@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 thanks CT1 for their LS on Extended NAS timers for Coverage Enhancement in 5GS. RAN2 would like to inform CT1 that there should be no differences in RAN delay for WB-N1 (eMTC) or NB-N1 (NB-IoT) using coverage enhancements in 5GS compared to EPS when the UE is either in RRC_IDLE or in RRC_CONNECTED or during state transitions between these states.

RAN2 would like to additionally inform CT1 that RAN2 has agreed WB-N1 UEs (eMTC UEs) in RRC_INACTIVE support eDRX up to 10.24s cycles without PTW. WB-N1 also supports coverage enhancements while in RRC_INACTIVE which may affect paging delay and the transition to RRC_CONNECTED state after paging. The maximum delay for state transition is 10s after sending UL RRC message (timer T300 in RRC). This may increase RAN delay compared to the existing delay with RRC_INACTIVE in eLTE, e.g., for delivery of downlink NAS PDU.

NB-N1 (NB-IoT) does not support RRC_INACTIVE when connected to 5GC.

2. Actions:

To CT1

RAN2 respectfully request CT1 to take the above information into account.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #109	2020-02-24 to 2020-02-28	Athens, Greece

TSG-RAN WG2 Meeting #109bis	2020-04-20 to 2020-04-24	TBD, Japan