3GPP TSG-CT WG1 Meeting #123-e	C1-202532

- Electronic meeting; 16-24 April 2020

Source:	MediaTek Inc.

Title:	Considering allowed NSSAI when requesting MA PDU session upgrade

Spec:	3GPP TS 24.193 v1.1.0

Agenda item:	16.2.5

Document for:	Agreement

1. Introduction

Based on the requirement in TS 23.501 subclause 5.32.2:

If the UE requests an S-NSSAI, this S-NSSAI should be allowed on both accesses. Otherwise, the MA PDU Session shall not be established.

and the network behavior specified in TS 23.502 subclause 4.22.6.3:

If the AMF determines that the UE is registered via both accesses in the same PLMN but the current S-NSSAI is not in the Allowed NSSAI for both accesses, the AMF shall reject the PDU session modification if the UE provides an "MA PDU Request" indication, or the AMF shall not forward "MA PDU Network-Upgrade Allowed" indication to the SMF if received from the UE.

the S-NSSAI provided by the UE shall be included in the allowed NSSAI of both access types in those scenarios, otherwise the request will be rejected by the network. This requirement is not well captured in the current procedure for “Converting PDU session transferred from EPS to MA PDU session” in TS 24.193.

2. Reason for Change

For reguesting MA PDU session upgrade after the UE performs inter-system change from S1 mode to N1 mode and has already registered over non-3GPP access in the same PLMN, the UE checks whether the S-NSSAI associated with PDU session of the 3GPP access is included in the allowed NSSAI of the non-3GPP access.

Otherwise, stage 2 requirement is not well captured in stage 3, and UE’s request will be reject by the network.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.193 v1.1.0.

* * * First Change * * * *

### 5.2.5	Converting PDU session transferred from EPS to MA PDU session

When an ATSSS capable UE has transferred a PDN connection from S1 mode to N1 mode in the network supporting N26 interface and the related URSP or UE local configuration does not mandate the PDU session shall be established over a single access:

a)	if the UE is registered over both 3GPP access and non-3GPP access in the same PLMN, and the S-NSSAI associated with the PDU session over 3GPP access is included in the allowed NSSAI of non-3GPP access, the UE may initiate the UE-requested PDU session modification procedure by sending the PDU SESSION MODIFICATION REQUEST message including 5GSM capability IE over 3GPP access as specified in clause 6.4.2.2 of 3GPP TS 24.501 [6]. The UE may set the Request type IE to either:

1)	"modification request" and include the MA PDU session information IE set to "MA PDU session network upgrade allowed" as specified in clause 9.11.3.63 of 3GPP TS 24.501 [6]; or

2)	"MA PDU request"

in the UL NAS TRANSPORT message as specified in clause 8.2.10 of 3GPP TS 24.501 [6]. When the UE receives the PDU SESSION MODIFICATION COMMAND message including the ATSSS container IE as specified in clause 6.4.2.3 of 3GPP TS 24.501 [6], the UE shall consider that the requested PDU session was converted by the network to an MA PDU session and user plane resources are established in both 3GPP access and non-3GPP access;

b)	if the UE is registered over both 3GPP access and non-3GPP access in different PLMNs, the UE may initiate the UE-requested PDU session modification procedure by sending the PDU SESSION MODIFICATION REQUEST message including 5GSM capability IE over 3GPP access as specified in clause 6.4.2.2 of 3GPP TS 24.501 [6]. The UE may set the Request type IE to either:

1)	"modification request" and include the MA PDU session information IE set to "MA PDU session network upgrade allowed" as specified in clause 9.11.3.63 of 3GPP TS 24.501 [6]; or

2)	"MA PDU request"

in the UL NAS TRANSPORT message as specified in clause 8.2.10 of 3GPP TS 24.501 [6]. When the UE receives the PDU SESSION MODIFICATION COMMAND message including the ATSSS container IE as specified in clause 6.4.2.3 of 3GPP TS 24.501 [6], the UE shall consider that the requested PDU session was converted by the network to an MA PDU session. The UE shall then initiate the UE-requested PDU session establishment procedure with the same PDU session ID, as specified in clause 6.4.1.2 of 3GPP TS 24.501 [6] over non-3GPP access, in order to establish user plane resources on the other access for the MA PDU session; or

c)	if the UE is registered over 3GPP access only, the UE may initiate the UE-requested PDU session modification procedure by sending the PDU SESSION MODIFICATION REQUEST message including 5GSM capability IE over 3GPP access as specified in clause 6.4.2.2 of 3GPP TS 24.501 [6], The UE may set the Request type IE to either:

1)	"modification request" and include the MA PDU session information IE set to "MA PDU session network upgrade allowed" as specified in clause 9.11.3.63 of 3GPP TS 24.501 [6]; or

2)	"MA PDU request"

in the UL NAS TRANSPORT message as specified in clause 8.2.10 of 3GPP TS 24.501 [6]. When the UE receives the PDU SESSION MODIFICATION COMMAND message including the ATSSS container IE as specified in clause 6.4.2.3 of 3GPP TS 24.501 [6], the UE shall consider that the requested PDU session was converted by the network to an MA PDU session. When the UE at a later point in time registers over the non-3GPP access, either in the same PLMN or in a different PLMN, the UE shall initiate the UE-requested PDU session establishment procedure with the same PDU session ID as specified in clause 6.4.1.2 of 3GPP TS 24.501 [6] over non-3GPP access in order to establish user plane resources on non-3GPP access for the MA PDU session.