3GPP TSG-CT WG1 Meeting #123-e	C1-202303

- Electronic meeting; 16-24 April 2020

Source:	Huawei, HiSilicon

Title:	Updates to MBMS bearer quality detection procedure

Spec:	3GPP TS 24.548 v1.0.0

Agenda item:	16.2.20

Document for:	Agreement

1. Introduction

This p-CR updates the MBMS bearer quality detection procedure for 3GPP TS 24.548.

2. Reason for Change

a)	Based on stage 2 in 3GPP TS 23.434 clause 14.3.2.4, the MBMS listening status report should be updated, quote:

#### 14.3.2.4	MBMS listening status report

Table 14.3.2.4-1 describes the information flow for the MBMS listening status report from NRM client to NRM server.

Table 14.3.2.4-1: MBMS listening status report

| Information element | Status | Description |
| --- | --- | --- |
| VAL user ID or VAL UE ID | M | The identity of the VAL user or VAL UE who wants to report the MBMS listening status. |
| TMGI(s) | M | TMGI(s) information. |
| MBMS listening status(s) | M | The MBMS listening status per TMGI. |
| MBMS reception quality level | O | The reception quality level per TMGI (see NOTE) |
| Unicast listening status | O | The unicast listening status. |
| NOTE: The set of quality levels helps service continuity in MBMS scenarios. A reception quality level may help to make an efficient switching decision to another bearer. How these levels are used is implementation specific. |  |  |

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.548 v1.0.0.

* * * First Change * * * *

##### 6.2.3.4.1	Client procedure

Upon determining the MBMS bearer quality, if the MBMS bearer quality reaches a certain threshold, the SNRM-C shall report the MBMS listening status. The SNRM-C:

NOTE 1:	The SNRM-C may determine the MBMS bearer quality by using the BLER of the received data. When no data is received, the quality estimation can consider the reference signals and the modulation and coding scheme (MCS). The UE may also use predictive methods to estimate the expected MBMS bearer quality (e.g. speed and direction) to proactively inform the NRM server of an expected loss of the MBMS bearer quality.

NOTE 2:	The threshold used to indicate MBMS bearer quality depends on VAL service type and the metrics used. The metrics used and the associated thresholds are out of scope of this specification.

NOTE 3:	The application/vnd.3gpp.seal-mbms-usage-info+xml can contain both the listening status "listening" and "not listening" at the same time.

a)	shall generate an application/vnd.3gpp.seal-mbms-usage-info+xml MIME body with an <mbms-listening-status-report> element in the <mbms-info> root element which:

1)	shall include an <identity> element set to the identity of the VAL user or VAL UE who wants to report the MBMS listening status;

2)		shall include an <mbms-listening-status> element set to "listening" if the SNRM-C is listening to the MBMS bearer or "not-listening" if the SNRM-C is not listening;

3)	shall include one or more <TGMI> elements for which the listening status applies; and

4)	may include an <mbms-reception-quality-level> element set to the reception quality level per TMGI; and

5)	if the <unicast-status> element is present in the MBMS announcement message, shall include an <unicast-listening-status> element set to "listening" or "not-listening" indicating the unicast listening status.

* * * End of Change * * * *