3GPP TSG-CT WG1 Meeting #123-e	C1-202304

- Electronic meeting; 16-24 April 2020

Source:	Huawei, HiSilicon

Title:	Structure and data semantics for MBMS bearer quality detection procedure

Spec:	3GPP TS 24.548 v1.0.0

Agenda item:	16.2.20

Document for:	Agreement

1. Introduction

This p-CR defines the structure and data semantics for the MBMS bearer quality detection procedure for 3GPP TS 24.548.

2. Reason for Change

The specification needs to define the structure and data semantics for the MBMS bearer quality detection procedure defined in 3GPP TS 23.434 clause 14.3.3.5.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.548 v1.0.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 24.547: "Identity management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification".

[4]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[5]	3GPP TS 24.486: "Vehicle-to-Everything (V2X) Application Enabler (VAE) layer; Protocol aspects; Stage 3".

[6]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[7]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[8]	3GPP TS 29.549: "Service Enabler Architecture Layer (SEAL); Application Programming Interface (API) specification".

[9]	IETF RFC 3428: "Session Initiation Protocol (SIP) Extension for Instant Messaging".

[10]	IETF RFC 3841: "Caller Preferences for the Session Initiation Protocol (SIP)".

[11]	IETF RFC 4566: "Session Description Protocol".

[12]	IETF RFC 4825: "The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".

[13]	IETF RFC 5795: "The Robust Header Compression (ROHC) Framework".

[14]	OMA OMA-TS-XDM_Group-V1_1_1-20170124-A: "Group XDM Specification".

[t24008]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

* * * NEXT Change * * * *

## 7.3	Structure

The network resource management document shall conform to the XML schema described in clause 7.4.

### 7.3.1	VALInfo document

The <seal-request-uri> element shall be the root element of the VALInfo document.

The <seal-request-uri> element shall include one of the following:

a)	a <VAL-user-id> element may include a <VAL-client-id> element; or

b)	a <VAL-group-id> element.

### 7.3.2	UnicastInfo document

The <unicast-info> element shall be the root element of the UnicastInfo document.

The <unicast-info> element shall include one of the following:

a)	a <request> element; or

b)	a <response> element.

The <request> element shall include the following:

a)	a <media-type> element;

b)	a <min-bandwidth> element;

c)	a <max-bandwidth> element; and

d)	an <application-identifier> element.

The <response> element shall include a <result> element.

### 7.3.3	MBMSInfo document

The <mbms-listening-status-report> element shall include:

a)	an <identity> element;

b)	a <TMGI> element;

c)	an <mbms-listening-status> element;

d)	an optional <mbms-reception-quality-level> elment; and

e)	an optional <unicast-listening-status> element.

* * * NEXT Change * * * *

## 7.5	Data semantics

### 7.5.1	VALInfo document

The <seal-request-uri> element is the root element of the XML document. The < seal-request-uri> element contains contains one of following sub-elements:

a)	<VAL-user-id>, an element contains the identity of the VAL user. This element contains an optional <VAL-client-id> attribute that contains the identity of the VAL client; or

b)	<VAL-group-id>, an element contains the group identity of a set of VAL users or VAL clients according to the VAL service.

The recipient of the XML ignores any unknown element and any unknown attribute.

### 7.5.2	UnicastInfo document

The recipient of the XML ignores any unknown element and any unknown attribute.

The <unicast-info> element is the root element of the XML document. The <unicast-info> element contains a <request> or <response> sub-element.

<request> is an optional element used to include the unicast resource management requested information. The <request> element contains the following sub-elements:

a)	<media-type>, an element contains the media type requested for the requested unicast resource;

b)	<min-bandwidth>, an element contains the minimum bandwidth of the requested unicast resource;

c)	<max-bandwidth>, an element contains the maximum bandwidth of the requested unicast resource; or

d)	<application-identifier>, an element contains the application identifier for the requested unicast resource;

<response> is an optional element used to include the unicast resource management response information. The <response> element contains the <result> sub-element. The <result> is a mandatory element which indicates a value "success" or "fail".

### 7.5.3	MBMSInfo document

<mbms-listening-status-report> element contains the following sub-elements:

a)	<identity>, an element contains the identity of the VAL user or VAL UE who wants to report the MBMS listening status;

b)	<TMGI>, an element contains the TMGI. The <TMGI> element is coded as described in 3GPP TS 24.008 [t24008] subclause 10.5.6.13 excluding the Temporary Mobile Group Identity IEI and Length of Temporary Mobile Group Identity contents (octet 1 and octet 2 in 3GPP TS 24.008 [t24008] subclause 10.5.6.13);

c)	<mbms-listening-status>, an element contains a string “listening” or “not-listening” used to indicate the MBMS listening status per TMGI;

d)	<mbms-reception-quality-level>, an optional element contains an integer used to indicate the reception quality level per TMGI; and

e)	<unicast-listening-status>, an optional element contains a string “listening” or “not-listening” used to indicate the unicast listening status.

* * * End of Change * * * *