3GPP TSG-CT WG1 Meeting #123-e	C1-202452

Electronic meeting, 16-24 April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.282 | CR | 0131 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Issue fixes in MCData pre-established session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData2 |  |  |  |  |  | Date: |  |  | 2020-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1) When the originating MCData client initiates a proceure to end the MCData commuication using pre-established call, it expects a SIP Re-INVITE from the Participating Function (PF), but as per current procedure the PF is not sending a Re-INVITE. 2) When the remote MCData client terminates the call – the PF needs to send a terminate request to the originating MCData client. But it sends terminated status. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Added a step in the PF – to send a ReINVITE to the originating MCData client upon receiving a BYE response. 2) Made a correction in the procedure to use terminate-request properly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MCData originating client will not be able to know whether the MCData communication using pre-established call is terminated successfully or not – and so the MCData originating client will not be able to initiate the next call. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.5.4.2.1, 9.2.5.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Next change *****

###### 9.2.5.4.2.1	Originating procedures

Upon receiving a SIP REFER request with the "method" SIP URI parameter set to value "BYE" in the URI in the Refer-To header field from the MCData client, the participating MCData function:

1)	shall determine the MCData ID of the calling user from public user identity in the P-Asserted-Identity header field of the SIP REFER request;

2)	if the participating MCData function cannot find a binding between the public user identity, then the participating MCData function shall reject the SIP REFER request with a SIP 404 (Not Found) response with the warning text set to "141 user unknown to the participating function" in a Warning header field as specified in subclause 4.9, and skip the rest of the steps;

3)	if the SIP REFER request contained a Refer-Sub header field containing "false" value and a Supported header field containing "norefersub" value, shall handle the SIP REFER request as specified in 3GPP TS 24.229 [5], IETF RFC 3515 [r3515] as updated by IETF RFC 6665 [36], and IETF RFC 4488 [r4488] without establishing an implicit subscription;

4)	shall generate a SIP 200 (OK) response to the SIP REFER request, and in the SIP 200 (OK) response:

a)	shall include the Supported header field with value "norefersub" according to rules and procedures of IETF RFC 4488 [r4488]; and

b)	shall check the presence of the Refer-Sub header field of the SIP REFER request and if it is present and set to the value "false" shall include the Refer-Sub header field with value "false" according to rules and procedures of IETF RFC 4488 [r4488];

5)	shall send the SIP 200 (OK) response to the SIP REFER request towards MCData client according to 3GPP TS 24.229 [5];

6)	shall generate a SIP BYE request, and in the SIP BYE request:

a)	shall set the Request-URI to the MCData session identity which was included at the Refer-To header field of the received REFER request; and

b)	shall copy the contents of the P-Asserted-Identity header field of the received REFER request to the P-Asserted-Identity header field of the outgoing SIP BYE request; and

7)	shall send the SIP BYE request toward the controlling MCData function according to 3GPP TS 24.229 [5].

Upon receiving a SIP 200 (OK) response to the SIP BYE request the participating MCData function shall interact with the media plane as specified in 3GPP TS 24.582 [15] for releasing media plane resources associated with the SIP session with the controlling MCData function. The participating MCData function shall generate a SIP re-INVITE request as specified in subclause 9.2.5.1.2 with following clarifications and send the request towards the originating MCData client according to 3GPP TS 24.229 [5]:

1)	shall set the Request-URI to a public service identity identifying the pre-established session; and

2)	shall set the <mcdata-communication-state> element with a value of "terminated".

***** Next change *****

###### 9.2.5.4.2.2	Terminating procedures

Upon receiving a SIP BYE request from the controlling MCData function, the participating MCData function:

1)	shall interact with the media plane as specified in 3GPP TS 24.582 [15];

2)	shall send a SIP 200 (OK) response to the controlling MCData function;

3)	shall generate a SIP re-INVITE request as specified in subclause 9.2.5.1.2 with following clarifications:

i)	shall set the Request-URI to a public service identity identifying the pre-established session; and

ii)	shall set the <mcdata-communication-state> element with a value of "terminate-request";

4)	shall send the SIP re-INVITE request towards the originating MCData client according to 3GPP TS 24.229 [5]; and

5)	upon receipt of a SIP 2xx response to the SIP re-INVITE, shall interact with the media plane as specified in 3GPP TS 24.582 [15].