- 3GPP TSG CT WG1 Meeting 123-e	C1-202617

- Electronic meeting, 16-24 April 2020

Title:	LS on manipulation of CAG Information element by a VPLMN

Release:	Rel-16

Work Item:	Vertical_LAN

Source:	3GPP CT1

To:	3GPP SA2

Cc:	3GPP SA3

Contact person:	Kundan Tiwari

k.tiwari@samsung.com

+919650954544

# 1	Overall description

CT1 has discussed a scenario in which a VPLMN can manipulate CAG information list, e.g. to keep the UE registered to the VPLMN although according to the updated CAG information list from the UDM other VPLMN can be selected. An example of this is given below. CT1 seeks guidance from SA2 how to avoid VPLMN manipulating the CAG information List.

- A UE first registers to a VPLMN 1 as only VPLMN 1 is available at the location. The country also has VPLMN 2, which is higher priority than VPLMN 1 for the user.

- The VPLMN 1 manipulates CAG information list received from the HPLMN or send a manipulated CAG information list even when the VPLMN did not receive the CAG information from HPLMN. In the manipulated CAG information list, the VPLMN sets the indication that the UE can access the 5GS of VPLMN 2 via a CAG Cell only. The VPLMN sends this to the UE.

- The UE once receiving this information shall never camp on the non-CAG cell of the VPLMN 2. In this way, the VPLMN 1 will keep the UE to its PLMN although higher priority PLMN is available in the country.

# 2	Actions

To 3GPP SA2

ACTION: 	CT1 ask SA2 to provide a guidance to solve the above scenario.

# 3	Dates of next TSG CT WG1 meetings

TSG CT WG1 Meeting 124e	TBC

TSG CT WG1 Meeting 125	USA	24-28 August 2020