3GPP TSG CT WG1 Meeting#123-e	C1-20xxxx (was 2281)

- Electronic meeting, 16-24 April 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.282 | CR | 0128 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of MCData Emergency Alerts at the MCData participating servers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData2 |  |  |  |  |  | Date: |  |  | 2020-04-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add stage 3 text for MCData Rel-16 Emergency Alert functionality, per WI eMCData2 (CP-190199) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add text in 24.282 describing the emergency alert functionality at the MCData participating servers, as follows: - New sections 16.2.3.1 and 16.2.3.2 in 24.282 are loosely based on text in 24.379 sections 12.1.2.1, 12.1.2.2 and 12.1.2.3, respectively |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing feature/functionality and falling out of sync with Rel-16 Stage 2 specs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2.2 (new), 16.2.2.1(new), 16.2.2.2(new), 16.2.2.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

### 16.2.2	Participating MCData function procedures

#### 16.2.2.1	Receipt of a SIP MESSAGE request for emergency notification from the served MCData client

Editor’s note: In the current release, support for emergency groups and emergency group communications may be absent, partial or limited, namely only provided to the extent of facilitating emergency alert functionality.

Upon receipt of a "SIP MESSAGE request for emergency notification for originating participating MCData function", the participating MCData function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP MESSAGE request with a SIP 500 (Server Internal Error) response. The participating MCData function may include a Retry-After header field in the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [4] and skip the rest of the steps;

NOTE 1:	if the SIP MESSAGE request contains an emergency indication set to a value of "true" or an alert indication set to a value of "true", the participating MCData function can, according to local policy, choose to accept the request.

2)	shall determine the MCData ID of the calling user from the public user identity in the P-Asserted-Identity header field of the SIP MESSAGE request, and shall authorise the calling user;

NOTE 2:	The MCData ID of the calling user is bound to the public user identity at the time of service authorisation, as documented in subclause 7.3.

3)	if the MCData user is not affiliated with the MCData group as determined by subclause 8.3.2.11, shall perform the actions specified in subclause 8.3.2.12 for implicit affiliation;

4)	if the actions for implicit affiliation specified in step 3) above were performed but not successful in affiliating the MCData user due to the MCData user already having N2 simultaneous affiliations, shall reject the "SIP MESSAGE request for emergency notification for originating participating MCData function" with a SIP 486 (Busy Here) response with the warning text set to "102 too many simultaneous affiliations" in a Warning header field as specified in subclause 4.9 and skip the rest of the steps;

NOTE 3:	N2 is the total number of MCData groups that an MCData user can be affiliated to simultaneously as specified in 3GPP TS 23.282 [2].

NOTE 4:	As this is a request for MCData emergency services, the participating MCData function can choose to accept the request.

5)	shall determine the public service identity of the controlling MCData function associated with the group identity in the received SIP MESSAGE request;

6)	shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6];

7)	shall set the Request-URI of the outgoing SIP MESSAGE request to the public service identity of the controlling MCData function associated with the group identified by the <mcdata-request-uri> element contained in the <mcdatainfo> element containing the <mcdata-Params> element of the application/vnd.3gpp.mcdata-info+xml MIME body in the incoming SIP MESSAGE request;

8)	shall copy the contents of the application/vnd.3gpp.mcdata-info+xml MIME body in the received SIP MESSAGE request into an application/vnd.3gpp.mcdata-info+xml MIME body as specified in clause D.1 included in the outgoing SIP MESSAGE request;

9)	shall set the <mcdata-calling-user-id> element of the <mcdatainfo> element containing the <mcdata-Params> element to the MCData ID determined in step 2) above;

10)	if the received SIP MESSAGE request contains an application/vnd.3gpp.mcdata-location-info+xml MIME body as specified in clause D.4, shall copy the contents of the application/vnd.3gpp.mcdata-location-info+xml MIME body in the received SIP MESSAGE request into an application/vnd.3gpp.mcdata-location-info+xml MIME body included in the outgoing SIP MESSAGE request;

11)	shall set the P-Asserted-Identity in the outgoing SIP MESSAGE request to the public user identity in the P-Asserted-Identity header field contained in the received SIP MESSAGE request; and

12)	shall send the SIP MESSAGE request as specified in 3GPP TS 24.229 [5].

Upon receipt of a SIP 2xx response in response to the SIP MESSAGE request sent in step 12):

1)	shall generate a SIP 200 (OK) response as specified in 3GPP TS 24.229 [5] with the follow clarifications:

a)	shall include the public user identity received in the P-Asserted-Identity header field of the incoming SIP 2xx response into the P-Asserted-Identity header field of the outgoing SIP 200 (OK) response;

2)	if the procedures of subclause 8.3.2.12 for implicit affiliation were performed in the present subclause, shall complete the implicit affiliation by performing the procedures of subclause 8.3.2.13; and

3)	shall send the SIP 200 (OK) response to the MCData client according to 3GPP TS 24.229 [5].

Upon receipt of a SIP 4xx, 5xx or 6xx response to the sent SIP MESSAGE request and if the implicit affiliation procedures of subclause 8.3.2.12 were invoked in the present subclause, the participating MCData function shall perform the procedures of subclause 8.3.2.14.

#### 16.2.2.2	Receipt of a SIP MESSAGE request for emergency notification for terminating MCData client

Editor’s note: In the current release, support for emergency groups and emergency group communications (in particular the use of the <emergency-ind> element) may be absent, partial or limited, namely only provided to the extent of facilitating emergency alert functionality.

In the procedures in this subclause:

1)	emergency indication in an incoming SIP MESSAGE request refers to the <emergency-ind> element of the application/vnd.3gpp.mcdata-info+xml MIME body; and

2)	alert indication in an incoming SIP MESSAGE request refers to the <alert-ind> element of the application/vnd.3gpp.mcdata-info+xml MIME body.

Upon receipt of a "SIP MESSAGE requests for emergency notification for terminating participating MCData function", the participating MCData function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP MESSAGE request with a SIP 500 (Server Internal Error) response. The participating MCData function may include a Retry-After header field in the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [4] and skip the rest of the steps;

NOTE 1:	if the SIP MESSAGE request contains an emergency indication set to a value of "true" or an alert indication set to a value of "true", the participating MCData function can, by means beyond the scope of this specification, choose to accept the request.

2)	shall use the MCData ID present in the <mcdata-request-uri> element of the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SIP MESSAGE request to retrieve the binding between the MCData ID and public user identity;

3)	if the binding between the MCData ID and public user identity does not exist, then the participating MCData function shall reject the SIP MESSAGE request with a SIP 404 (Not Found) response and skip the rest of the steps. Otherwise, continue with the rest of the steps;

4)	shall generate an outgoing SIP MESSAGE request as specified in subclause 6.3.2.1; and

5)	shall send the SIP MESSAGE request as specified in 3GPP TS 24.229 [5].

Upon receipt of SIP 2xx responses to the outgoing SIP MESSAGE requests, the participating MCData function shall follow the procedures specified in 3GPP TS 24.229 [5].

#### 16.2.2.3	Receipt of a SIP MESSAGE request indicating successful delivery of emergency notification

Upon receipt of a SIP MESSAGE request routed to the terminating participating MCData function with the Request-URI set to the public service identity of the terminating participating MCData function and the SIP MESSAGE request contains an application/vnd.3gpp.mcdata-info+xml MIME body with an <alert-ind-rcvd> element present, the participating MCData function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP MESSAGE request with a SIP 500 (Server Internal Error) response. The participating MCData function may include a Retry-After header field in the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [4] and skip the rest of the steps;

2)	shall use the MCData ID present in the <mcdata-request-uri> element of the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SIP MESSAGE request to retrieve the binding between the MCData ID and public user identity;

3)	if the binding between the MCData ID and public user identity does not exist, then the participating MCData function shall reject the SIP MESSAGE request with a SIP 404 (Not Found) response and skip the rest of the steps. Otherwise, continue with the rest of the steps;

4)	shall generate an outgoing SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] and:

a)	shall include in the SIP MESSAGE request all Accept-Contact header fields and all Reject-Contact header fields, with their feature tags and their corresponding values along with parameters according to rules and procedures of IETF RFC 3841 [8] that were received (if any) in the incoming SIP MESSAGE request;

b)	shall set the Request-URI of the outgoing SIP MESSAGE request to the public user identity associated to the MCData ID of the MCData user that was in the Request-URI of the incoming SIP MESSAGE request;

c)	shall copy the contents of the application/vnd.3gpp.mcdata-info+xml MIME body received in the incoming SIP MESSAGE request into an application/vnd.3gpp.mcdata-info+xml MIME body included in the outgoing SIP MESSAGE request; and

d)	shall copy the contents of the P-Asserted-Identity header field of the incoming SIP MESSAGE request to the P-Asserted-Identity header field of the outgoing SIP MESSAGE request; and

5)	shall send the SIP MESSAGE request as specified in 3GPP TS 24.229 [5].

Upon receipt of SIP 2xx responses to the outgoing SIP MESSAGE requests, the participating MCData function shall follow the procedures specified in 3GPP TS 24.229 [5].

***** End of changes *****