3GPP TSG CT WG1 Meeting#123-e	C1-20xxxx (was 2288)

- Electronic meeting, 16-24 April 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.282 | CR | 0130 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Auxiliary procedures in support of Emergency Alerts for MCData |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData2 |  |  |  |  |  | Date: |  |  | 2020-04-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add stage 3 text for MCData Rel-16 Emergency Alert functionality, per WI eMCData2 (CP-190199) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add text in 24.282 describing miscellaneous procedures that support Emergency Alert functionality as follows: - New sections 6.3.7.1.1, 6.3.7.1.2, 6.3.7.1.3, 6.3.7.1.4, 6.3.7.1.5, 6.3.7.2.1, 6.3.7.2.2 and 6.3.7.2.3 in 24.282 are loosely based on text in 24.379 sections 6.3.3.1.6, 6.3.3.1.11, 6.3.3.1.12, 6.3.3.1.19, 6.3.3.1.20, 6.3.3.1.13.1, 6.3.3.1.13.3 and 6.3.3.1.1.3.4, respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing feature/functionality and falling out of sync with Rel-16 Stage 2 specs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1, 6.3.1.1, 6.3.7(new), 6.3.7.1(new), 6.3.7.1.1 (new), 6.3.7.1.2(new), 6.3.7.1.3(new), 6.3.7.1.4(new), 6.3.7.1.5(new), 6.3.7.2 (new), 6.3.7.2.1(new), 6.3.7.2.2(new), 6.3.7.2.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

### 6.3.1	Distinction of requests at the MCData server

#### 6.3.1.1	SIP MESSAGE request

Editor’s note: In the current release, support for emergency groups and emergency group communications (in particular the use of the <emergency-ind> element) may be absent, partial or limited, namely only provided to the extent of facilitating emergency alert functionality.

The MCData server needs to distinguish between the following SIP MESSAGE request for originations and terminations:

-	SIP MESSAGE requests routed to the participating MCData function as a result of processing initial filter criteria at the S-CSCF in accordance with the origination procedures as specified in 3GPP TS 24.229 [5] with the Request-URI set to the MBMS public service identity of the participating MCData function. Such requests are known as "SIP MESSAGE request for an MBMS listening status update";

-	SIP MESSAGE request routed to the participating MCData function containing a Content-Type header field set to "application/vnd.3gpp.mcdata-location-info+xml" and includes an XML body containing a Location root element containing a Report element. Such requests are known as "SIP MESSAGE request for location reporting";

-	SIP MESSAGE request routed to the MCData client containing a Content-Type header field set to "application/vnd.3gpp.mcdata-location-info+xml" and includes an XML body containing a Location root element containing a Configuration element. Such requests are known as "SIP MESSAGE request for location report configuration";

-	SIP MESSAGE request routed to the MCData client containing a Content-Type header field set to "application/vnd.3gpp.mcdata-location-info+xml" and includes an XML body containing a Location root element containing a Request element. Such requests are known as "SIP MESSAGE request for location report request";

-	SIP MESSAGE request routed to the originating participating MCData function with an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds", and an ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" in a P-Asserted-Service header field. Such requests are known as "SIP MESSAGE request for standalone SDS for originating participating MCData function";

-	SIP MESSAGE request routed to the originating participating MCData function with an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd", and an ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" in a P-Asserted-Service header field, and with an application/vnd.3gpp.mcdata-info+xml MIME body containing a <request-type> element containing the value "msf-disc-req". Such requests are known as "SIP MESSAGE request for absolute URI discovery request for participating MCData function";

-	SIP MESSAGE request routed to the terminating participating MCData function with an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd", and an ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" in a P-Asserted-Service header field, and with an application/vnd.3gpp.mcdata-info+xml MIME body containing a <request-type> element containing the value "msf-disc-res". Such requests are known as "SIP MESSAGE request for absolute URI discovery response for participating MCData function";

-	SIP MESSAGE request routed to the controlling MCData function with an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd", and an ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" in a P-Asserted-Service header field, and with an application/vnd.3gpp.mcdata-info+xml MIME body containing a <request-type> element containing the value "msf-disc-req". Such requests are known as "SIP MESSAGE request for absolute URI discovery request for controlling MCData function";

-	SIP MESSAGE request routed to the originating participating MCData function with an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd", and an ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" in a P-Asserted-Service header field. Such requests are known as "SIP MESSAGE request for FD using HTTP for originating participating MCData function";

-	SIP MESSAGE request routed to the terminating participating MCData function with an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd", and an ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" in a P-Asserted-Service header field, and with an application/vnd.3gpp.mcdata-signalling MIME body containing an FD NETWORK NOTIFICATION message. Such requests are known as "SIP MESSAGE network notification for FD using HTTP for terminating participating MCData function";

-	SIP MESSAGE request routed to the terminating participating MCData function with an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds", and an ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" in a P-Asserted-Service header field. Such requests are known as "SIP MESSAGE request for standalone SDS for terminating participating MCData function";

-	SIP MESSAGE request routed to the terminating participating MCData function with an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd", and an ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" in a P-Asserted-Service header field. Such requests are known as "SIP MESSAGE request for FD using HTTP for terminating participating MCData function";

-	SIP MESSAGE request routed to an MCData server with an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds", an ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" in a P-Asserted-Service header field, and with an application/vnd.3gpp.mcdata-signalling MIME body containing an SDS NOTIFICATION message Such requests are known as "SIP MESSAGE request for SDS disposition notification for MCData server";

-	SIP MESSAGE request routed to an MCData server with an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd", an ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" in a P-Asserted-Service header field, and with an application/vnd.3gpp.mcdata-signalling MIME body containing an FD NOTIFICATION message. Such requests are known as "SIP MESSAGE request for FD disposition notification for MCData server";

-	SIP MESSAGE request routed to the controlling MCData function with an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds", and an ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" in a P-Asserted-Service header field. Such requests are known as "SIP MESSAGE request for standalone SDS for controlling MCData function";

-	SIP MESSAGE request routed to the controlling MCData function with an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd", and an ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" in a P-Asserted-Service header field. Such requests are known as "SIP MESSAGE request for FD using HTTP for controlling MCData function";

-	SIP MESSAGE requests routed to the controlling MCData function with the Request-URI set to the public service identity of the controlling MCData function and containing a Content-Type header field set to "application/vnd.3gpp.mcdata-info+xml" and including an XML body containing a <mcdatainfo> root element containing a <mcdata-Params> element containing an <emergency-ind> element or an <alert-ind> element. Such requests are known as "SIP MESSAGE requests for emergency notification for controlling MCData function";

-	SIP MESSAGE requests routed to the originating participating MCData function with the Request-URI set to the public service identity of the participating MCData function and containing a Content-Type header field set to "application/vnd.3gpp.mcdata-info+xml" and including an XML body containing a <mcdatainfo> root element containing a <mcdata-Params> element containing an <emergency-ind> element or an <alert-ind> element. Such requests are known as "SIP MESSAGE requests for emergency notification for originating participating MCData function";

-	SIP MESSAGE requests routed to the terminating participating MCData function with the Request-URI set to the public service identity of the terminating participating MCData function and containing a Content-Type header field set to "application/vnd.3gpp.mcdata-info+xml" and including an XML body containing a <mcdatainfo> root element containing a <mcdata-Params> element containing an <emergency-ind> element or an <alert-ind> element. Such requests are known as "SIP MESSAGE requests for emergency notification for terminating participating MCData function";

-	SIP MESSAGE requests routed to the terminating participating MCData function with the Request-URI set to the public service identity of the terminating participating MCData function and containing an "application/vnd.3gpp.mcdata-info+xml" MIME body with an <alert-ind-rcvd> element present. Such requests are known as "SIP MESSAGE requests indicating delivery of emergency notification”; and

-	SIP MESSAGE request routed to the terminating participating MCData function with an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd", and an ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" in a P-Asserted-Service header field, and with an application/vnd.3gpp.mcdata-signalling MIME body containing an DEFERRED DATA REQUEST message. Such requests are known as "SIP MESSAGE request for list of deferred group communications".

If a SIP MESSAGE request received at an MCData server is not in accordance with the SIP MESSAGE requests listed above, then the MCData server shall reject the SIP MESSAGE request with a SIP 403 (Forbidden) response.

***** Next change *****

### 6.3.7	Procedures referenceable from other procedures

#### 6.3.7.1	Emergency alert and emergency communications procedures

##### 6.3.7.1.1	Sending a SIP re-INVITE request for MCData emergency alert or emergency group communication

Editor’s note: In the current release, support for emergency groups and emergency group communications (in particular the use of the <emergency-ind> element) may be absent, partial or limited, namely only provided to the extent of facilitating emergency alert functionality.

This subclause is referenced from other procedures.

The controlling MCData function shall generate a SIP re-INVITE request according to 3GPP TS 24.229 [5].

The controlling MCData function:

1)	shall include an SDP offer with the media parameters as currently established with the terminating MCData client according to 3GPP TS 24.229 [5];

2)	shall include an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdata-calling-user-id> element set to the MCData ID of the initiating MCData user;

3)	if the in-progress emergency group state of the group is set to a value of "true" the controlling MCData function:

a)	shall include a Resource-Priority header field with the namespace populated with the values for an MCData emergency group communication as specified in subclause 6.3.7.1.4;

b)	shall include in the application/vnd.3gpp.mcdata-info+xml MIME body the <emergency-ind> element set to a value of "true"; and

c)	if the <alert-ind> element is set to "true" in the received SIP re-INVITE request and MCData emergency alerts are authorised for this group and MCData user as determined by the procedures of subclause 6.3.7.2.1, shall populate the application/vnd.3gpp.mcdata-info+xml MIME body and application/vnd.3gpp.mcdata-location-info+xml MIME body as specified in subclause 6.3.7.1.3. Otherwise, shall set the <alert-ind> element to a value of "false" in the application/vnd.3gpp.mcdata-info+xml MIME body;

4)	if the in-progress emergency group state of the group is set to a value of "false":

a)	shall include a Resource-Priority header field populated with the values for a normal MCData group communication as specified in subclause 6.3.7.1.4; and

b)	if the received SIP re-INVITE request contained an application/vnd.3gpp.mcdata-info+xml MIME body with the <emergency-ind> element set to a value of "false" and this is an authorised request to cancel an MCData emergency group communication as determined by the procedures of subclause 6.3.7.2.3:

i)	shall include an application/vnd.3gpp.mcdata-info+xml MIME body with the <emergency-ind> element set to a value of "false"; and

ii)	if the received SIP re-INVITE request contained an application/vnd.3gpp.mcdata-info+xml MIME body with the <alert-ind> element set to a value of "false" and this is an authorised request to cancel an MCData emergency alert as determined by the procedures of subclause 6.3.7.2.2, shall:

A)	include in the application/vnd.3gpp.mcdata-info+xml MIME body an <alert-ind> element set to a value of "false"; and

B)	if the received SIP request contains an <originated-by> element in the application/vnd.3gpp.mcdata-info+xml MIME body, copy the contents of the received <originated-by> element to an <originated-by> element in the application/vnd.3gpp.mcdata-info+xml MIME body in the outgoing SIP re-INVITE request.

##### 6.3.7.1.2	Generating a SIP MESSAGE request for notification of in-progress emergency status change

Editor’s note: In the current release, support for emergency groups and emergency group communications may be absent, partial or limited, namely only provided to the extent of facilitating emergency alert functionality.

This subclause is referenced from other procedures.

This subclause describes the procedures for generating a SIP MESSAGE request to notify affiliated but not participating members of an MCData group of the change of status of the in-progress emergency state or emergency alert status of an MCData group. The procedure is initiated by the controlling MCData function when there has been a change of in-progress emergency or the emergency alert status of an MCData group.

The controlling MCData function:

1)	shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6];

2)	shall include an Accept-Contact header field containing the g.3gpp.mcdata media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

3)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref with the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata" along with parameters "require" and "explicit" according to IETF RFC 3841 [8];

4)	shall set the Request-URI to the address of the terminating participating function associated with the MCData ID of the targeted MCData user;

5)	shall include a P-Asserted-Identity header field set to the public service identity of controlling MCData function;

6)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata" (coded as specified in 3GPP TS 24.229 [5]), in a P-Asserted-Service-Id header field according to IETF RFC 6050 [7];

7)	shall include an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with the <mcdata-request-uri> element set to the value of the MCData ID of the targeted MCData user; and

8)	shall include in the application/vnd.3gpp.mcdata-info+xml MIME body an <mcdata-calling-group-id> element set to the MCData group ID of the MCData group on which the MCData emergency communication or the emergency alert state has changed.

##### 6.3.7.1.3	Populate mcdata-info and location-info MIME bodies for emergency alert

This subclause is referenced from other procedures.

This subclause describes the procedures for populating the application/vnd.3gpp.mcdata-info+xml and application/vnd.3gpp.mcdata-location-info+xml MIME bodies for an MCData emergency alert. The procedure is initiated by the controlling MCData function when it has received a SIP request initiating an MCData emergency alert and generates a message containing the MCData emergency alert information required by 3GPP TS 23.282 [2].

The controlling MCData function:

1)	shall include, if not already present, an application/vnd.3gpp.mcdata-info+xml MIME body as specified in Annex D.1, and set the <alert-ind> element to a value of "true";

2)	shall determine the value of the MCData user's Mission Critical Organization from the <MissionCriticalOrganization> element, of the MCData user profile document identified by the MCData ID and profile index associated with MCData user (see the MCData user profile document in 3GPP TS 24.484 [12]);

3)	shall include in the <mcdatainfo> element containing the <mcdata-Params> element an <mc-org> element set to the value of the MCData user's Mission Critical Organization; and

4)	shall copy the contents of the application/vnd.3gpp.mcdata-location-info+xml MIME body in the received SIP request into an application/vnd.3gpp.mcdata-location-info+xml MIME body included in the outgoing SIP request.

##### 6.3.7.1.4	Retrieving Resource-Priority header field values for emergency communications

Editor’s note: In the current release, support for emergency groups and emergency group communications may be absent, partial or limited, namely only provided to the extent of facilitating emergency alert functionality.

This subclause is referenced from other procedures.

When determining the Resource-Priority header field namespace and priority values as specified in IETF RFC 8101 [67] for an MCData emergency communication, the controlling MCData function:

1)	shall retrieve the value of the <resource-priority-namespace> element contained in the <emergency-resource-priority> element contained in the <OnNetwork> element of the MCData service configuration document (see the service configuration document in 3GPP TS 24.484 [12]); and

2)	shall retrieve the value of the <resource-priority-priority> element contained in the <emergency-resource-priority> element contained in the <OnNetwork> element of the MCData service configuration document (see the service configuration document in 3GPP TS 24.484 [12]).

When determining the Resource-Priority header field namespace and priority values as specified in IETF RFC 8101 [67] for a normal MCData communication, the controlling MCData function:

1)	shall retrieve the value of the <resource-priority-namespace> element contained in the <normal-resource-priority> element contained in the <OnNetwork> element of the MCData service configuration document (see the service configuration document in 3GPP TS 24.484 [12]); and

2)	shall retrieve the value of the <resource-priority-priority> element contained in the <normal-resource-priority> element contained in the <OnNetwork> element of the MCData service configuration document (see the service configuration document in 3GPP TS 24.484 [12]).

NOTE:	The "normal" Resource-Priority header field value is needed to return to a normal priority value from a priority value adjusted for an MCData emergency communication. The "normal" priority received from the EPS by use of the "normal" Resource-Priority header field value is expected to be the same as the "normal" priority received from the EPS when initiating a communication with no Resource-Priority header field included.

##### 6.3.7.1.5	Generating a SIP MESSAGE request to indicate successful receipt of an emergency alert or emergency cancellation

Editor’s note: In the current release, support for emergency groups and emergency group communications may be absent, partial or limited, namely only provided to the extent of facilitating emergency alert functionality.

This subclause is referenced from other procedures.

This subclause describes the procedures for generating a SIP MESSAGE request to notify the originator of an emergency alert or emergency cancellation that the request was successfully received.

The controlling MCData function:

1)	shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6];

2)	shall include an Accept-Contact header field containing the g.3gpp.mcdata media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

3)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref with the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata" along with parameters "require" and "explicit" according to IETF RFC 3841 [8];

4)	shall set the Request-URI to the address of the terminating participating function associated with the MCData ID of the targeted MCData user;

5)	shall include a P-Asserted-Identity header field set to the public service identity of controlling MCData function; and

6)	shall include an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with the <mcdata-request-uri> element set to the value of the MCData ID of the targeted MCData user.

#### 6.3.7.2	Authorisations

##### 6.3.7.2.1	Determining authorisation for initiating an MCData emergency alert

If the controlling MCData function has received a SIP request targeted to an MCData group with the <alert-ind> element of the application/vnd.3gpp.mcdata-info+xml MIME body set to a value of "true", the controlling MCData function shall check the following conditions:

1)	if the <allow-activate-emergency-alert> element of the <actions> element of a <rule> element of the <ruleset> element of the MCData user profile document identified by the MCData ID and profile index of the calling user (see the MCData user profile document in 3GPP TS 24.484 [12]) is set to a value of "true":

a)	if the "entry-info" attribute of the <entry> element of the <EmergencyAlert> element contained within the <MCData-group-call> element of the MCData user profile document (see the MCData user profile document in 3GPP TS 24.484 [12]) is set to a value of "DedicatedGroup" and:

i)	if the MCData group identity targeted for the emergency alert is contained in the <uri-entry> element of the <entry> element of the <EmergencyAlert> element contained within the <MCData-group-call> element of the MCData user profile document (see the MCData user profile document in 3GPP TS 24.484 [12]); and

ii)	if the <allow-MCData-emergency-alert> element of the <actions> element of a <rule> element of the <ruleset> element of the <list-service> element of the group document identified by the MCData group identity is set to a value of "true" as specified in 3GPP TS 24.481 [11]; or

b)	if the "entry-info" attribute of the <entry> element of the <EmergencyAlert> element contained within the <MCData-group-call> element of the MCData user profile (see the MCData user profile document in 3GPP TS 24.484 [12]) is set to a value of "UseCurrentlySelectedGroup" and the <allow-MCData-emergency-alert> element of the <actions> element of a <rule> element of the <ruleset> element of the <list-service> element of the group document identified by the MCData group identity targeted for the emergency alert is set to a value of "true" as specified in 3GPP TS 24.481 [11];

then the MCData emergency alert request shall be considered to be an authorised request for an MCData emergency alert targeted to a MCData group. In all other cases, the MCData emergency alert request shall be considered to be an unauthorised request for an MCData emergency alert targeted to an MCData group.

If the controlling MCData function has received a SIP request targeted to an MCData user with the <alert-ind> element of the application/vnd.3gpp.mcdata-info+xml MIME body set to a value of "true", the controlling MCData function shall check the following conditions:

1)	if the <allow-activate-emergency-alert> element of the <actions> element of the <rule> element of the <ruleset> element of the MCData user profile document identified by the MCData ID and profile index of the calling user (see the MCData user profile document in 3GPP TS 24.484 [12]) is set to a value of "true"; and

a)	if the "entry-info" attribute of the <entry> element of the <PrivateEmergencyAlert> element contained within the <OnNetwork> element of the MCData user profile document (see the MCData user profile document in 3GPP TS 24.484 [12]) is set to a value of "UsePreConfigured" and the MCData ID of the MCData user targeted for the communication is contained in the <uri-entry> element of the <entry> element of the <PrivateEmergencyAlert> element contained within the <OnNetwork> element (see the MCData user profile document in 3GPP TS 24.484 [12]); or

b)	if the "entry-info" attribute of the <entry> element of the <PrivateEmergencyAlert> element contained within the <OnNetwork> element of the MCData user profile document (see the MCData user profile document in 3GPP TS 24.484 [12]) is set to a value of "LocallyDetermined";

then the MCData emergency alert request shall be considered to be an authorised request for an MCData emergency alert targeted to an MCData user. In all other cases, it shall be considered to be an unauthorised request for an MCData emergency alert targeted to an MCData user.

##### 6.3.7.2.2	Determining authorisation for cancelling an MCData emergency alert

If the controlling MCData function has received a SIP request with the <alert-ind> element of the application/vnd.3gpp.mcdata-info+xml MIME body set to a value of "false" and:

1)	if the <allow-cancel-emergency-alert> element of the <ruleset> element of the MCData user profile document identified by the MCData ID and profile index of the calling user (see the MCData user profile document in 3GPP TS 24.484 [12]) is set to a value of "true", then the MCData emergency alert cancellation request shall be considered to be an authorised request for an MCData emergency alert cancellation; and

2)	if the <allow-cancel-emergency-alert> element of the <ruleset> element of the MCData user profile document identified by the MCData ID and profile index of the calling user (see the MCData user profile document in 3GPP TS 24.484 [12]) is set to a value of "false", then the MCData emergency alert cancellation request shall be considered to be an unauthorised request for an MCData emergency alert cancellation.

##### 6.3.7.2.3	Determining authorisation for cancelling an MCData emergency communication

Editor’s note: In the current release, support for emergency groups and emergency group communications (in particular the use of the <emergency-ind> element) may be absent, partial or limited, namely only provided to the extent of facilitating emergency alert functionality.

If the controlling MCData function has received a SIP request for an MCData group communication with the <emergency-ind> element of the application/vnd.3gpp.mcdata-info+xml MIME body set to a value of "false" and:

1)	if the <allow-cancel-group-emergency> element of the <ruleset> element of the MCData user profile document identified by the MCData ID and profile index of the calling user (see the MCData user profile document in 3GPP TS 24.484 [12]) is set to a value of "true", then the MCData emergency communication cancellation request shall be considered to be an authorised request for an MCData emergency group communication cancellation; and

2)	If the <allow-cancel-group-emergency> element of the <ruleset> element of the MCData user profile document identified by the MCData ID and profile index of the calling user (see the MCData user profile document in 3GPP TS 24.484 [12]) is set to a value of "false", then the MCData emergency group communication cancellation request shall be considered to be an unauthorised request for an MCData emergency group communication cancellation.

If the controlling MCData function has received a SIP request for an MCData private communication with the <emergency-ind> element of the application/vnd.3gpp.mcdata-info+xml MIME body set to a value of "false" and:

1)	if the <allow-cancel-private-emergency-call> element of the <ruleset> element of the MCData user profile document identified by the MCData ID and profile index of the calling user (see the MCData user profile document in 3GPP TS 24.484 [12]) is set to a value of "true", then the MCData emergency private communication cancellation request shall be considered to be an authorised request for an MCData emergency private communication cancellation; and

2)	if the <allow-cancel-private-emergency-call> element of the <ruleset> element of the MCData user profile document identified by the MCData ID and profile index of the calling user (see the MCData user profile document in 3GPP TS 24.484 [12]) is set to a value of "false" or not present, then the MCData emergency private communication cancellation request shall be considered to be an unauthorised request for an MCData emergency private communication cancellation.

***** End of changes *****