3GPP TSG CT WG1 Meeting#123-e	C1-20xxxx (was 2260)

- Electronic meeting, 16-24 April 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.282 | CR | 0126 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for MCData emergency alert and communications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData2 |  |  |  |  |  | Date: |  |  | 2020-04-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add stage 3 text for MCData Rel-16 Emergency Alert functionality, per WI eMCData2 (CP-190199) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Miscellaneous Additions/modifications of text in 24.282 supporting the functionality of emergency alert and communication. Some states and definitions are loosely based on corresponding clauses from 24.379. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing feature/functionality and falling out of sync with Rel-16 Stage 2 specs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3, 3.1, 3.2, 4.10(new), D.1.3, G.4.2(new), G.4.3(new), G.4.4(new), G.4.5(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2";

[3]	3GPP TS 23.280:" Common functional architecture to support mission critical services; Stage 2";

[4]	IETF RFC 3261 (June 2002): "SIP: Session Initiation Protocol".

[5]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[6]	IETF RFC 3428 (December 2002): "Session Initiation Protocol (SIP) Extension for Instant Messaging".

[7]	IETF RFC 6050 (November 2010): "A Session Initiation Protocol (SIP) Extension for the Identification of Services".

[8]	IETF RFC 3841 (August 2004): "Caller Preferences for the Session Initiation Protocol (SIP)".

[9]	IETF RFC 4826 (May 2007): "Extensible Markup Language (XML) Formats for Representing Resource Lists".

[10]	3GPP TS 24.379: "Mission Critical Push To Talk (MCPTT) call control Protocol specification".

[11]	3GPP TS 24.481: "Mission Critical Services (MCS) group management Protocol specification".

[12]	3GPP TS 24.484: "Mission Critical Services (MCS) configuration management Protocol specification".

[13]	IETF RFC 4483 (May 2006): "A Mechanism for Content Indirection in Session Initiation Protocol (SIP) Messages.

[14]	IETF RFC 4122 (July 2005): "A Universally Unique IDentifier (UUID) URN Namespace".

[15]	3GPP TS 24.582: "Mission Critical Data (MCData) media plane control Protocol specification";

[16]	IETF RFC 3840 (August 2004): "Indicating User Agent Capabilities in the Session Initiation Protocol (SIP)".

[17]	IETF RFC 4975 (September 2007): "The Message Session Relay Protocol (MSRP)".

[18]	IETF RFC 5366 (October 2008): "Conference Establishment Using Request-Contained Lists in the Session Initiation Protocol (SIP)".

[19]	IETF RFC 6135 (February 2011): "An Alternative Connection Model for the Message Session Relay Protocol (MSRP) ".

[20]	IETF RFC 6714 (August 2012): "Connection Establishment for Media Anchoring (CEMA) for the Message Session Relay Protocol (MSRP)".

[21]	IETF RFC 6086 (January 2011): "Session Initiation Protocol (SIP) INFO Method and Package Framework".

[22]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[23]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[24]	3GPP TS 24.482: "Mission Critical Services (MCS) identity management Protocol specification.

[25]	3GPP TS 24.334: "Proximity-services (ProSe) User Equipment (UE) to Proximity-services (ProSe) Function Protocol aspects; Stage 3".

[26]	3GPP TS 33.180: "Security of the Mission Critical Service".

[27]	IETF RFC 3856 (August 2004): "A Presence Event Package for the Session Initiation Protocol (SIP)".

[28]	W3C: "XML Encryption Syntax and Processing Version 1.1", https://www.w3.org/TR/xmlenc-core1/.

[29]	W3C: "XML Signature Syntax and Processing (Second Edition)", http://www.w3.org/TR/xmldsig-core/.

[30]	IETF RFC 4648 (October 2006): "The Base16, Base32, and Base64 Data Encodings".

[31]	3GPP TS 23.003: "Numbering, addressing and identification".

[32]	IETF RFC 2045 (November 1996): "Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies".

[33]	IETF RFC 2392 (August 1998): "Content-ID and Message-ID Uniform Resource Locators".

[34]	IETF RFC 3903 (October 2004): "Session Initiation Protocol (SIP) Extension for Event State Publication".

[35]	IETF RFC 4354 (January 2006): "A Session Initiation Protocol (SIP) Event Package and Data Format for Various Settings in Support for the Push-to-Talk over Cellular (PoC) Service".

[36]	IETF RFC 6665 (July 2012): "SIP-Specific Event Notification".

[37]	3GPP TS 29.283: "Diameter Data Management Applications".

[38]	IETF RFC 4028 (April 2005): "Session Timers in the Session Initiation Protocol (SIP)".

[39]	IETF RFC 3856 (August 2004): "A Presence Event Package for the Session Initiation Protocol (SIP)".

[40]	IETF RFC 3863 (August 2004): "Presence Information Data Format (PIDF)".

[41]	IETF RFC 4661 (September 2006): "An Extensible Markup Language (XML)-Based Format for Event Notification Filtering".

[42]	3GPP TS 24.483: "Mission Critical Services (MCS) Management Object (MO)".

[43]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[44]	IETF RFC 5627 (October 2009): "Obtaining and Using Globally Routable User Agent URIs (GRUUs) in the Session Initiation Protocol (SIP)".

[45]	IETF RFC 4567 (July 2006): "Key Management Extensions for Session Description Protocol (SDP) and Real Time Streaming Protocol (RTSP)".

[46]	IETF RFC 3986 (January 2005): "Uniform Resource Identifier (URI): Generic Syntax".

[47]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[48]	3GPP TS 29.582: "Mission Critical Data (MCData) signalling control interworking with LMR systems; Protocol specification".

[49]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[50]	IETF RFC 5245 (April 2010): "Interactive Connectivity Establishment (ICE): A Protocol for Network Address Translator (NAT) Traversal for Offer Answer Protocols".

[51]	IETF RFC 3515 (April 2003): "The Session Initiation Protocol (SIP) Refer Method".

[52]	IETF RFC 7647 (September 2015): "Clarifications for the use of REFER with RFC6665".

[53]	IETF RFC 4488 (May 2006): "Suppression of Session Initiation Protocol (SIP) REFER Method Implicit Subscription".

[54]	IETF RFC 4538 (June 2006): "Request Authorization through Dialog Identification in the Session Initiation Protocol (SIP)".

[55]	IETF RFC 6509 (February 2012): "MIKEY-SAKKE: Sakai-Kasahara Key Encryption in Multimedia Internet KEYing (MIKEY)".

[56]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[57]	3GPP TS 29.468: "Group Communication System Enablers for LTE (GCSE_LTE); MB2 reference point; Stage 3".

[58]	Void.

[59]	IETF RFC 5761 (April 2010): "Multiplexing RTP Data and Control Packets on a Single Port".

[60]	IETF RFC 5795 (March 2010): "The RObust Header Compression (ROHC) Framework".

[61]	IETF RFC 3095 (July 2001): "RObust Header Compression (ROHC): Framework and four profiles: RTP, UDP, ESP, and uncompressed".

[62]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[63]	3GPP TS 23.203: "Policy and charging control architecture".

[64]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[65]	3GPP TS 29.199-09: "Open Service Access (OSA); Parlay X web services; Part 9: Terminal location".

[66]	OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C: "RESTful Network API for Network Message Storage".

[67]	IETF RFC 8101 (March 2017): "IANA Registration of New Session Initiation Protocol (SIP) Resource-Priority Namespace for Mission Critical Push To Talk Service".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

An MCData user is affiliated to an MCData group: The MCData user has expressed interest in an MCData group it is a member of, and both the MCData server serving the MCData user and the MCData server owning the MCData group have authorized the MCData user's interest in the MCData group communication.

An MCData user is affiliated to an MCData group at an MCData client: The MCData user is affiliated to the MCData group, the MCData client has a registered IP address for an IMPU related to the MCData ID, and the MCData server serving the MCData user has authorised the MCData user's interest in the MCData group at the MCData client.

Affiliation status: Applies for an MCData user to an MCData group and has one of the following states:

a)	the "not-affiliated" state indicating that the MCData user is not interested in the MCData group and the MCData user is not affiliated to the MCData group;

b)	the "affiliating" state indicating that the MCData user is interested in the MCData group but the MCData user is not affiliated to the MCData group yet;

c)	the "affiliated" state indicating that the MCData user is affiliated to the MCData group and there was no indication that MCData user is no longer interested in the MCData group; and

d)	the "deaffiliating" state indicating that the MCData user is no longer interested in the MCData group but the MCData user is still affiliated to the MCData group.

Group identity: An MCData group identity or a temporary MCData group identity.

MCData client ID: is a globally unique identification of a specific MCData client instance. MCData client ID is a UUID URN as specified in IETF RFC 4122 [14].

MCData emergency alert state: MCData client internal perspective of the state of an MCData emergency alert.

MCData emergency group state: MCData client internal perspective of the in-progress emergency state of an MCData group maintained by the controlling MCData function.

MCData emergency group communication state: MCData client internal perspective of the state of an MCData emergency group communication.

For the purpose of the present document, the following terms and definitions given in 3GPP TS 33.180 [26] apply:

Client Server Key (CSK)

Multicast Signalling Key (MuSiK)

Multicast Signalling Key Identifier (MuSiK-ID)

MBMS subchannel control key (MSCCK)

MBMS subchannel control key identifier (MSCCK-ID)

Private Call Key (PCK)

Signalling Protection Key (SPK)

XML Protection Key (XPK)

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CSK			Client-Server Key

MBMS			Multimedia Broadcast and Multicast Service

MC			Mission Critical

MCS			Mission Critical Service

MCData			Mission Critical Data

MCData group ID	MCData group Identity

MDEA			MCData Emergency Alert

MDEG			MCData Emergency Group

MDEGC			MCData Emergency Group Communication

MDES			MCData Emergency State

MIME			Multipurpose Internet Mail Extensions

MONP			MC service Off-Network Protocol

QCI			QoS Class Identifier

RTP			Real-time Transport Protocol

SAI			Service Area Identifier

SDP			Session Description Protocol

SIP			Session Initiation Protocol

SPK			Signalling Protection Key

URI			Uniform Resource Identifier

XPK			XML Protection Key

***** Next change *****

## 4.10	MCData emergency groups and emergency group communications

Editor’s note: In the current release, support for emergency groups and emergency group communications may be absent, partial or limited, namely only provided to the extent of facilitating emergency alert functionality.

MCData emergency groups and emergency group communications as defined by 3GPP TS 23.282 [2] are supported by the procedures in this specification. There are a number of state variables used to manage MCData emergencies, including:

-	MCData emergency state: in accordance with 3GPP TS 23.282 [2], indicates (see subclause G.4.2) that the MCData user is in a life-threatening situation. This MCData client state variable is changed via action by the MCData user of the device or by an authorised MCData user. While the MCData emergency state is set on the client, all communications originated by the client will be MCData emergency communications, assuming the MCData user is authorised for MCData emergency communications.

-	in-progress emergency group state: in accordance with 3GPP TS 23.282 [2], this state variable (see subclause G.4.3) indicates whether or not there is an MCData emergency group communication ongoing on the specified group. This state is managed by the controlling MCData function. All group communications originated on this MCData group when in an in-progress emergency state are MCData emergency group communications until this state is cancelled, regardless of the originator being (or not) in an MCData emergency state.

-	MCData emergency group (MDEG) state: this is an internal state (see subclause G.4.4) managed by the MCData client which tracks the in-progress emergency state of the group (see 3GPP TS 23.282 [2]) managed by the controlling MCData function. Ideally, the MCData client would not need to track the in-progress emergency group state, but doing so enables the MCData client to request MCData emergency-level priority earlier than otherwise possible. For example, if the MCData user wishes to join an MCData emergency group communication and is not in MCData emergency state itself, the MCData client should have emergency level priority. If it has knowledge of the in-progress emergency state of the group, it can request priority by including a Resource-Priority header field set to the MCPTT namespace specified in IETF RFC 8101 [67], and appropriate priority level in the SIP INVITE request (or SIP re-INVITE request).

-	MCData emergency group communication (MDEGC) state: this is an internal state (see subclause G.4.5) corresponding to an ongoing group communication. The state is managed by the MCData client, which in conjunction with the MCData emergency alert state (see subclause 4.4), aids in managing the MCData emergency state and related actions.

***** Next change *****

## D.1.3	Semantic

Editor’s note: In the current release, support for emergency groups and emergency group communications (in particular the use of the <emergency-ind> element) may be absent, partial or limited, namely only provided to the extent of facilitating emergency alert functionality.

The <mcdatainfo> element is the root element of the XML document. The <mcdatainfo> element can contain subelements.

NOTE 1:	The subelements of the <mcdata-info> are validated by the <xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> particle of the <mcdata-info> element

If the <mcdatainfo> contains the <mcdata-Params> element then:

1)	the <mcdata-access-token>, <mcdata-request-uri>, <mcdata-controller-psi>, <mcdata-calling-user-id>, <mcdata-called-party-id>, <mcdata-calling-group-id>, <alert-ind>, <originated-by> and <mcdata-client-id> can be included with encrypted content;

2)	for each element in 1) that is included with content that is not encrypted:

a)	the element has the "type" attribute set to "Normal";

b)	if the element is the <mcdata-request-uri>, <mcdata-calling-user-id>, <mcdata-called-party-id> or <mcdata-calling-group-id> or <originated-by> then the <mcdataURI> element is included;

c)	if the element is the <mcdata-access-token> or <mcdata-client-id>, then the <mcdataString> element is included; and

d)	if the element is <alert-ind> then the <mcdataBoolean> element is included;

3)	for each element in 1) that is included with content that is encrypted:

a)	the element has the "type" attribute set to "Encrypted";

b)	the <xenc:EncryptedData> element from the "http://www.w3.org/2001/04/xmlenc#" namespace is included and:

i)	can have a "Type" attribute can be included with a value of "http://www.w3.org/2001/04/xmlenc#Content";

ii)	can include an <EncryptionMethod> element with the "Algorithm" attribute set to value of "http://www.w3.org/2009/xmlenc11#aes128-gcm";

iii)	can include a <KeyInfo> element with a <KeyName> element containing the base 64 encoded XPK-ID; and

iv)	includes a <CipherData> element with a <CipherValue> element containing the encrypted data.

NOTE 2:	When the optional attributes and elements are not included within the <xenc:EncryptedData> element, the information they contain is known to sender and the receiver by other means.

If the <mcdatainfo> contains the <mcdata-Params> element then:

1)	the <mcdata-access-token> can be included with the access token received during authentication procedure as described in 3GPP TS 24.382 [49];

2)	the <request-type> can be included with:

a)	a value of "one-to-one-sds" to indicate that the MCData client wants to initiate a one-to-one SDS request;

b)	a value of "group-sds" to indicate the MCData client wants to initiate a group SDS request;

c)	a value of "one-to-one-fd" to indicate that the MCData client wants to initiate a one-to-one FD request;

d)	a value of "group-fd" to indicate that the MCData client wants to initiate a group FD request;

e)	a value of "msf-disc-req" to indicate that the MCData client wishes to discover the absoluteURI of the media storage function for HTTP requests;

f)	a value of "msf-disc-res" when the participating MCData function sends the absolute URI to the MCData client;

g)	a value of "notify" when the controlling MCData function needs to send a notification to the MCData client;

h)	a value of "one-to-one-sds-session" to indicate that the MCData client wants to initiate a one-to-one SDS session; and

i)	a value of "group-sds-session" to indicate the MCData client wants to initiate a group SDS session.

3)	the <mcdata-request-uri> can be included with an MCData group ID;

4)	the <mcdata-calling-user-id> can be included, set to MCData ID of the originating user;

5)	the <mcdata-called-party-id> can be included, set to the MCData ID of the terminating user;

6)	the <mcdata-calling-group-id> can be included to indicate the MCData group identity to the terminating user;

7)	the <alert-ind> can be:

a)	set to "true" to indicate that an alert to be sent; or

b)	set to "false" to indicate that an alert to is be cancelled;

8)	the <originated-by> can be included, set to the MCData ID of the originating user of an MCData emergency alert when being cancelled by another authorised MCDATA user;

9)	the <mcdata-client-id>: can be included, set to the MCData client ID of the MCData client that originated a SIP INVITE request, SIP REFER request or SIP MESSAGE request; and

10)	the <mcdata-controller-psi> can be included, set to the PSI of the controlling MCData function that handled the one-to-one or group MCData data request; and

11)	the <anyExt> can be included with the following elements not declared in the XML schema:

a)	a <pre-established-session-ind> of type "xs:Boolean":

i)	set to a value of "true" by MCData client in pre-established session setup request to indicate MCData participating function about initiation of pre-established session.

b)	a <mcdata-communication-state> of type "xs:string" can be included to indicate state of MCData communication within pre-established session. The <mcdata-communication-state> can be set to:

i)	a value of "establish-request" by MCData participating function to indicate to the MCData client about MCData communication establishement request within pre-established session;

ii)	a value of "establish-success" by MCData participating function or MCData client to indicate that the MCData communication is established successfully;

iii)	a value of "establish-fail" by MCData participating function or MCData client to indicate that the MCData communication establishement is failed or rejected;

iv)	a value of "terminate-request" by MCData participating function to indicate to the MCData client about MCData communication termination request within pre-established session; and

v)	a value of "terminated" by MCData participating function or MCData client to indicate MCData communication is terminated.

c)	an <emergency-ind> of type "xs:Boolean" can be included and set to:

i)	"true" to indicate that the communication that the MCData client is initiating is an emergency MCData communication; or

ii)	"false" to indicate that the MCData client is cancelling an emergency MCData communication (i.e. converting it back to a non-emergency communication);

d)	an  <alert-ind-rcvd> of type "xs:Boolean":

i)	can be set to “true” and included in a SIP MESSAGE to indicate that the emergency alert or cancellation was received successfully;

e)	an <mc-org> of type “xs:string” can be:

i)	set to the MCData user's Mission Critical Organization and included in an emergency alert sent by the MCData server to terminating MCData clients;

The recipient of the XML ignores any unknown element and any unknown attribute.

***** Next change *****

## G.4.2	MCData emergency state

The MCData emergency state is managed by the MCData client and MCData user. High-level characteristics of this state are captured in table G.4.2-1.

Table G.4.2-1: MCData emergency state

| MCData emergency state | State-setting events | State-clearing events | Comments |
| --- | --- | --- | --- |
| Values: "set": MCData user is in a life-threatening situation "clear": MCData user is not in a life-threatening situation Managed by: MCData client and MCData user | MCData emergency alert initiated MCData emergency communication initiated | MCData emergency alert cancelled (by initiator) MCData emergency alert cancelled (by authorised-user) MCData emergency communication cancelled by initiator (if there is no outstanding MCData emergency alert) MCData user manually clears the state | While the MCData client is in the MCData emergency state, all group communications it makes will be MCData emergency group communications, providing the group is authorised for MCData emergency group communications. While in an emergency group communication while in the MCData emergency state, the MCData user is an emergency participant and will have pre-emptive priority over non-emergency participants in the emergency group communication. |

## G.4.3	In-progress emergency group state

This state conforms with 3GPP TS 23.282 [2]. It is managed by the controlling MCData function. High-level characteristics of this state are captured in table G.4.3-1.

Table G.4.3-1: in-progress emergency group state

| In-progress emergency group state values | State-entering events | Comments |
| --- | --- | --- |
| "true" | acceptance by the controlling MCData function of an MCData emergency group communication request. |  |
| "false" | initial state prior to any communication activity acceptance by the controlling MCData function of an MCData emergency group cancel request. |  |

## G.4.4	MCData emergency group state

The MCData emergency group state is the MCData client's perspective of the in-progress emergency group state which is managed by the controlling MCData function. The MCData emergency group (MDEG) state is managed by the MCData client to enable the requesting of MCData emergency-level priority as early as possible in the origination of an MCData emergency group communication. High-level characteristics of this state are captured in table G.4.4-1.

Table G.4.4-1: MCData emergency group state

| MCData emergency group state values | State-entering events | Comments |
| --- | --- | --- |
| MDEG 1: no-emergency | initial state prior to any communication activity Emergency group communication cancel request received on behalf of another user from the MCData server Emergency group communication cancel response (success) in response to initiator's request |  |
| MDEG 2: in-progress | Emergency group communication response received (confirm) to initiator's emergency group communication request Emergency group communication request received (on behalf of another user) | In this state, all participants in communications on this group will receive emergency level priority whether or not they are in the MCData emergency state themselves. |
| MDEG 3: cancel-pending | Emergency group communication cancel request sent by initiator | The controlling MCData server may not grant the cancel request for various reasons, e.g., other users in an MCData emergency state remain in the communication. |
| MDEG 4: confirm-pending | Emergency group communication request sent by initiator | The controlling MCData server may not grant the request for various reasons, e.g., the MCData group is not configured as being emergency-capable so it can't be assumed that the group will enter the in-progress state. |

## G.4.5	MCData emergency group communication state

Table G.4.5-1 provides the semantics of the MCData emergency group communication (MDEGC) state values. This is an internal state of the MCData client and is managed by the MCData client. This state variable aids in the managing of the information elements of MCData emergency group communications and MCData emergency alerts and their cancellations.

Table G.4.5-1: MCData emergency group communication state

| MCData emergency group communication state values | Semantics | Comments |
| --- | --- | --- |
| MDEGC 1: emergency-gc-capable | MCData emergency-capable client is not currently in an MCData emergency group communication that it has originated, nor is it in the process of initiating one. | MCData emergency state: may or may not be set in this state, depending upon the MCData client's MDEA state |
| MDEGC 2: emergency-communication-requested | MCData client has initiated an MCData emergency group communication request. | MCData emergency state: is set |
| MDEGC 3: emergency-communication-granted | MCData client has received an MCData emergency group communication grant. | If the MCData user initiates a communication while the MCData emergency state is still set, that communication will be an MCData emergency group communication, assuming that the group is authorised for the client to initiate emergency group communications on. MCData emergency state: is set |

***** End of change *****