- 3GPP TSG CT WG1 Meeting 124-e	TDoc C1-203417

- Electronic meeting, 2-10 June 2020

Title:	LS on mandate to provide "any PLMN" entry in the non-3GPP access node selection information

Release:	Rel-16

Work Item:	5GProtoc16

Source:	CT1#124-e

To:	SA2

Contact person:	John-Luc Bakker

JBakker@blackberry.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N.A.

# 1	Overall description

CT1 have discussed the requirement that mandates the inclusion of the "any PLMN" entry in the non-3GPP access node selection information (TS 23.501).

The equivalent of the "any PLMN" entry in the ePDG selection information is optional (see TS 23.402).

Mandatory inclusion of the "any PLMN" entry in the non-3GPP access node selection information implies that Home-routed or LBO Roaming scenarios (e,g. presented in Figures 4.2.8.2.2-2 and 4.2.8.2.3-2 of TS 23.501) are not supported when the UE is roaming, the N3AN node configuration information is provisioned, the UE is registered to a VPLMN with 5GS (or EPS), and the HPLMN configured the UE with e.g. specific N3IWF or ePDG IP address or FQDN.

Some companies believe that, for such case(s), unless selection of the N3IWF or ePDG in the visited country is required, Home-routed or LBO Roaming should be enabled and controlled by the HPLMN's policy, and believe that the HPLMN's policy can be expressed by omitting the 'Any_PLMN' N3AN node selection information entry in the N3AN node configuration information.

2	Actions

To SA2

ACTION: 	3GPP TSG CT WG1 asks SA2 group to confirm whether the presence of the "any PLMN" entry in the non-3GPP access node selection information is mandatory.

# 3	Dates of next TSG CT WG1 meetings

TSG CT WG1 Meeting 125	USA	24-28 August 2020

TSG CT WG1 Meeting 126	India	12-16 October 2020