- 3GPP TSG CT WG1 Meeting 124-e	TDoc C1-20xxxx

- Electronic meeting, 2-10 June 2020

Title:	Inclusion of pending S-NSSAI(s) in the requested NSSAI

Response to:

Release:	Rel-16

Work Item:	eNS

Source:	3GPP CT1

To:	3GPP SA2

Cc:

Contact person:	Lin Shu

shulin@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

CT1 has observed following requirement in TS 23.501 sub 5.15.5.2.1:

"The UE shall not include in the Requested NSSAI any of the S-NSSAIs from the Pending NSSAI the UE stores, regardless of the Access Type."

CT1 also has following NAS protocol design principles related to requested NSSAI and pending NSSAI:

- If the UE intends to register to slice(s), the UE shall include all their S-NSSAI(s) in the requested NSSAI to the network;

- If the network receives a requested NSSAI from the UE with at least one S-NSSAI allowed, the network will treat it as a complete requested NSSAI to create allowed NSSAI, rejected NSSAI, and/or pending NSSAI;

- If the network does not receive a requested NSSAI from the UE or none of requested NSSAI is allowed, the network will use default S-NSSAIs to create allowed NSSAI and/or pending NSSAI;

- The UE can change the slice(s) it is currently registered to and then trigger a new mobility registration update procedure;

- The pending NSSAI is maintained as access agnostic.

During the stage 3 NAS protocol implementation, CT1 has identified following questions if stage 3 is going to above stage 2 requirement based on above NAS protocol design principles.

Below example is taken for questions:

- UE has configured NSSAI = {A, B, C, D}

- UE sent requested NSSAI = {A, B} over 3GPP access

- UE received pending NSSAI = {A, B}

Question #1: Before the completion of network slice-specific authentication and authorization (NSSAA) for {A, B}, if the UE needs to initiate a new mobility registration update procedure to register to different slices over 3GPP access, e.g. to register to {A, B, C} (C is a new one) or {A, C} (previous requested B was revoked), then what should the UE do?

Question #2: Before the completion of NSSAA for {A, B}, if the UE has to initiate a new mobility registration update procedure to register to same slices {A, B} over 3GPP access (e.g. the UE moves out of the current TAI list), then what should the UE do?

Question #3: Before the completion of NSSAA for {A, B} over 3GPP access, if the UE has to register to non-3GPP access for slices which are overlapped with {A, B}, e.g. {A, B}, {A, B, C} or {A, C}, then what should the UE do?

Question #4: In above questions, as per above SA2 requirement, if the UE is allowed to include an incomplete requested NSSAI (e.g. only {C}) or no requested NSSAI to the network, then what should the network do for creating allowed NSSAI, rejected NSSAI, and/or pending NSSAI?

# 2	Actions

To 3GPP SA2

ACTION: 	3GPP CT1 kindly asks SA2 to answer above questions.

# 3	Dates of next TSG CT WG1 meetings

TSG CT WG1 Meeting 125-e 	e-meeting	20-28 August 2020

TSG CT WG1 Meeting 126-e	TBD