3GPP TSG-CT WG1 Meeting #126-e	C1-205915

- Electronic meeting; 15-23 October 2020

Source:	Qualcomm Incorproated

Title:	Key issue #5: Handling of new RAT type(s)

Spec:	3GPP TR 24.821

Agenda item:	17.2.4

Document for:	Agreement

1. Introduction

This contribution proposed to add Key Issue #5 Handling of new RAT types.

2. Reason for Change

In [1], SA2 has agreed on introducing new RAT types for satellite access: NR(LEO), NR(MEO), NR(GEO) and NR(OTHERSAT). These RAT types are associated with network/service capabilities (e.g. QoS, charging) as well as the UE capabilites (e.g. tx power). The impact of the new RAT type(s) on PLMN selection needs to be evaluated.

The following aspects need to be studied:

- Which satellite access RAT types are relevant for PLMN selection? In the simlest form, it may be sufficient to differentiate only terrestrial access vs. satellite access. In a more granular scheme, it would be possible to differentiate all four satellite access RAT types.

- What is the prioritization of the new RAT type and how it is implemented?

For PLMN selection for satellite access, handling of new RAT type(s) is a valid issue for which the solution needs to be provided.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.821.

4. References

[1]	S2-2006591 General description for NR based on satellite constellations (XiaoMi)

* * * First Change * * * *

# 5.5	Key issue #5: Handling of new RAT type(s)

SA2 has agreed on introducing new RAT types for satellite access: NR(LEO), NR(MEO), NR(GEO) and NR(OTHERSAT). These RAT types are associated with network/service capabilities (e.g. QoS, charging) as well as the UE capabilites (e.g. tx power). The impact of the new RAT type(s) on PLMN selection needs to be evaluated.

The following questions are expected to be studied within this key issue:

-	which satellite access RAT types are used for PLMN selection:

-	all four satellite access RAT types specified in 3GPP TS 23.501 [4];

-	one generic satellite access RAT type is sufficient; or

-	no new RAT type is needed.

-	if more than one RAT types are used for PLMN selection, what is the prioritization of the new RAT types and how it is implemented in the PLMN selection procedure?

-	what information is configured in the UE related to RAT prioritization.