3GPP TSG-CT WG1 Meeting #127e	C1-XXXXXX

Electronic meeting, 13-20 Nov 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.623 | CR | 0079 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding handling of the UE configuration parameter “Access_Point_Name_Parameter_Reading_Rule“ for the UE to read the XCAP APN name parameter from correct input source. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2020/11/04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | During CT1#125e meeting, the new configuration parameter “Access_Point_Name_Parameter_Reading_Rule “ was agreed in UE via tDoc “C1-205389 “. Using this configuration parameter, the network can configure the UE to use the correct input source for reading access point name parameter. Since the UE can have multiple inputs to read APN parameters (eg UICC, Settings menu or Netowork provided), the handling of this configuraiton paramter will be crtical in UE to select correct input source for reading the APN name and add it in the Access point name IE. Via this tDoc, we are adding the handling of this configuration parameter in UE to correctly read Access point name parameter from correct input source. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added handling of the new configuration paramter “Access_Point_Name_Pameter_Reading_Rule “ so that UE can select correct input source to read APN paramter and add it in the Access point name IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The open market UE will fail to add correct APN name in Access point name IE when UE has multiple input sources having different Access point name values. This will critically impact end user experience. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 24.229 CR 6476 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Next change *****

#### 5.2.1.3	Policy on access type used for XCAP

The policy on access type used for the XCAP enables HPLMN control of access used for XCAP messages.

The policy on access type used for the XCAP can be set to one of the following values:

a)	any access type;

b)	3GPP accesses only;

c)	EPC or 5GCN via WLAN IP-CAN only;

d)	Non-seamless WLAN offload only;

e)	3GPP accesses preferred, non-seamless WLAN offload as secondary; and

f)	3GPP accesses preferred, EPC or 5GCN via WLAN IP-CAN as secondary.

The UE may support the policy on access type used for the XCAP.

If the UE supports the optional configuration parameter "Access_Point_Name_Parameter_Reading_Rule", as defined in 3GPP TS 24.167 [19] and has been configured with this parameter, then the UE shall use it to retrieve the access point name and use it in PDP context activation procedure or in PDN connection procedure for XCAP.

If the UE supports the policy on access type used for the XCAP:

a)	when the UE needs to send an XCAP request:

1)	if the policy on access type used for the XCAP is set to "any access type", the UE shall send XCAP requests from an IP address associated with a bearer of any access;

2)	if the policy on access type used for the XCAP is set to "3GPP accesses only":

A)	the UE shall attempt to obtain a PDP context for XCAP as specified in 3GPP TS 24.008 [29] or a EPS bearer context for XCAP as specified in 3GPP TS 24.301 [30] or a 5GS QoS flow using NG-RAN as specified in 3GPP TS 24.501 [33];

B)	if the UE obtains the PDP context for XCAP or the EPS bearer context for XCAP or the 5GS QoS flow using NG-RAN, the UE shall send XCAP requests from an IP address associated with the obtained PDP context for XCAP or the obtained EPS bearer context for XCAP or the 5GS QoS flow using NG-RAN; and

C)	if the UE cannot obtain the PDP context for XCAP or the EPS bearer context for XCAP or the 5GS QoS flow using NG-RAN, the UE shall not send XCAP request;

3)	if the policy on access type used for the XCAP is set to "EPC or 5GCN via WLAN IP-CAN only":

A)	the UE shall attempt to associate with a WLAN as specified in 3GPP TS 24.302 [31] or as specified in 3GPP TS 24.502 [34] and the UE shall attempt to obtain:

-	a PDN connection for XCAP as specified in 3GPP TS 24.302 [31], or 3GPP TS 24.244 [32]; or

-	a 5GS QoS flow using WLAN as specified in 3GPP TS 24.502 [34];

B)	if the UE obtains the PDN connection or 5GS QoS flow for XCAP, the UE shall send XCAP requests from an IP address associated with the PDN connection or 5GS QoS flow for XCAP; and

C)	if the UE cannot obtain the PDN connection or 5GS QoS flow for XCAP, the UE shall not send an XCAP request;

4)	if the policy on access type used for the XCAP is set to "Non-seamless WLAN offload only":

A)	the UE shall attempt to associate with a WLAN as specified in 3GPP TS 24.302 [31];

B)	if the UE associates with a WLAN and the WLAN is either untrusted non-3GPP access or the UE established NSWO via TWAN, the UE shall send XCAP requests from an IP address associated with the WLAN; and

C)	if the UE cannot associate with a WLAN or the UE associates with TWAN and NSWO is not available, the UE shall not send an XCAP request;

5)	if the policy on access type used for the XCAP is set to "3GPP accesses preferred, non-seamless WLAN offload as secondary":

A)	the UE shall attempt to obtain a PDP context for XCAP as specified in 3GPP TS 24.008 [298] or a EPS bearer context for XCAP as specified in 3GPP TS 24.301 [30] or a 5GS QoS flow for XCAP as specified in 3GPP TS 24.501 [33];

B)	if the UE obtains the PDP context for XCAP or the EPS bearer context for XCAP or a 5GS QoS flow for XCAP, the UE shall send XCAP requests from an IP address associated with the obtained PDP context for XCAP or the obtained EPS bearer context for XCAP; and

C)	if the UE cannot obtain the PDP context for XCAP or the EPS bearer context for XCAP or a 5GS QoS flow for XCAP:

i)	the UE shall attempt to associate with a WLAN as specified in 3GPP TS 24.302 [31];

ii)	if the UE associates with a WLAN and the WLAN is either untrusted non-3GPP access or the UE established NSWO via TWAN, the UE shall send XCAP requests from an IP address associated with the WLAN; and

iii)	if the UE cannot associate with a WLAN or the UE associates with TWAN and NSWO is not available, the UE shall not send an XCAP request; and

6)	if the policy on access type used for the XCAP is set to "3GPP accesses preferred, EPC or 5GCN via WLAN IP-CAN as secondary":

A)	the UE shall attempt to obtain a PDP context for XCAP as specified in 3GPP TS 24.008 [29] or a EPS bearer context for XCAP as specified in 3GPP TS 24.301 [30] or a 5GS QoS flow using NG-RAN as specified in 3GPP TS 24.501 [33];

B)	if the UE obtains the PDP context for XCAP or the EPS bearer context for XCAP or the 5GS QoS flow for XCAP, the UE shall send XCAP requests from an IP address associated with the obtained PDP context for XCAP or the obtained EPS bearer context for XCAP or the 5GS QoS flow using NG-RAN; and

C)	if the UE cannot obtain the PDP context for XCAP or the EPS bearer context for XCAP or the 5GS QoS flow using NG-RAN:

i)	the UE shall attempt to associate with a WLAN as specified in 3GPP TS 24.302 [31] or as specified in 3GPP TS 24.502 [34] and the UE shall attempt to obtain:

-	a PDN connection for XCAP as specified in 3GPP TS 24.302 [31], or 3GPP TS 24.244 [32]; or

-	a 5GS QoS flow using WLAN as specified in 3GPP TS 24.502 [34];

ii)	if the UE obtains the PDN connection or 5GS QoS flow for XCAP, the UE shall send XCAP requests from an IP address associated with the PDN connection or 5GS QoS flow for XCAP; and

iii)	if the UE cannot obtain the PDN connection or 5GS QoS flow for XCAP, the UE shall not send an XCAP request; and

b)	the UE may support being configured with the policy on access type used for the XCAP using one or more of the following methods:

a)	the EFXCAPConfigData  file described in 3GPP TS 31.102 [15B];

b)	the EFXCAPConfigData file described in 3GPP TS 31.103 [15A]; and

c)	the AccessForXCAP node of 3GPP TS 24.424 [22].

If the UE is configured with both the AccessForXCAP node of 3GPP TS 24.424 [22] and the EFXCAPConfigData file described in 3GPP TS 31.102 [15B] or 3GPP TS 31.103 [15A], then the EFXCAPConfigData file shall take precedence.

NOTE:	Precedence for files configured on both the USIM and ISIM is defined in 3GPP TS 31.102 [15B].