3GPP TSG-CT WG1 Meeting #127-e	C1-XXXX

Electronic meeting, 13-20 November 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.173 | CR | 0145 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of lower layer congestion notification for MMTEL video |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2020-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | In case UE need to perform service request procedure or tracking area updating procedure for initiating an MO-MMTEL video call, it’s possible that network side is congested with following situations: - network rejects procedure with EMM cause #22 (Congestion); or - UE fails to initiate service request procedure due to T3346 or T3325 running; or - UE fails to initiate registration procedure for mobility and periodic registration update due to T3346 running Specifications describe handling for MMTEL voice session but do not specify similar handling for MMTEL video session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Abnormal case J.2.1.4 and M.2.1.3 enhanced to handle EMM provided notification for MO-MMTEL video call failed due to congestion, T3346 or T3325 running. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It could be a unacceptable long period for user to wait before MO-MMTEL video call can be initiated if it failed due to congestion or T3325 is running. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | J.2.1.4 and M.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 24.501 CR 2811 and TS 24.301 CR 3463 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Next change *****

### J.2.1.4	Abnormal cases

Upon request from a user to establish a MMTEL voice or an MMTEL video session as described in subclause 5.2, if:

a)	the UE receives, from the lower layers, a notification that the service request:

1)	was not accepted due to congestion; or

2)	resulted in starting timer T3325 (see 3GPP TS 24.008 [80]); and

b)	an originating initial INVITE transaction, for which a 2xx response has not yet been received, exists;

then:

a)	if a provisional response has been received, the UE shall cancel INVITE transaction; and

b)	if an alternative radio access network is available:

1)	if the service request was not accepted due to congestion, the UE shall attempt the MMTEL voice or an MMTEL video session on the alternative radio access network; or

2)	if the service request resulted in starting timer T3325, the UE should attempt the MMTEL voice or an MMTEL video session on the alternative radio access network.

***** Next change *****

### M.2.1.3	Abnormal cases

Upon request from a user to establish an MMTEL voice or an MMTEL video session as described in subclause 5.2, if

a)	the UE receives, from the lower layers, a notification that the service request

1)	was not accepted due to congestion; or

2)	resulted in starting timer T3525 (see 3GPP TS 24.501 [42]); and

b)	an originating initial INVITE transaction, for which a 2xx response has not yet been received, exists;

then:

a)	if a provisional response has been received, the UE shall cancel the INVITE transaction; and

b)	if an alternative radio access network is available, the UE shall attempt the MMTEL voice or an MMTEL video session on the alternative radio access network.