- 3GPP TSG CT WG1 Meeting 127-e	TDoc C1-207515

- Electronic meeting, 13-20 November 2020

Title:	LS on Private call transfer

Release:	Rel-17

Work Item:	eMONASTERY2

Source:	CT1#127-e

To:	SA6

Contact person:	Peter Beicht

Peter.Beicht@kontron.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

The procedure in TS 23.379 v17.4.0 subclause 10.7.6.2.1 indicates that Client 2 sends a message to the MCPTT server in step 3 that results in the MCPTT server managing the removal of the existing private call between Client 1 and Client 2. This approach adds unnecessary complexity to the MCPTT server. A simpler approach is to have Client 2 indicate back to Client 1 the MCPTT ID the new target of the call. Client 1 and Client 2 can then end their private call using existing client initiating procedures. As currently defined, Client 1 can initiate a new private call to Client 3 using the MCPTT ID provided (possibly translated from a functional alias in the MCPTT server). By keeping the overall procedure simple, possible impacts on future features are minimised.

Similar simplifications can be applied to the procedure in TS 23.379 v17.4.0 subclause 10.7.6.2.2.

CT1 respectfully asks SA6 if there are any good reasons for the server to initiate the call release between client 1 and 2, and not for the clients to initiate the call release? If it is acceptable for SA6 that the clients initiate the call release of the call between client 1 and 2, CT1 kindly asks SA6 to change the procedures do the call release client based.

# 2	Actions

To SA6:

ACTION: 	CT1 respectfully requests that the procedures in subclauses 10.7.6.2.1 and 10.7.6.2.2 be modified per the approach described above that the call between client 1 and client 2 is released by the clients instead of by the server.

# 3	Dates of next TSG CT WG1 meetings

TSG CT WG1 Meeting 127-bis-e (electronic)	25-29 January 2021

TSG CT WG1 Meeting 128-bis-e (electronic)	25 February – 5 March 2021