- 3GPP TSG CT WG1 Meeting 127-e	TDoc C1-207578

- Electronic meeting, 13-20 November 2020

Title:	LS on Storage of KAUSF

Response to:	---

Release:	Rel-17

Work Item:	Stage-3 5GS NAS protocol development for Rel-17 (5GProtoc17)

Source:	CT1

To:	SA3

Cc:	---

Contact person:	Robert Zaus

rzaus at apple dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

At CT1#127-e, CT1 discussed the requirements for storing KAUSF, KSEAF, SOR counter and UE parameter update counter on the USIM (or in the non-volatile memory of the ME, if no corresponding file is present on the USIM).

Currently, TS 24.501, annex C, specifies that the SOR counter and the UE parameter update counter shall be stored on the USIM, but there is no requirement when exactly the ME shall store these 2 counters and the associated keys KAUSF and KSEAF on the USIM.

According to TS 33.501, clause 3.1, the keys KAUSF and KSEAF are not part of the 5G NAS security context, so the requirements in TS 33.501, clause 6.8.1.1.1, Transition from RM-REGISTERED to RM-DEREGISTERED, for storing the 5G NAS security contexts are not applicable.

For the case when the network uses EAP based authentication, CT1 assumes that the ME can use the receipt of the EAP-success message as trigger to store KAUSF, KSEAF, the SOR counter and the UE parameter update counter on the USIM.

But CT1 could not reach a conclusion for the case when the network uses 5G AKA based authentication. For this case, CT1 has considered the following options:

a)	Store KAUSF, KSEAF, the SOR counter and the UE parameter update counter on the USIM when the UE has calculated the authentication response RES from the 5G authentication challenge data and sends the Authentication Response.

b)	Store KAUSF, KSEAF, the SOR counter and the UE parameter update counter on the USIM when the UE receives the first NAS Security Mode Command creating a full native 5G NAS security context derived from KAUSF or when the UE receives and successfully verifies the integrity of the first SOR container or UE Parameter Update container protected with the new KAUSF.

c)	Store KAUSF, KSEAF, the SOR counter and the UE parameter update counter on the USIM when the UE when the UE enters state 5GMM-DEREGISTERED from any other state except 5GMM-NULL (similar to the requirement for a full native 5G NAS security context).

Option a) and c) have the drawback that in certain cases the UE will override the old KAUSF, although the network determines that the UE failed the authentication and on the network side the old KAUSF remains valid.

Option b) has the drawback that in certain cases, if the UE performs a deregistration with power-off before it stored the new KAUSF on the USIM, the UE and the AUSF can get out-of-sync with regard to the KAUSF considered valid.

Question: Can SA3 provide guidance when and under which conditions the UE should store KAUSF, KSEAF, SOR counter and UE parameter update counter on the USIM?

# 2	Actions

To SA3

ACTION: 	3GPP TSG CT WG1 kindly asks SA3 to provide guidance on when and under which conditions the UE should store KAUSF, KSEAF, SOR counter and UE parameter update counter on the USIM?

# 3	Dates of next TSG CT WG1 meetings

TSG CT WG1 Meeting 127-bis-e (electronic)	25-29 January 2021

TSG CT WG1 Meeting 128-e (electronic)	25 February – 5 March 2021