| 3GPP TR 24.821 V0.4.0 (2021-03) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Study on PLMN selection for satellite access (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	9

3.3	Abbreviations	9

4	Deployment scenarios for PLMN selection	9

4.1	Scenario A	9

4.2	Scenario B	9

4.3	Scenario C	10

4.4	Scenario D	11

5	Key issues	12

5.1	Key issue #1: Determination of the country of the UE’s location	12

5.2	Key issue #2: Use of core network of PLMN in country where UE is located	12

5.3	Key issue #3: PLMN selection in international areas	13

5.4	Key issue #4: Handling of shared/global PLMN IDs	13

5.5	Key issue #5: Handling of new satellite RAT	14

5.6	Key issue #6: PLMN search for satellite access	14

5.7	Key issue #7: Emergency calls	14

6	Solutions	15

6.1	Mapping of solutions to key issues	15

6.2	Solution 1	15

6.2.1	Target key issue	15

6.2.2	Solution description	15

6.2.3	Impacts to existing procedures due to the solution	16

6.3	Solution 2: Determining PLMNs that can be selected in the country of UE location based on USIM configuration	16

6.3.1	Target key issue	16

6.3.2	Solution Description	16

6.3.3	Impacts	17

6.4	Solution 3	17

6.4.1	Target key issue	17

6.4.2	Solution description	17

6.4.3	Impacts to existing procedures due to the solution	18

6.5	Solution 4: PLMN selection inside international areas	18

6.5.1	Target key issue	18

6.5.2	Solution description	18

6.5.3	Impacts	19

6.6	Solution 5	19

6.6.1	Target key issue	19

6.6.2	Solution description	19

6.6.3	Impacts to existing procedures due to the solution	20

6.7	Solution 6: Handling of PLMNs with shared MCC	20

6.7.1	Target Key Issue	20

6.7.2	Solution description	20

6.7.3	Impacts	20

6.8	Solution 7	20

6.8.1	Target key issue	20

6.8.2	Solution description	20

6.8.3	Impacts to existing procedures due to the solution	21

6.9	Solution 8: Range of values for timer T for UE in satellite access	21

6.9.1	Introduction	21

6.9.2	Solution description	21

6.9.3	Impacts	22

6.10	Solution 9: Determination of the country of the UE’s location	22

6.10.1	Target key issue	22

6.10.2	Solution description	22

6.10.2.1	Determination of the UE geolocation	22

6.10.2.2	Determination of the country of the UE location	22

6.10.3	Impacts to the existing procedures due to the solution	23

6.11	Solution 10	23

6.11.1	Target key issue	23

6.11.2	Solution description	23

6.11.3	Impacts to existing procedures due to the solution	23

6.12	Solution 11: PLMN selection inside international areas	24

6.12.1	Target key issue	24

6.12.2	Solution description	24

6.12.3	Impacts	24

6.13	Solution 12	24

6.13.1	Target key issue	24

6.13.2	Solution Description	24

6.13.3	Impacts	25

6.14	Solution 13	25

6.14.1	Target key issue	25

6.14.2	Solution Description	25

6.14.3	Impacts	26

6.15	Solution 14	26

6.15.1	Target key issue	26

6.15.2	Solution Description	26

6.15.3	Impacts	27

6.16	Solution 15: Access technology selection based on PLMN Selector list with wildcarded PLMN ID	27

6.16.1	Target key issue	27

6.16.2	Solution Description	27

6.16.3	Impacts	28

6.17	Solution 16: Restrictions in camping on an acceptable cell	29

6.17.1	Target key issue	29

6.17.2	Solution description	29

6.17.3	Impacts to existing procedures due to the solution	29

6.18	Solution 17: PLMN search for satellite access	29

6.18.1	Target key issue	29

6.18.2	Solution description	29

6.18.3	Impacts	30

6.19	Solution 18	30

6.19.1	Target key issue	30

6.19.2	Solution Description	31

6.19.3	 Impacts	31

6.20	Solution 19: Use of network indication of MCC(s)	31

6.20.1	Target key issue	31

6.20.2	Solution Description	32

6.20.3	 Impacts	32

6.21	Solution 20: Handling or RAT prioritization	32

6.21.1	Target key issue	32

6.21.2	Solution description	32

6.21.3	Impacts to the existing procedures due to the solution	34

6.22	Solution 21: Use of user and device settings for searching Satellite access or Terrestrial access PLMNs prior to PLMN selection	34

6.22.1	Target key issue	34

6.22.2	Solution description	34

6.22.3	Impacts	35

7	Evaluation	35

7.1	Evaluation of Solutions for Key Issue #1	35

7.2	Evaluation of Solutions for Key Issue #2	36

7.3	Evaluation of Solutions for Key Issue #3	36

7.4	Evaluation of Solutions for Key Issue #4	37

7.5	Evaluation of Solutions for Key Issue #5	37

7.6	Evaluation of Solutions for Key Issue #6	38

7.7	Evaluation of Solutions for Key Issue #7	38

8	Conclusion	38

Annex A (informative): Change history	39

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

This technical report studies the key issues and the respective solutions related to the support for network selection procedure that accommodates 5G systems using satellite radio access networks. The objective is to:

-	identify the specifics of the satellite access networks with respect to the legacy 3GPP access networks that are relevant to the PLMN selection procedure; and

-	consider the need to modify the legacy PLMN selection procedure to address the identified specifics.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[3]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[4]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2"

[5]	3GPP TR 23.737: "Study on architecture aspects for using satellite access in 5G".

[6]	ITU-T E.212 The international identification plan for public networks and subscriptions (09/2016).

[7]	ITU RR-2020-00013-Vol.1-EA5 Radio Regulations, Edition of 2020

[8]	S3i200056 Response LS on the "LS OUT on Location of UEs and associated key issues"

[9]	3GPP TS 31.102: "Characteristics of the USIM Application"

[10]	3GPP TS 24.368: "Non-Access Stratum (NAS) configuration Management Object (MO)"

[11]	3GPP TS 22.011: "Service accessibility"

[12]	3GPP TS 22.261: "Service requirements for the 5G system"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3GPP satellite access: access network using satellite-mounted transmitters and one of the following RAT types: NR(LEO), NR(MEO), NR(GEO) and NR(OTHERSAT). 3GPP satellite access is considered a 3GPP access.

International areas: geographical areas, international waters and international airspace that are not under the jurisdiction of any country.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

NTN	non-terrestrial network

PLMN	public landline mobile network

PLMN ID	PLMN identity

TN	terrestrial network

# 4	Deployment scenarios for PLMN selection

## 4.1	Scenario A

In Scenario A, shown in Figure  4.1.-1, a satellite access cell is contained within a single country (Country A). This scenario is expected to be deployed frequently. It would happen when a country is large enough to accommodate an entire satellite beam within its borders. The PLMN ID broadcast in the cell has MCC of Country A. There is a possibility of un-intended cross-border leakage into adjacent countries (Country B and Country C).

Figure 4.1-1: Deployment scenario A

## 4.2	Scenario B

In Scenario B, shown in Figure  4.2-1, a satellite access cell spans across multiple countries (Country A, Country B and Country C). There is a possibility of un-intended cross-border leakage into adjacent countries (Country D). This scenario is expected to be deployed often in parts of the world where smaller countries are bordering each other, such as in Europe, Central America, Middle East.

Figure 4.2-1: Deployment scenario B

There are the following options for the PLMN ID broadcast in the cell:

Scenario B1: Three PLMN IDs are broadcast, with three different MCCs corresponding to Country A, Country B and Country C, respectively. This scenario assumes that the PLMN operator is registered and obtains MNC in each country.

Scenario B2: Single PLMN ID corresponding to one country (Country A) is broadcast. Extra-territorial use of a PLMN ID, along with the conditions for such use, is defined in ITU-T E.212 [6] (see Annex E).

Scenario B3: PLMN ID with a global/shared PLMN ID (with MCC 9xx) is broadcast. The use of global/shared PLMN ID, along with the associated conditions, is defined in ITU-T E.212 [6] (see Annex A and subclause F.6).

## 4.3	Scenario C

In Scenario C, shown in Figure  4.3-1, a satellite access cell spans across a country (Country A), its contiguous area (water or airspace) and leaks into adjacent international area. The leakage into international area may be intentional or unintentional. This scenario is expected to be deployed in larger countries that have access to oceans and large seas.

Figure 4.3-1: Deployment scenario C

The PLMN ID broadcast in the cell has MCC of Country A.

## 4.4	Scenario D

In Scenario D, shown in Figure  4.4-1, a satellite access cell is dedicated to the coverage of international areas. There may be leakage into Country A or its contiguous area.

Figure 4.3-1: Deployment scenario D

There are the following options for the PLMN ID broadcast in the cell:

Scenario D1: The PLMN ID broadcast in the cell is a global/shared PLMN ID (with MCC 9xx)

Scenario D2: The PLMN ID broadcast in the cell is the PLMN ID of the operator of the PLMN. The MCC will be the MCC of the home country of the PLMN.

This scenario presents several (non-exhaustive) challenges:

1)	frequency band used in international areas: according to ITU Radio Regulations [7], the frequency bands allocated to maritime mobile-satellite services is small relative to other types of mobile satellite services;

2)	interference management; and

3)	low power efficiency, due to beaming satellite signals into vast areas with very low subscriber density.

Due to the above challenges, it needs to be determined whether this deployment scenario should be considered. However, it should be noted that item1), 2) and 3) are not related to PLMN selection for satellite access.

# 5	Key issues

## 5.1	Key issue #1: Determination of the country of the UE’s location

In [8], SA3-LI has established the requirement to "enforce the use of a Core Network of PLMN in the country where the UE is physically located". This means that, if the UE is able to determine the country of its physical location, it needs to select a PLMN belonging to the country. This goes beyond the requirements in the legacy PLMN selection procedure in 3GPP TS 23.122 [3], where the country is defined by the MCC or a range of MCCs (see 3GPP TS 23.122 [3]), of the serving PLMN.

NOTE:	Where relaxation of regulation requiring UE to select PLMN of country it is physically located in exist, how UE can learn about this relaxation and if it is needed can also be part of this study

MCC-based determination of the country would not work for satellite access because:

-	a 3GPP satellite access cell may span across multiple countries;

-	there are global/shared PLMN IDs with MCC that does not correspond to any country (MCC 9xx); and

-	in international areas, the UE cannot rely on the MCC.

The following questions are expected to be studied within this key issue:

-	how the UE determines the country of its physical location and whether it is in the scope of 3GPP;

▪	what assumptions can/should be made about the methods for determining UE location (e.g. GNSS availability etc.) and the UE capabilities;

▪	how the UE maps its physical geo-location (e.g. lat/long) into a country of its location; and

▪	determination that the UE is located in international areas; and

-	to what extent and under which conditions the UE could still rely on the MCC of the serving PLMN to determine the country of its physical location.

## 5.2	Key issue #2: Use of core network of PLMN in country where UE is located

In [8], SA3-LI has established the requirement to "enforce the use of a Core Network of PLMN in the country where the UE is physically located".

The core network location verification can take place during the registration procedure (by AS or NAS). However, from the protocol efficiency and resource utilization efficiency point of view, CT1 can study if there are ways to minimizes the possibility of registration rejection due to selection onto PLMNs whose CNs are not in same physical country as the UE.

The following questions are expected to be studied within this key issue:

-	how the UE determines which PLMN(s) can be selected in determined country of its physical location (if it is determined that the UE is not in an international area);

-	what information could be available to the UE to help in selecting, in a given country, a PLMN whose CN is physically located in the same country as the UE:

▪	pre-configured in the UE; and

▪	provided by PLMN

.

The regulatory requirement mentioned in [8] cannot apply to PLMN selection in international areas (e.g. international waters and airspace) as international areas are not under the jurisdiction of any country, see definition in subclause 3.1. Hence this key issue is not relevant for international areas.

This key issue is related to Key issue #1.

## 5.3	Key issue #3: PLMN selection in international areas

Satellite access networks are expected to cover large portions of the globe. This includes international areas. Additionally, in [8], SA3-LI also provided requirements specific to international areas: "Any solution addressing extraterritorial (e.g. international maritime zone and aeronautical) use cases should provide means to notify the HPLMN on roaming in and out of those areas".

The legacy PLMN selection procedure in 3GPP TS 23.122 [3] does not address international areas, i.e. areas that do not belong to any country.

The following questions are expected to be studied within this key issue:

-	how the UE selects PLMN if it is determined that the UE is in an international area; and

-	what information could be available to aid in the PLMN selection in international areas:

▪	pre-configured in the UE; and

▪	provided by PLMN;

This key issue is related to Key issue #1.

## 5.4	Key issue #4: Handling of shared/global PLMN IDs

PLMN IDs with MCC = 9xx are shared/global PLMN IDs. They are not associated with any country and are assigned by ITU. Satellite access networks providing coverage globally are expected to use these PLMN IDs.

The legacy PLMN selection procedure does not fully accommodate shared/global PLMN IDs:

-	the definition of the "country" in 3GPP TS 23.122 [3] is based on MCC or a range of MCCs; and

-	According to the current definition in 3GPP TS 23.122 [3], subclause 4.4.3.3, the UE in a VPLMN shall limit its searches for higher priority PLMN of the same country (as defined in 3GPP TS 23.122 [3], Annex  B) as the serving VPLMN. So a UE served by a VPLMN with a MCC not in the global/shared MCC range would not be able to re-select to a PLMN using global/shared MCC, and vice-versa.

The following questions are expected to be studied within this key issue:

-	what modifications are needed to the PLMN selection procedure to handle shared/global PLMN IDs;

-	should these changes also apply to the legacy PLMN selection procedure;

-	what information could be available to aid in handling shared/global PLMN IDs:

▪	pre-configured in the UE; and

▪	provided by PLMN; and

-	how to accommodate LI requirements when handling shared/global PLMN IDs.

## 5.5	Key issue #5: Handling of new satellite RAT

SA1 has specified a new Access Technology Identifier "satellite NG-RAN" to be used in the Operator Controlled PLMN Selector list and the User Controlled PLMN Selector list in USIM (see 3GPP TS 22.011 [11]). SA2 has specified deployment scenarios in which a PLMN deploys both terrestrial and satellite access with overlapping coverage (see 3GPP TR 23.737 [5]).

NOTE:	SA2 has agreed on introducing new core network RAT types for satellite access: NR(LEO), NR(MEO), NR(GEO) and NR(OTHERSAT). The core network RAT types are not expected to have an impact on the PLMN selection procedure.

The impact of the new satellite RAT in USIM on PLMN selection procedure needs to be evaluated.

The following questions are expected to be studied within this key issue:

-	what is the prioritization of the new RAT and how it is implemented in the PLMN selection procedure?

-	what information is configured in the UE related to RAT prioritization.

## 5.6	Key issue #6: PLMN search for satellite access

The range for the search interval for periodic search for higher priority PLMNs is specified in TS 23.122 [3]. Currently, there are two different search interval ranges: for non-NB-IoT UE, and for NB-IoT UE, respectively. What interval will be used for PLMN search in satellite access needs to be studied.

In the legacy PLMN selection procedure, the UE searches for higher priority PLMN only among PLMNs with the same country as the RPLMN. It needs to be studied whether this rule should remain for satellite access.

The following questions are expected to be studied within this key issue:

-	Whether there are different priority levels for available PLMNs in satellite access?

-	Whether the periodic search for higher priority PLMNs in satellite access is needed?

-	Whether the need to have a separate range of values for the search interval for periodic search for higher priority PLMNs in satellite access; and

-	Whether the search for higher priority PLMN should be limited to PLMNs where MCC matches the MCC or country of the RPLMN.

## 5.7	Key issue #7: Emergency calls

Satellite access networks are expected to cover the large geographical areas which cover some countries and several PLMNs from the legacy communication system perspective. To support emergency call services for UE might be influenced by adopting satellite access. Therefore, it is needed to study for the routing of emergency call to appropriate PSAP in the case of adopting satellite access.

The following questions are expected to be studied within this key issue:

-	What modifications in automatic PLMN selection mode are necessary for a UE using satellite access unable to obtain normal service from a PLMN?

6	Solutions

## 6.1	Mapping of solutions to key issues

Table 6.1-1: Mapping of solutions to key issues

|  | Key issue |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solution | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | X |  |  |  |  |  |  |
| 2 |  | X |  |  |  |  |  |
| 3 |  | X |  |  |  |  |  |
| 4 |  |  | X |  |  |  |  |
| 5 |  |  | X |  |  |  |  |
| 6 |  |  |  | X |  |  |  |
| 7 |  |  |  | X |  |  |  |
| 8 |  |  |  |  |  | X |  |
| 9 | X |  |  |  |  |  |  |
| 10 | X |  |  |  |  |  |  |
| 11 | X |  |  |  |  |  |  |
| 12 |  |  |  |  | X |  |  |
| 13 |  |  |  |  |  | X |  |
| 14 |  |  |  |  |  | X |  |
| 15 |  |  |  |  | X |  |  |
| 16 |  |  |  |  |  |  | X |
| 17 |  |  |  |  |  | X |  |
| 18 |  |  |  |  |  |  | X |
| 19 |  | X |  |  |  |  |  |
| 20 |  |  |  |  | X |  |  |
| 21 |  |  |  | X | X |  |  |

## 6.2	Solution 1

### 6.2.1	Target key issue

This solution addresses key issue #1.

### 6.2.2	Solution description

The salient features of the solution are as follows.

a)	How the UE determines the country of its physical location and whether it is in the scope of 3GPP

The solution relies on various locating methods, which are specific to UE implementation.

The mapping (physical location to a country/international area) information provisioned in the UE is used to determine the country/international area of its physical location. How to provision this information to the UE is outside the scope of 3GPP in this solution.

b)	To what extent and under which conditions the UE could still rely on the MCC of the serving PLMN to determine the country of its physical location

A UE can rely on the MCC of the serving PLMN to determine the country of its physical location.

NOTE 1:	The UE can rely on the MCC of the serving PLMN, as network requirements are assumed such that a PLMN only keep registrations for UEs verified to be physically located in a location the network is allowed to serve and the broadcast MCC corresponds to this location, e.g. the MCC of the serving PLMN corresponds to the country the PLMN serves.

NOTE 2:	There can be temporary periods of time when the UE can be located in a different country than the country corresponding to the MCC of the serving PLMN, e.g. before the network or the UE can determine UE mobility to a different country, but the UE can assume the MCC to correctly reflect the country of its physical location when service is provided by the serving PLMN.

### 6.2.3	Impacts to existing procedures due to the solution

No impact.

## 6.3	Solution 2: Determining PLMNs that can be selected in the country of UE location based on USIM configuration

### 6.3.1	Target key issue

This is a candidate solution to Key Issue 2. It is relevant to the deployment scenario A and B (B1 through B3).

### 6.3.2	Solution Description

It is assumed that the UE knows in which country it is located. How this is achieved is a topic in Key Issue 1.

For each PLMN in the "HPLMN selector with Access Technology" and "User Controlled PLMN Selector with Access Technology" and "Operator Controlled PLMN Selector with Access Technology" files in USIM, there can be configured a list of countries in which the PLMN can be selected. This list is optional for each PLMN. If the list is not configured for a PLMN, the PLMN is considered as selectable only in the country of its MCC, unless the PLMN has a shared PLMN ID (MCC 9xx), in which case the PLMN is considered as selectable in all countries.

NOTE:	How the list of countries in which the PLMN can be selected is encoded in USIM is left to CT6 to discuss. The encoding should suppport all the relevant deployment scenarios.

Below is an example of the proposed USIM configuration. In this example, the new proposed list of countries is implemented as an additional column D in the existing Operator Controlled PLMN Selector with Access Technology file. How exactly it ends up being implemented would be up to CT6 to decide.

Table 6.3-1: HPLMN selector with Access Technology file in USIM according to the solution

| A. Priority | B. PLMN ID | C. RAT(s) supported | D. Allowed in following countries (MCC) |
| --- | --- | --- | --- |
| 1 | 310 070 (AT&T USA) | E-UTRA, NR, NR_GEO | Canada (302), Mexico (334) (NOTE 1) |
| NOTE 1: A PLMN is always allowed in its home country |  |  |  |

Table 6.3-2: Operator Controlled PLMN Selector with Access Technology file in USIM according to the solution

| A. Priority | B. PLMN ID | C. RAT(s) supported | D. Allowed in following countries (MCC) |
| --- | --- | --- | --- |
| 1 | 310 270 (Verizon USA) | E-UTRA, NR | Empty (NOTE 1) (NOTE 2) |
| 2 | 302 720 (AT&T Canada) | E-UTRA, NR, NR_GEO | USA (310) (NOTE 1) |
| 3 | 334 50 (AT&T Mexico) | E-UTRA, NR, NR_GEO | USA (310) (NOTE 1) |
| 4 | 901 44 (AT&T global) | NR_GEO | Empty (NOTE 3) |
| 5 | 901 88 (XYZ) | NR_GEO | China (460) (NOTE 1) |
| NOTE 1: A PLMN is always allowed in its home country NOTE 2: Column D may not be applicable to PLMNs that do not support NTN access NOTE 3: If column D is empty, a PLMN with a shared MCC is allowed in all countries |  |  |  |

### 6.3.3	Impacts

UE impacts:

-	New optional UE configuration in USIM and as part of SoR IEs indicating the countries in which each PLMN can be selected.

Network impacts:

-	New optional UE configuration indicating the countries in which each PLMN can be selected. Impact on SP-AF, SOR-AF, UDM and AMF. E.g. SOR-AF and UDM need the UE location information to determine the operator list of PLMN/Access Technologies.

NOTE:	Enhancements of Steering of Roaming functionality to include information on countries in which each PLMN can be selected and additional UE location information, need updates to Steering of Roaming stage 2 requirements. Such updates are to be assessed and agreed in the normative phase.

## 6.4	Solution 3

### 6.4.1	Target key issue

This solution addresses key issue #2.

### 6.4.2	Solution description

The salient features of the solution are as follows.

a)	How the UE determines which PLMN(s) can be selected in determined country of its physical location (if it is determined that the UE is not in an international area);

The mapping (a country to one or more MCCs) information provisioned in the UE is used to determine the PLMN(s) which can be selected from the determined country. How to provision this information to the UE is outside the scope of 3GPP in this solution;

b)	What information could be available to the UE to help in selecting, in a given country, a PLMN that meets the LI requirements of that country;

The mapping (a country to one or more MCCs) information provisioned in the UE. The mapping from an MCC to PLMNs is straightforward. Any LI requirements are under network and operator responsibility and implicitly met, but not explicitly known, by the UE by following information provisioned in the UE; and

c)	What modifications are needed, if any, to the PLMN selection procedure to take the LI requirement into account

Key differences compared to the current PLMN selection procedure are as follows.

1)	The UE prioritizes PLMNs with the MCC(s) corresponding to the country of its physical location (e.g. MCC(s) recommended by the network, where the MCC(s) recommended by the network shall be valid only during the PLMN selection followed by the reception of the MCC(s));

Editor's note:	The inclusion of the recommended MCC(s) is dependent on the SA2 agreements. The logic NW uses to derive this recommended MCC list is FFS.

Editor's note:	If there is no PLMN to which the UE is allowed to select in the current UE location, the UE behaviour is FFS.

NOTE:	By following the PLMN prioritization provisioned in the UE and the MCC(s) recommended by the network, the UE implicitly meets any LI requirements that may apply for the network.

2)	While a UE is in not only in a VPLMN but also in the (E)HPLMN, if the UE detects change in the country of its present location, the UE needs to attempt to operate as specified in bullets 1) and 2) in addition to periodically scanning a higher priority PLMN with the same MCC as the currently serving PLMN's MCC.

### 6.4.3	Impacts to existing procedures due to the solution

UE:

-	The UE prioritizes PLMNs with the MCC(s) corresponding to the country of its physical location (e.g. the MCC(s) recommended by the network), where the MCC(s) recommended by the network is valid only during the first PLMN selection followed by the reception of the MCC(s).

-	The "particular" state needs to be supported, e.g. the UE should indicate that the UE is in the "particular" state during the registration, a UE which could not select a PLMN whose MCC does not correspond to the country of its physical location or a PLMN whose MCC is included in the list of MCCs recommended by the network has to be in a "particular" state.

-	Receipt and handling of the list of MCCs recommended by the network has to be supported.

Network:

-	The network should be able to handle a UE in the "particular" state.

-	The network should be able to send the list of recommended MCCs taking into account whether the UE indicates that it is in the "particular" state.

## 6.5	Solution 4: PLMN selection inside international areas

### 6.5.1	Target key issue

This is a candidate solution to Key Issue 3. It is relevant to deployment scenarios C and D, i.e. in international areas.

### 6.5.2	Solution description

It is assumed that the UE knows when it is located inside international areas. How this is achieved is a subject of Key Issue 1.

PLMN selection in international areas should include the following adjustments w.r.t. the legacy PLMN selection procedure:

1.	A transition in and out of international areas should trigger a PLMN selection. The same procedure as for a PLMN selection upon switch-on should be followed. Additionally:

a)	the equivalent PLMN list is deleted; and

b)	the RPLMN is not considered.

NOTE:	As soon as the UE determines it is in an international area, the UE can give higher priority or lower priority to searching for PLMNs on bands which are used in NTN depoyments.

Editor's Note: It is FFS if such priortization is appropriate given that there can be scenarios where the UE is in international areas onboard a vessel which offers mobile services through onboard "basestation" operating as 3GPP access through 3GPP Terrestrial Radio Access.

2.	During periodic search for higher priority PLMN inside international areas, the UE does not eliminate PLMNs whose MCC is different from the MCC of the serving PLMN; and

3.	UE configuration should optionally indicate:

a)	whether or not a PLMN is selectable inside international areas; and

b)	PLMN’s priority inside international areas.

If no indication is provided for 3.a), the UE should assume that the PLMN is selectable inside international areas.

If no indication is provided for 3.b), the UE should assume that the PLMN’s priority is the same inside and outside international areas.

The UE configuration in item 3.a) above can be implemented using a new indication for each PLMN entry in the "User Controlled PLMN Selector with Access Technology" and "Operator Controlled PLMN Selector with Access Technology" files in USIM.

The UE configuration in item 3.a) and 3.b) above can be implemented using separate "User Controlled PLMN Selector with Access Technology" and "Operator Controlled PLMN Selector with Access Technology" files for international areas.

### 6.5.3	Impacts

UE impacts for UE supporting satellite radio access:

-	New PLMN selection trigger (transition in/out of international area)

-	Changes to the UE logic for eliminating candidate PLMNs during PLMN selection:

▪	the equivalent PLMN list is deleted;

▪	the RPLMN is considered not available; and

▪	PLMN with MCC is different from the MCC of the serving PLMN is not eliminated

-	New optional UE configuration in USIM indicating:

▪	whether or not a PLMN is selectable inside international areas; and

▪	PLMN’s priority inside international areas.

Network impacts:

-	provisionng and updating of new optional UE configuration in USIM.

-	new optional UE configuration: impact on SP-AF, SOR-AF, UDM and AMF.

NOTE:	Enhancements of Steering of Roaming functionality to include information on whether or not a PLMN is selectable inside international areas and PLMN’s priority inside international areas, need updates to Steering of Roaming stage 2 requirements. Such updates are to be assessed and agreed in the normative phase.

## 6.6	Solution 5

### 6.6.1	Target key issue

This solution addresses key issue #3.

### 6.6.2	Solution description

The salient features of the solution are as follows.

a)	How the UE selects PLMN if it is determined that the UE is in an international area

The UE selects a PLMN according to the legacy PLMN selection process. The LI requirement "Any solution addressing extraterritorial (e.g. international maritime zone and aeronautical) use cases should provide means to notify the HPLMN on roaming in and out of those areas" is not related to the PLMN selection process.

b)	What information could be available to aid in the PLMN selection in international areas

Existing lists, e.g. "HPLMN Selector with Access Technology", "User/Operator Controlled PLMN Selector with Access Technology", "Forbidden PLMNs", "Equivalent HPLMN".

### 6.6.3	Impacts to existing procedures due to the solution

No impact.

## 6.7	Solution 6: Handling of PLMNs with shared MCC

### 6.7.1	Target Key Issue

This is a solution to the following questions in Key Issue 4:

-	what modifications are needed to the PLMN selection procedure to handle PLMN IDs with shared MCC (e.g. 9xx);

-	should these changes also apply to the legacy PLMN selection procedure.

It is relevant to deployment scenarios B3 and D1 in which the serving PLMN uses shared MCC.

### 6.7.2	Solution description

When the UE is performing periodic search for higher priority PLMN, the UE should not eliminate any candidate PLMN due to the value of its MCC being different from the MCC of the serving PLMN if:

1)	The candidate PLMN has a PLMN ID with shared MCC; or

2)	The serving PLMN has a PLMN ID with shared MCC.

### 6.7.3	Impacts

Impact on the UE logic for eliminating candidate PLMNs during periodic search for higher priority PLMN when the UE is using satellite access.

## 6.8	Solution 7

### 6.8.1	Target key issue

This solution addresses key issue #4.

### 6.8.2	Solution description

The salient features of the solution are as follows.

a)	What modifications are needed to the PLMN selection procedure to handle shared/global PLMN IDs

No change is required in addition to modifications to the PLMN selection procedure to take the LI requirement into account (which is addressed in key issues #2) because in order for the UE to implicitly satisfy the LI requirement addressed in the key issue, the UE follows information provisioned in the UE and information provided by the network. If a UE cannot determine the country of its physical location (due to its physical location in an international area) or a PLMN belonging to the country of the UE's physical location is not available or allowable, the UE can select a PLMN according to the legacy algorithm. In this case, selection of a shared/global PLMN ID is not prohibited.

b)	Should these changes also apply to the legacy PLMN selection procedure

No.

c)	What information could be available to aid in handling shared/global PLMN IDs

Existing lists can be used. For example, "User Controlled PLMN Selector with Access Technology" can be used to (de)prioritize selecting a shared/global PLMN ID if a UE cannot determine the country of its physical location or a PLMN belonging to the country of the UE's physical location is not available.

d)	How to accommodate LI requirements when handling shared/global PLMN IDs

A shared/global PLMN ID can be selected if:

-	a UE cannot determine the country of its physical location (due to its physical location in an international area); or

-	a PLMN belonging to the country of the UE's physical location is not available or allowable (i.e. if the UE is in a situation where the LI requirement cannot be fulfilled).

Or even if:

-	the UE can determine the country of its physical location; and

-	a PLMN belonging to the country of the UE's physical location is available and allowable;

if one or more shared/global PLMN IDs are allowed to be selected from the country (which can be known from the mapping information discussed in key issue #2) according to information provisioned in the UE, a shared/global PLMN IDs can be selected.

### 6.8.3	Impacts to existing procedures due to the solution

No impact.

## 6.9	Solution 8: Range of values for timer T for UE in satellite access

### 6.9.1	Introduction

This solution is intended to resolve the range of value for timer T, described within KI#6(PLMN search for satellite access).

Presently, for the purpose of setting the value of Timer T (see 3GPP TS 23.122 [3], the UE is considered to be a non-NB-IoT UE or a NB-IoT UE. This differentiation came about from a request from RAN2 detailed in C1-161743 and subsequent CT1 agreed CR in C1-163109.

NOTE:	The stage 1 requirement for the different range of values for timer T was introduced in 22.011CR0235 (S1-161301).

As detailed in C1-161743 and C1-163109, the need for a longer range of values for timer T, is to meet the expectation that NB-IoT devices have a 10+years of battery life and is not about NB-IoT UEs operating under restricted or limited NB coverage.

However, a UE operating in satellite access mode face no such battery lifetime expectations. Indeed UEs in satellite access mode are not like NB-IoT devices but more like UEs in normal everyday use by the average human user who can charge their mobile phones from time to time.

In this solution, for the purpose of setting timer T, a UE in satellite access mode can be considered as a non-NB-IOT UE. Moreover, the same use of any setting for timer T in the USIM as specified in 3GPP TS 31.102 [9] and the MinimumPeriodicSearchTimer as specified in 3GPP TS 24.368 [10] can be reused as it is today.

### 6.9.2	Solution description

The principle of this solution is to consider that UE in satellite access mode, be considered for the purpose of setting the value of timer T, as a non-NB-IoT UE as described in 3GPP TS 23.122 [3], subclause 4.4.3.3.1.

Thus, for a UE that is accessing the 5GC through satellite access, T is either in the range 6 minutes to 8 hours in 6 minute steps or it indicates that no periodic attempts shall be made. If no value for T is stored in the SIM, a default value of 60 minutes is used for T.

If the UE is configured with the MinimumPeriodicSearchTimer as specified in 3GPP TS 24.368 [10] or 3GPP TS 31.102 [9], the UE shall not use a value for T that is less than the MinimumPeriodicSearchTimer. If the value stored in the SIM, or the default value for T (when no value is stored in the SIM), is less than the MinimumPeriodicSearchTimer, then T shall be set to the MinimumPeriodicSearchTimer.

### 6.9.3	Impacts

This solution has no impacts to what is already specified for the range and value of timer T.

## 6.10	Solution 9: Determination of the country of the UE’s location

### 6.10.1	Target key issue

This solution addresses key issue #1.

### 6.10.2	Solution description

There are two steps in the determination of the country of the UE location:

Step 1: determine the UE geolocation (lat, long)

Step 2: determine the country that corresponds to the UE location

#### 6.10.2.1	Determination of the UE geolocation

In the RAN WID on Solution for NR to support non-terrestrial access (NTN) in RP-201256, the following is stated (emphasis added):

The work item aims to specify the enhancements identified for NR NTN (non-terrestrial networks) especially LEO and GEO with implicit compatibility to support HAPS (high altitude platform station) and ATG (air to ground) scenarios according to the following principles:

- FDD is assumed for core specification work for NR-NTN.

- NOTE: This does not imply that TDD cannot be used for relevant scenarios e.g. HAPS, ATG

- Earth fixed Tracking area is assumed with Earth fixed and moving cells

- UEs with GNSS capabilities are assumed.

- Transparent payload is assumed

Based on the underlined text, a UE supporting satellite access is assumed to have GNSS capabilities. Furthermore, if the UE has satellite access coverage strong enough to camp on it, it is more likely than not to be able to determine its location using GNSS. The inverse logic also applies: if the UE cannot determine its location using GNSS signals then it is unlikely to successfully camp on a PLMN using satellite access. In the unlikely scenario where the UE camps on a PLMN using satellite access and is not able to determine its location using GNSS, the UE could attempt to determine its geolocation by other, implementation specific means.

#### 6.10.2.2	Determination of the country of the UE location

Once the UE geolocation is known, the UE determines the country of the UE location in an implementation-specific way. Since GNSS capabilities are assumed, it is not necessary to rely on the MCC of the serving PLMN to determine the country of UE's physical location.

### 6.10.3	Impacts to the existing procedures due to the solution

UE impacts:

-	none

Editor’s note:	UE impacts are FFS.

NW impacts:

-	none

## 6.11	Solution 10

### 6.11.1	Target key issue

This solution addresses key issue #1 and is based on solution 1.

### 6.11.2	Solution description

The salient features of the solution are as follows.

a)	How the UE determines the country of its physical location (if needed, see bullet b) and whether it is in the scope of 3GPP:

The solution relies on various locating methods, which can be legacy or specific to UE implementation.

If the UE supports TN access, the UE can rely on (legacy) procedures to determine a country using an MCC of a TN PLMN serving the same area:

i)	A UE can use the MCC of a TN PLMN for determining a country and for selecting an NTN PLMN of that country, if the TN PLMN serving the area where the UE is located is a PLMN:

1)	broadcasted by an TN cell; or

NOTE:	While cross-border TN-cell leakage can occur, an MCC broadcasted by the TN cell generally minimizes the possibility of registration rejection due to wrong country (see key issue #2).

2)	received as part of the procedures for determining whether the visited country mandates the selection of the N3IWF or ePDG in this country, when accessing the network via non-3GPP access.

When a TN PLMN serving the area where the UE is located has not been determined, solutions specific to UE implementation for determining the country of the UE’s physical location apply.

The mapping (physical location to a country/international area) information provisioned in the UE is used to determine the country/international area of its physical location. How to provision this information to the UE is outside the scope of 3GPP in this solution.

b)	to what extent and under which conditions the UE could still rely on the MCC of the serving PLMN to determine the country of its physical location.

Editor's note:	A solution for item b) is FFS.

### 6.11.3	Impacts to existing procedures due to the solution

No impact.

## 6.12	Solution 11: PLMN selection inside international areas

### 6.12.1	Target key issue

This is a candidate solution to Key Issue #1 to determine which country the UE is physically located.

The solution is also relevant to Key Issue #3 to determine when the UE enters to international area in deployment scenario C.

### 6.12.2	Solution description

The UE may indicate its capability for PLMN Selection Assistance (PSA) information. Based on UE’s support for PSA information, the HPLMN may provision the PSA information to the UE as part of the steering of roaming procedures. Alternatively, PSA may be configured in the ME or USIM.

Editor’s Note: The requirement for PSA capability indication needs to be provided by SA2.

Editor’s Note: Architectural impacts on determining the PSA, if any, need to be discussed in SA2.

The PSA information includes a polygon geofence to assist the UE in knowing the geographical boundaries for country A or its contiguous waters.

Editor’s Note: Whether there is a need to dynamically update PSA has to be studied.

The UE shall derive its current location based on UE implementation method outside of the scope of 3GPP. If the UE computes that its current location is inside the polygon geofence, the UE shall consider to be within a country A’s territorial area. If the UE computes that current location is outside the polygon geofence, UE shall consider to be within country A’s extraterritorial area.

In order to address the SA3-LI requirement mentioned in KI#3 description in subclause 5.3, if the UE detects that it transits between the extraterritorial area and the territorial area, the UE shall inform the serving network via dedicated NAS signalling, the serving network should then inform the HPLMN. In order to avoid frequent indications, geographical or timer-based hysteresis should be applied.

### 6.12.3	Impacts

UE supporting satellite NG-RAN access: indicate capability for PSA information, need to derive UE’s location periodically and apply the PSA information.

AMF: provision the PSA information to the UE.

UDM: include the PSA information in the steering of roaming information.

NOTE:	Enhancements of Steering of Roaming functionality to include PLMN Selection Assistance information, need updates to Steering of Roaming stage 2 requirements. Such updates are to be assessed and agreed in the normative phase.

## 6.13	Solution 12

### 6.13.1	Target key issue

This solution addresses key issue #5. It is applicable for the deployment scenario A/B/C/D.

### 6.13.2	Solution Description

According to 3GPP TS 22.011 [11] subclause 3.2.2.1:

"It shall be possible to have an associated Access Technology identifier e.g., NG-RAN, satellite NG-RAN, E-UTRAN (WB-S1 mode), E-UTRAN (NB-S1 mode), UTRAN, GERAN or GERAN EC-GSM-IoT associated with each entry in the PLMN Selector lists."

and in S1-204379 (the SA1’s reply to CT1’ LS), there was an agreement in SA1 on adding "satellite NG-RAN" to the set of possible associated access technology identifiers.

In addition, the agreed SA1 CR assumes that one identifier "satellite NG-RAN" is sufficient.

Therefore, the solution to the key issue #5 is as follows:

a) 	An access technology identifier "satellite NG-RAN" is used for PLMN selection.

b)	"satellite NG-RAN" is applicable as an access technology identifier in the "Operator Controlled PLMN Selector list", "HPLMN Selector with Access Technology" and "User Controlled PLMN Selector list", "satellite NG-RAN".

c)	"satellite NG-RAN" needs to be considered in the "SOR transparent container" for 5G steering of roaming scenario.

The existing prioritization mechanism for different RATs(i.e., the new satellite NG-RAN and old RATs e.g., NG-RAN, E-UTRAN, …etc) is a general design and can accommodate many new RATs; thus it can be extended (to include the new satellite NG-RAN) and reused. No new prioritization mechanism is needed, the PLMN+ACT selection shall follow the original "prioritization" mechanism as specified in TS 23.122 [3].

### 6.13.3	Impacts

UE impacts:

-	New Access Technology identifier "satellite NG-RAN" needs to be supported during PLMN selection.

- 	New Access Technology identifier "satellite NG-RAN" in "SOR transparent container" IE needs to be supported.

Network impacts:

-	New Access Technology identifier "satellite NG-RAN" in "SOR transparent container" IE needs to be supported.

NOTE:	Enhancements of Steering of Roaming functionality to include information New Access Technology identifier, need updates to Steering of Roaming stage 2 requirements. Such updates are to be assessed and agreed in the normative phase.

## 6.14	Solution 13

### 6.14.1	Target key issue

This solution addresses key issue #6.

### 6.14.2	Solution Description

The solution to the key issue #6 d) is as follows:

d)	Whether the search for higher priority PLMN should be limited to PLMNs where MCC matches the MCC or country of the RPLMN.

Based on the following consideration, the search for higher priority PLMN should not be limited to PLMNs where MCC matches the MCC or country of the RPLMN:

-	For users near or in the international area(e.g.sea , Scenario C/D), a global/shared PLMN ID (with MCC 9xx) is broadcast. If the search for higher priority PLMN is limited to PLMNs where MCC matches the MCC or country of the RPLMN, the PLMN with MCC 9XX will not be considered in borders.

-	For users in a scenario mentioned in TR 23.737 [5] KI#10 (Multi-national cell coverage area can span over multiple countries) and the Multi-national cell of satellite NG RAN broadcast multi-PLMN ID with different MCC, only searching for "same country as the current serving VPLMN" will keep the UE registered in the same VPLMN even though the UE is moving between countries A, B and C.

The search for higher priority PLMN should consider the MCCs allowed from the UE's location as a precondition. It is suggested the following mechanism:

1)	Enable a UE using satellite NG RAN to determine its location and support location-MCC mapping, following a solution to KI#1 and KI#2.

2)	During periodic search, the UE using satellite NG RAN checks it’s location and does the MCC mapping. The mapped MCC needs to be considered together with the priority/order list of PLMNs(e.g. HPLMN/EHPLMN/EPLMN).

Editor's note:	It is FFS whether the PLMNs on the priority/order list matching the mapped MCC have the higher priority or the PLMNs on the priority/order list not matching the mapped MCC are excluded.

3)	The UE using satellite NG RAN can also check it’s location periodically and record the mapped MCC change for periodic search.

Editor's note:	 It is FFS how the UE implements the periodic checking and whether the periodic checking works together with the periodic search.

Editor's note:	It is FFS how MCC(9xx) and the international areas case are considered for periodic search.

NOTE:	The MCC allowed from the UE's location for periodic search may also be provided by the network. How to provide MCC information from the network is not in the scope of this TR/study of PLMN selection.

### 6.14.3	Impacts

UE impacts:

-	Consider the MCCs allowed from the UE's location as a precondition during periodic search.

-	Consider the MCCs allowed from the UE's location as a precondition.

Network impacts: For PLMN Selection: none.

NOTE:	Network impact to support configuring and providing MCCs is expected and is not in the scope of this TR, but in the scope of 5GSAT_ARCH WI in SA2.

## 6.15		Solution 14

### 6.15.1	Target key issue

This solution addresses key issue #6.

### 6.15.2	Solution Description

The solution to the key issue #6 a) and b) is as follows:

a)	Whether there are different priority levels for available PLMNs in satellite access?

According to 3GPP TS 22.011 [11], SA1 only defines one identifier “satellite NG-RAN” for satellite access and assumes it is sufficient. RAT types for satellite access: NR(LEO), NR(MEO), NR(GEO) and NR(OTHERSAT) are not differentiated in Access technology for satellite access.

In addition, according to 3GPP TS 23.122 [3], the NAS only focuses on PLMN and PLMN/Access technology combination instead of RAT type during PLMN selection.

Therefore, it is no need to divide available PLMNs in satellite access into different groups by RAT types and consider different priority levels for each group.

b)	Whether the periodic search for higher priority PLMNs in satellite access is needed?

Based on the same consideration( i.e, satellite access is one of access technologies for 5GCN, whether to a satellite only PLMN or a PLMN containing satellite and terrestrial RAN), the periodic search for higher priority PLMNs in satellite access ranking by RAT types(e.g. NR(LEO), NR(MEO), NR(GEO) and NR(OTHERSAT)) is not needed. It is suggested periodic search for higher priority PLMNs follow the original "prioritization" mechanism as specified in TS 23.122 [3] (e.g. HPLMN/EHPLMN/EPLMN).

### 6.15.3	Impacts

UE impacts: None

Network impacts: None

## 6.16	Solution 15: Access technology selection based on PLMN Selector list with wildcarded PLMN ID

### 6.16.1	Target key issue

This is a candidate solution to Key Issue 5. It is applicable for the deployment scenario A/B/C/D.

### 6.16.2	Solution Description

It is assumed that access technology preference during PLMN Selection can be indicated using the PLMN Selector lists. Note that, as per existing requirements, a PLMN in a Selector list may have multiple occurrences with different access technology identifiers. This allows indication of preference of particular access technologies (e.g. Sat NG-RAN) over other access technologies.

Whilst it is possible to indicate a preference for e.g. "any network that provides NG-RAN" this way, this may lead to unduly long PLMN Selector lists. It is therefore proposed to allow introduction of ‘wildcarded’ PLMN IDs in the User Controlled PLMN Selector list and the Operator Controlled PLMN Selector list. The interpretation of an entry on a PLMN Selector list with a wildcarded PLMN ID shall be: "any PLMN ID that has any of the listed (R)AT technologies".

NOTE:	How a wildcarded PLMN ID is encoded in the USIM file can be discussed in normative phase. The simplest way, with least impact, would be to interpret a specific MCC+MNC (e.g. 999 00) combination as a wildcarded PLMN ID. Another way could be to encode a specific wildcard indication that can be added for an entry on the PLMN Selector list. This would have impact on the file structure of the PLMN Selector lists.

The tables below show some examples of the use of wildcarded PLMN IDs in Operator Controlled PLMN Selector lists. Note that the wildcarded PLMN IDs can also be used in conjunction with other than Satellite (R)AT indicators. This makes the solution future-proof for when additional (R)AT indicators are specified.

Table 6.16-1: Example PLMN Selector list: any satellite network (e.g. for mission critical)

| A. Priority | B. PLMN ID | C. RAT(s) supported |
| --- | --- | --- |
| 1 | Operator A | GERAN, UTRA, E-UTRA, NG-RAN, Sat NG-RAN |
| 2 | Operator B | GERAN, UTRA, E-UTRA, NG-RAN, Sat NG-RAN |
| 3 | * (wildcard) | Sat NG-RAN |
| 4 | Operator C | GERAN, UTRA, E-UTRA, NG-RAN |
| 5 | Operator D | GERAN, UTRA, E-UTRA, NG-RAN |

Table 6.16-2: Example PLMN Selector list: non satellite networks first (e.g. for IoT devices)

| A. Priority | B. PLMN ID | C. RAT(s) supported |
| --- | --- | --- |
| 1 | Operator A | GERAN, UTRA, E-UTRA, NG-RAN |
| 2 | Operator B | GERAN, UTRA, E-UTRA, NG-RAN |
| 3 | * (wildcard) | GERAN, UTRA, E-UTRA, NG-RAN |
| 4 | Operator C | Sat NG-RAN |
| 5 | * (wildcard) | Sat NG-RAN |

Table 6.16-3: Example PLMN Selector list: 5G capable networks only

| A. Priority | B. PLMN ID | C. RAT(s) supported |
| --- | --- | --- |
| 1 | Operator A | NG-RAN, Sat NG-RAN |
| 2 | Operator B | NG-RAN, Sat NG-RAN |
| 3 | Operator C | NG-RAN, Sat NG-RAN |
| 4 | Operator D | NG-RAN, Sat NG-RAN |
| 5 | * (wildcard) | NG-RAN, Sat NG-RAN |

Editor's Note: Encoding of wildcard is under CT6 responsibility and is FFS.

Editor's Note: The choice on which unique PLMN ID to use need to be referred to GSMA or to ITU or both and is thus FFS.

Editor's Note: confirmation from SA1 on the use of a wildcarded PLMN ID is FFS.

When processing an entry on the PLMN Selector list with a wildcarded PLMN ID, the UE shall search for any PLMN that provides service with any of the (R)AT associated with that entry, with in case of Sat NG-RAN (R)AT the conditions as defined by the present study. It shall not search using other (R)ATs, even if these may be available at the UE location.

Note that after PLMN selection and registration, the UE may move or may be moved by the network to other (R)ATs that the serving PLMN also provides; using normal procedures. This may include selecting a (R)AT that was not included on the PLMN Selector list.

### 6.16.3	Impacts

UE impacts:

-	Encoding for the wildcard PLMN ID needs to be specified, i.e. specify a PLMN ID value that is to be interpreted as wildcarded PLMN ID.

-	New behavior for interpreting a wildcard PLMN ID needs to be specified.

-	Existing UEs will not be impacted, in case the wildcard is represented by a unique PLMN ID value, as UEs that do not find that unique PLMN ID will continue with the next entry on the list.

Network impacts: none.

-	The HPLMN can configure and update the Operator Controlled PLMN Selector with Access Technology file in the USIM in the same way as currently done.

## 6.17	Solution 16: Restrictions in camping on an acceptable cell

### 6.17.1	Target key issue

This solution addresses key issue #7.

### 6.17.2	Solution description

When a UE is unable to obtain normal service from a PLMN and needs to attempt to camp on an acceptable cell, the UE can camp on an acceptable cell with RAT satellite NG-RAN only if the acceptable cell with RAT satellite NG-RAN belongs to a PLMN allowed in the country where the UE is physically located.

NOTE:	How a UE can determine whether a PLMN is allowed in the country where the UE is physically located is covered by Key Issues #1 and #2.

Editor's note:	The behaviour of a UE unable to obtain normal service when there is no PLMN allowed in the country where the UE is physically located, is FFS.

### 6.17.3	Impacts to existing procedures due to the solution

UE

When a UE camps on an acceptable cell, it checks whether the cell belongs to a PLMN allowed in the country where the UE is physically located, if the cell is a satellite NG-RAN cell.

## 6.18	Solution 17: PLMN search for satellite access

### 6.18.1	Target key issue

This is a candidate solution to Key Issue #6 and relevant for a UE supporting satellite NG-RAN access technology.

### 6.18.2	Solution description

The UE needs to be configured with a scaling factor either in the ME or USIM. Alternatively, the scaling factor may be provisioned by the HPLMN via steering of roaming procedures.

Editor’s Note: Architectural impacts, if any, need to be discussed in SA2.

When searching for higher priority PLMNs only available via satellite NG-RAN access, the UE should increase the periodic search timer T by the scaling factor.

Editor’s Note: The requirement for the periodic search timer T adjusted by the scaling factor depends on SA1's assessment and feedback whether changes to the background search timer are needed and if so, what the acceptable range is. The adjustment to the periodic search timer also needs to be verified with RAN2.

The UE shall continue to use existing periodic search timer T for higher priority PLMN search via terrestrial access, while the use of the scaled periodic search timer T is limited to the cases when only PLMNs with satellite NG-RAN access are of higher priority than the current RPLMN.

An example is provided to demonstrate the use of the scaling factor:

Table 6.18-1: Example 1 for Operator Controlled PLMN Selector with Access Technology file in USIM according to the solution

| Priority | PLMN ID | RAT(s) supported | Note |
| --- | --- | --- | --- |
| 1 | 310-410 | satellite NG-RAN | PLMN allowed via satellite NG-RAN access |
| 3 | 310-210 | NR | UE camped on this PLMN |
| 4 | 311-211 | NR | Low priority PLMN of the same country |

When the UE is camped on PLMN 310-210, and the high priority PLMN search is only for satellite NG-RAN access, the UE uses a periodic search timer = T (EFHPPLMN) * scaling factor.

Table 6.18-2: Example 2 for Operator Controlled PLMN Selector with Access Technology file in USIM according to the solution

| Priority | PLMN ID | RAT(s) supported | Note |
| --- | --- | --- | --- |
| 1 | 910-410 | satellite NG-RAN | Shared MCC, valid candidate for HPPLMN search as per solution #6 |
| 2 | 310-266 | E-UTRA, NR | Higher priority terrestrial network |
| 3 | 310-210 | NR | UE camped on this PLMN |
| 4 | 316-211 | NR | Low priority PLMN of the same country |

When the UE is camped on PLMN 310-210, and the higher priority PLMN(s) include both E-UTRA, NR as well as satellite NG-RAN access, then the periodic search timer remains T (EFHPPLMN).

### 6.18.3	Impacts

UE supporting satellite NG-RAN access: ability to store the scaling factor and support for configurable periodic search timer.

AMF: provision the scaling factor to the UE via steering of roaming procedures.

UDM: include the scaling factor in the steering of roaming procedures.

## 6.19	Solution 18

### 6.19.1	Target key issue

This solution addresses key issue #7.

### 6.19.2	Solution Description

This is a solution to the issue in Key Issue 7.

According to the clause 2 of the TS 23.122[3], if the MS is unable to find a suitable cell to camp on, or the SIM is not inserted, or if it receives certain responses to an LR request, it attempts to camp on a cell irrespective of the PLMN identity, and enters a "limited service" state in which it can only attempt to make emergency calls or to access RLOS.

On the other hand, for a UE using satellite access, according to TR 23.737[5] solution #13, which is selected as the way forward for key issue #10, the network should verify the UE location first during registration procedure over satellite access to route either the normal service or the emergency service. So, camping on a cell irrespective of the PLMN identity may cause a rejection to the registration request. In addition, receiving a cause other than the one telling the UE the selected PLMN is not allowed in the present UE location means the selected PLMN has already verified the UE’s location and the cell UE camps on is an acceptable cell.

Based on the above considerations, it is suggested to distinguish the situations listed in the clause 3.5 of the TS 23.122 [3] when UE attempting to camp on an acceptable cell to enter a "limited service" state in which it can make emergency calls:

-	For the UE using satellite access and unable to find a suitable cell to camp on, it may camp on an acceptable cell with the PLMN identity containing the MCC matching the country it locates;

-	For the UE using satellite access and the USIM is not inserted, while a new PLMN selection is not triggered, it may camp on an acceptable cell irrespective of the PLMN identity;

-	For the UE using satellite access and receiving a REGISTRATION REJECT during the registration procedure, it may camp on an acceptable cell of the selected PLMN or the one with the PLMN identity containing the MCC matching the country it locates.

NOTE:	Whether the UE using satellite access and receiving a REGISTRATION REJECT during the registration procedure may camp on an acceptable cell of the selected PLMN depends on the cause included in the REGISTRATION REJECT.

### 6.19.3		Impacts

UE impacts:

-	Distinguish the situations listed in the clause 3.5 of the TS 23.122[3] when UE attemping to camp on an acceptable cell to enter a "limited service" state in which it can make emergency calls.

Editor’s note:	The impacts on camping on acceptable cell and the definition of an acceptable cell need to be confirmed by RAN2.

Network impacts:

-	Need to verify the UE location first during registration and emergency registration procedure over satellite access.

-	Need to respond with REGISTRATION REJECT, including suitable cause value to tell the UE that the selected PLMN is not allowed in the present UE location, as it is requesting registration and  emergency registration in a country that is different from its location.

## 6.20	Solution 19: Use of network indication of MCC(s)

### 6.20.1	Target key issue

This is a candidate solution to Key Issue 2.

### 6.20.2	Solution Description

NOTE:	The following behavior of PLMN selection procedures rely on the assumption that NAS signaling procedures are updated such that the network can provide a list of possible MCCs that could be used by the UE during a subsequent PLMN selection, given the UE location. The agreement of such update is the SA2 area of responsibility.

Solution assumption also considers that the objective of the list of MCCs sent by the network is to inform the UE of the country the UE is currently located in, as determined by the network.

If the UE receives an optional list of MCCs for use during subsequent PLMN selection, the UE shall consider this list together with USIM files content as valid for subsequent selection of PLMNs providing satellite NG-RAN.

During this period, PLMN procedures as defined in 3GPP TS 23.122 [3] are applied, but excluding PLMNs which MCC is not present in list of MCCs, with following exception:

-	shared/global PLMN

Editor's note:	extra-territorial MCC use case is FFS, depending on C1-210437 and C1-210439 LS responses.

Editor's note:	capability for HPLMN to have some control on the use of the MCC list by the UE as described in the present solution is FFS.

Editor's note:	the MCC list provided by the network, its semantics, and the procedures by which it is provided is subject to confirmation by SA2.

At the end of the period, UE shall discard any list of possible MCCs received from the AMF.

Editor's note:	the complete definition of the period in which UE makes use of MCCs received from AMF and the duration of validity of the received list of MCCs are FFS.

### 6.20.3		Impacts

UE impacts:

-	Use of a new optional list of possible MCCs in PLMN selection, if available.

Network impacts:

For PLMN Selection:.

NOTE:	Network impact to determine an MCC list and provision this list to the UE is expected to impact the UE to network procedures and is not in the scope of this TR.

## 6.21	Solution 20: Handling or RAT prioritization

### 6.21.1	Target key issue

This solution addresses key issue #5.

### 6.21.2	Solution description

To address the need for more flexible prioritization of satellite access wr.t. terrestrial access, we propose to allow to configure the UE that supports both terrestrial and satellite access as:

-	TN-prefered: terrestrial access preferred over satellite access; or

-	NTN-prefered: satellite access preferred over terrestrial access.

To illustrate the impact of the configuration of the UE as TN-prefered vs. NTN-prefered, consider this example:

EFOPLMNwACT (or EFPLMNwACT) file in USIM lists PLMNs in the following order:

| Priority | PLMN | RATs supported |
| --- | --- | --- |
| 1 | A | NR |
| 2 | A | Satellite NG-RAN |
| 3 | B | NR, Satellite NG-RAN |
| 4 | C | Satellite NG-RAN |

TN-prefered roaming UE would derive the following prioritization:

| Priority | PLMN | RATs supported |
| --- | --- | --- |
| 1 | A | NR |
| 2 | B | NR |
| 3 | A | Satellite NG-RAN |
| 4 | B | Satellite NG-RAN |
| 5 | C | Satellite NG-RAN |

NTN-prefered roaming UE would derive the following prioritization:

| Priority | PLMN | RATs supported |
| --- | --- | --- |
| 1 | A | Satellite NG-RAN |
| 2 | B | Satellite NG-RAN |
| 3 | C | Satellite NG-RAN |
| 4 | A | NR |
| 5 | B | NR |

NOTE:	The tables above are just an illustration. No new lists need to be configured or provisioned. The only configuration is "TN-preferred" vs. "NTN-preferred."

TN-prefered and NTN-prefered prioritization would also apply to the HPLMN (i.e. EFHPLMNwAcT file) and to PLMNs not listed in USIM (i.e. "any PLMN").

The UE could be configured as TN-prefered or NTN-prefered by the user and/or by the network. Network configuration could be achieved using a new configuration parameter and provisioned in the UE using the following methods:

a)	pre-configured in the ME;

b)	pre-configured in the USIM; or

c)	sent to the UE by the network using the Steering of Roaming procedure.

User configuration would override the network configuration.

NOTE:	In the UE that supports both terrestrial and satellite access, it assumed that either access can be disabled by the user, such that a "TN-only" and "NTN-only" configuration is also possible. A UE configured in such way would follow the legacy PLMN prioritization.

### 6.21.3	Impacts to the existing procedures due to the solution

UE impacts:

-	support for new UE configuration as TN-prefered vs. NTN-prefered

-	during PLMN selection, the UE filters out PLMNs based on the configuration

NW impacts:

-	support for UE configuration as TN-prefered vs. NTN-prefered (optional)

## 6.22	Solution 21: Use of user and device settings for searching Satellite access or Terrestrial access PLMNs prior to PLMN selection

### 6.22.1	Target key issue

This solution addresses key issue #4 (Handling of shared/global PLMN IDs) and key issue #5 (Handling of new satellite RAT).

### 6.22.2	Solution description

This solution consider that mobile handsets of today are dominantly multi-mode devices and that device and user settings are available to indicate which radio technologies supported by those multi-mode devices are the preferred one. That device or user setting or both allows the multi-mode device to efficiently search for available PLMNs in an orderly manner according to preferred radio technology and thus assist in timely selection of a PLMN to provide service to the user.

For instance, out of the box, a UE could be set to "LTE only" or "LTE preferred/UMTS secondary" or "UMTS preferred/LTE secondary"or "UMTS preferred" or "5G preferred/LTE secondary" etc, etc. And when such settings are there in multi mode devices, if those devices need to make a search for PLMNs they will search using those preset or (user) adjusted preferences to first search on what is preferred and after that (when nothing of the preferred radio technology can be found) to search for the secondary technology.

NOTE 1:	The allowance of user setting in today's mobile hansets to indicate the preference of radio access technology already exist without any need to specifically document these setting in CT1's Technical Specifications.

This solution envisage that for a multi-mode handset with satellite radio capabilities, such preset or user adjusted settings will exist. So a satellite capable multi-mode handset will have means to set a preference for terrestrial access or satellite access. In this solution, use of such presets of "Terrestrial AccessN only" or "Terrestrial Access preferred/Satellite secondary" or "Satellite only" or "Satellite preferred/Terrestrial Access secondary" will guide the UE's search for available PLMNs in the preferred radio technology. And if a UE is set to "Satellite only" or "Satellite preferred", then satellite radio/frequency bands will first be search before other modes.

NOTE 2:	The actual semantics of the MMI display denoting "satellite" or "terrestrial is a UE implementation issue. It is borne in mind that any such MMI display need to consider that the average user are likely to not know the technical distinction between GEO, MEO or LEO configurations of satellites.

Thus it is not a matter of a search for global/shared PLMN IDs but a search for what available PLMNs one can find under the radio technology chosen or set within that UE. However, having this device or user setting, overcomes the concern that the UE can mistakenly search for terrestrial access PLMNs that have global/shared PLMN IDs.

With this solution, once the available PLMNs under a preferred technology (or required technology) are found then it is a matter of prioritising those available PLMNs using the existing USIM and ME files such as EFPLMNwAcT (User controlled PLMN selector with Access Technology), EFOPLMNwACT (Operator controlled PLMN selector with Access Technology), EFHPLMNwAcT (HPLMN selector with Access Technology).
In this way of working - i.e. first use the device settings of radio technology to perform a search for available PLMNs, then utilise the USIM files to establish the priority of the available PLMNs – there are no impacts to the currently specified PLMN selection procedures other than having to introduce the extra NR radio access technology identifier which in any case has to be done in accordance to SA1's indication in incoming LS C1-210041.

Additionally, by separately looking for available PLMNs in dedicated radio access technology, the concern that terrestrial access PLMNs with share/global PLMN IDs might be mixed into the search and selection of satellite access PLMNs with shared/global PLMN IDs is avoided.

### 6.22.3	Impacts

UE impacts:

-	Introduction within UE device settings – as implementation options - the indications of satellite access or terrestrial access as a radio technology preference.

-	Introduction of the new Access Technology Identifier to the required EF files (under the responsibilities of CT6). This introduction is already necessary under KI#5 and by SA1's LS in C1-210041.

Network impacts:	None

NOTE 3:	No specification impacts expected.

# 7	Evaluation

## 7.1	Evaluation of Solutions for Key Issue #1

Table 7.1-1

| Solution <x> |  |
| --- | --- |
| Solution <y> |  |

## 7.2	Evaluation of Solutions for Key Issue #2

Table 7.2-1

| Solution <x> |  |
| --- | --- |
| Solution <y> |  |

## 7.3	Evaluation of Solutions for Key Issue #3

Table 7.3-1

| Solution <x> |  |
| --- | --- |
| Solution <y> |  |

## 7.4	Evaluation of Solutions for Key Issue #4

Table 7.4-1

| Solution <x> |  |
| --- | --- |
| Solution <y> |  |

## 7.5	Evaluation of Solutions for Key Issue #5

Table 7.5-1

| Solution <x> |  |
| --- | --- |
| Solution <y> |  |

## 7.6	Evaluation of Solutions for Key Issue #6

Table 7.6-1

| Solution <x> |  |
| --- | --- |
| Solution <y> |  |

## 7.7	Evaluation of Solutions for Key Issue #7

Table 7.7-1

| Solution <x> |  |
| --- | --- |
| Solution <y> |  |

# 8	Conclusion

Annex A (informative):
Change history

| Date | Meeting | TDoc | Subject/Comment | New version |
| --- | --- | --- | --- | --- |
| Oct 2020 | 126e | C1-206686, C1-206684, C1-206688, C1-206689, C1-206690, C1-206991, C1-206693, C1-206694 |  | 0.1.0 |
| Nov 2020 | 127e | C1-207098, C1-207170, C1-207390, C1-207672, C1-207673, C1-207674, C1-207675, C1-207597, C1-207599, C1-027745, C1-207748, C1-207746, C1-207762 |  | 0.2.0 |
| Jan 2021 | 127bis-e | C1-210435, C1-210369, C1-210422 , C1-210388, C1-210420, C1-210320, C1-210204 , C1-210421, C1-210356, C1-210397, C1-210398 , C1-210399, C1-210317, C1-210408, C1-210436 |  | 0.3.0 |
| Feb 2021 | 128e | C1-210687, C1-210771, C1-210938, C1-211160, C1-211179, C1-211212, C1-211213, C1-211308, C1-211378, C1-211392, C1-211377, C1-211145, C1-211507, C1-211508, C1-211509 |  | 0.4.0 |