3GPP TSG-CT WG1 Meeting #128-e	C1-21xxxx

- Electronic meeting; 25 February – 5 March 2021								(was C1-210725)

Source:	Qualcomm Incorporated

Title:	Update of Solution #28 to Key Issue #6

Spec:	TR 24.811 v0.2.0

Agenda item:	17.2.9

Document for:	Agreement

1. Reason for Change

Solution #28 currently contains several Editor’s notes. It is proposed to resolve the Editor’s notes which do not have any dependencies on SA3 or RAN WGs’ feedback as follows:

| Editor’s note | Proposed resolution |
| --- | --- |
| It is FFS how the AMF of PLMN A determines the previously selected PLMN of the Disaster Inbound Roamers. | The AMF of PLMN A determines the previously selected PLMN of the Disaster Inbound roamers when the Disaster Inbound Roamers register on PLMN A, as specified in the solution(s) to Key Issue #4 (Registration to the roaming PLMN without Disaster Condition in case of Disaster Condition). |
| How often the UE shall check for this broadcast indication is FFS. | There is no need for the UE to periodically check for a change in the broadcast indication, the existing RAN mechanism to notify the UE of System Information change as specified in TS 38.304 and TS 38.331 applies. |

2. Proposal

It is proposed to agree the following changes to 3GPP TR 24.811 v0.2.0.

* * * First Change * * * *

### 6.28.2	Detailed description

Upon being notified that a Disaster Condition in PLMN D no longer applies, a PLMN without Disaster Condition (PLMN A) currently serving Disaster Inbound Roamers may perform one or more of the following:

a)	over the 3GPP access, turn off the broadcast indication (e.g. in SIB) that a Disaster Condition in PLMN D applies;

Editor's note: Extension of broadcast signalling is subject to agreement of RAN WGs.

Editor's note: Dynamic update of broadcast signalling is subject to agreement of RAN WGs.

Editor's note:	Input from SA3 is needed regarding potential security risks resulting from using broadcast signalling to convey information related to disaster roaming.

b)	for the Disaster Inbound Roamers in 5GMM-CONNECTED mode, which previously selected PLMN D:

1)	perform a generic UE configuration update procedure with an indication that the Disaster Condition in another PLMN no longer applies; or

2)	perform a generic UE configuration update procedure with "re-registration requested", then reject the UE’s registration request with 5GMM cause #ZZZ "disaster condition in other PLMN no longer applies"; or

If a Disaster Inbound Roamer has an emergency PDU session or a high priority service, the AMF of PLMN A performs the handling above after release of the emergency PDU session or after the high priority service is finished.

c)	for the Disaster Inbound Roamers in 5GMM-IDLE mode which previously selected PLMN D and which attempt to transition to 5GMM-CONNECTED mode by initiating a registration or service request procedure, reject the UE’s request with 5GMM cause #ZZZ "disaster condition in other PLMN no longer applies".

The AMF of PLMN A determines the previously selected PLMN of the Disaster Inbound roamers when the Disaster Inbound Roamers register on PLMN A, as specified in the solution(s) to Key Issue #4 (Registration to the roaming PLMN without Disaster Condition in case of Disaster Condition).

Upon:

a)	being notified by the lower layers that the indication (e.g. SIB flag) broadcast by PLMN A that a Disaster Condition applies in PLMN D has been turned off;

c)	receiving a CONFIGURATION UPDATE COMMAND message with an indication that the Disaster Condition in another PLMN no longer applies; or

d)	receiving a REGISTRATION REJECT or SERVICE REJECT message with 5GMM cause #ZZZ "disaster condition in other PLMN no longer applies";

the UE shall remove the stored indication that a Disaster Condition applies to PLMN D, enter 5GMM-REGISTERED.PLMN-SEARCH and perform PLMN selection as specified in 3GPP TS 23.122 [7] subclause 4.4.3.1.1.

* * * End of Changes * * * *