3GPP TSG-CT WG1 Meeting #133e-bis	C1-220326

- E-meeting, 17-21 January 2022					  						was C1-217286, C1-216882,

- C1-216210, C1-215961

Source:	Samsung, AT&T

Title:	Eees_EASDiscovery_Notify operation for Eees_EASDiscovery API

Spec:	3GPP TS 24.558 v1.0.0

Agenda item:	17.2.16

Document for:	Agreement

1. Introduction

This pCR specifies Eees_EASDiscovery_Notify service operation.

2. Reason for Change

The Eees_EASDiscovery_Notify operation is defined in clause 8.5.4.4 of 3GPP TS 23.558. It is required to provide service operations details in stage#3.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.558 v1.0.0.

* * * First Change * * * *

#### 5.3.2.4	Eees_EASDiscovery_Notify

##### 5.3.2.4.1	General

This service operation is used by the EES to notify the EEC about the EAS discovery information.

##### 5.3.2.4.2	EES notifying the EAS discovery information to EEC using Eees_EASDiscovery_Notify operation

The EES determines to notify the EEC with the EAS discovery information, when an event occurs at the EES that satisfies trigger conditions for updating EAS discovery information of a subscribed EEC. The EES may obtain the UE's location as specified in clause 5.3 of 3GPP TS 29.122 [3].

If EAS discovery filters are provided by the EEC, the EES identifies the EAS(s) based on the provided EAS discovery filters and the UE location.

Editor’s note: Considering ACR scenarios supported by the AC and the EEC while identifying the EAS(s) is FFS.

If EAS discovery filters are not provided, the EES identifies the EAS(s), if available, based on the UE-specific service information at the EES and the UE location; and by applying the ECSP policy (e.g. based only on the UE location).

NOTE:	If valid UE location information is not available in local cache, then the EES obtains the UE location information by consuming the 3GPP core network capabilities.

If EES determines that the UE location is outside the Geographical or Topological Service Area of an EAS, then the EES shall not include this EAS in the EAS discovery notification.

If the EES identifies the EAS(s) then to notify the EAS discovery information events, the EES shall either send an HTTP POST message using the Notification Destination URI received in the subscription request, as specified in clause 6.3.4.2 or push the notifications using the WebSocket channel which is already set up between the EES and the EEC based on its subscription creation request (see websockNotifConfig IE within EasDiscoverySubscription in clause 6.3.5.2.4).

The EEC may cache the EAS information (e.g. EAS endpoint) for subsequent use. If the "lifeTime" attribute is included in the notification, the EEC may cache the EAS information only for the duration specified by the Lifetime IE.

* * * End of Change * * * *