3GPP TSG-CT WG1 Meeting #133e-bis	C1-220328

- E-meeting, 17-21 January 2022					  						was C1-217288, C1-216884,

- C1-216213, C1-215963

Source:	Samsung, AT&T

Title:	Eees_EASDiscovery_Unsubscribe operation for Eees_EASDiscovery API

Spec:	3GPP TS 24.558 v1.0.0

Agenda item:	17.2.16

Document for:	Agreement

1. Introduction

This pCR specifies Eees_EASDiscovery_Unsubscribe service operation.

2. Reason for Change

The Eees_EASDiscovery_Unsubscribe operation is defined in clause 8.5.4.6 of 3GPP TS 23.558. It is required to provide service operations details in stage#3.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.558 v1.0.0.

* * * First Change * * * *

#### 5.3.2.6	Eees_EASDiscovery_Unsubscribe

##### 5.3.2.6.1	General

This service operation is used by the EEC to cancel an existing subscription for EAS discovery information.

##### 5.3.2.6.2	EEC unsubscribing to EAS discovery subscription from EES using Eees_EASDiscovery_Unsubscribe operation

To cancel an existing subscription for EAS discovery information from the EES, the EEC shall send an HTTP DELETE message to the EES, on the resource URI identifying the "Individual EAS Discovery Subscription" resource as specified in clause 6.3.2.3.3.2. Upon receiving the HTTP DELETE request, the EES:

a)	shall verify and check if the EEC is authorized to delete the Individual EAS Discovery Subscription resource;

b)	if the EEC is authorized to perform the operation, then the EES shall delete the individual EAS Discovery subscription resource identified by the "subscriptionId" provided within the request URI;

c)	on success, shall return the "204 No Content" message to the EEC, indicating the successful removal of the subscription resource.

d)	if the operation fails, shall return an appropriate HTTP error response as specified in Table 5.2.6-1 of 3GPP TS 29.122 [3].

* * * End of Change * * * *