3GPP TSG-CT WG1 Meeting #133bis-e	C1-220429

- E-meeting, 17 - 21 January 2022

Source:	CATT

Title:	Additional parameters announcement procedure

Spec:	3GPP TS 24.554 v1.0.0

Agenda item:	17.2.18

Document for:	Agreement

1. Introduction

According to S2-2109160 CR0019 to TS 23.304, Cell ID announcement procedure is replaced by Additional parameters announcement procedure, to accommodate more parameters (i.e. TAI in addition to Cell ID) for announcement. This needs to be reflected in Stage 3.

2. Reason for Change

Update TS 24.554 to align with the latest progress in SA2.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.554 v1.0.0.

* * * First Change * * * *

###### 8.2.1.2.4.2	Announcing procedure for relay discovery additional information

The 5G ProSe UE-to-network relay UE announces the relay discovery additional information:

a)	if the 5G ProSe remote UE requests the 5G ProSe UE-to-network relay UE to announce the NG-RAN Cell Global ID (NCGI) or TAI of the cell serving the 5G ProSe UE-to-network relay UE, and as a response the 5G ProSe UE-to-network relay UE acknowledges with the ProSe additional parameters announcement response message, then the 5G ProSe UE-to-network relay UE includes the NCGI or TAI of the serving cell in the PROSE PC5 DISCOVERY message for relay discovery additional information until the timer T5107 expires (see the subclause 8.2.8).

NOTE 1:	5G ProSe UE-to-network relay UE announces the relay discovery additional information only when it is in NG-RAN coverage.

Figure 8.2.1.2.4.2.1 illustrates the interaction of the 5G ProSe UE-to-network relay UE and the 5G ProSe remote UE in the announcing UE procedure for relay discovery additional information.

Figure 8.2.1.2.4.2.1: Announcing procedure for relay discovery additional information

The 5G ProSe UE-to-network relay UE may start announcing relay discovery additional information if:

a)	the 5G ProSe UE-to-network relay UE is currently authorised to perform 5G ProSe direct discovery Model A announcing in the serving PLMN if the UE is served by NG-RAN; and

1)	additional parameters announcement for the serving cell of the 5G ProSe UE-to-network relay UE has been requested and responded to 5G ProSe remote UEs, the timer T5107 has not expired (periodic reporting);

2)	additional parameters announcement for the serving cell of the 5G ProSe UE-to-network relay UE has been requested and responded to 5G ProSe remote UEs, the timer T5107 has not expired, and the 5G ProSe UE-to-network relay UE detects camping on a new serving cell; or

3)	additional parameters announcement for the serving cell of the 5G ProSe UE-to-network relay UE has been requested and responded to 5G ProSe remote UEs, the timer T5107 has not expired, and the 5G ProSe UE-to-network relay UE detects entering a new tracking area.

When the 5G ProSe UE-to-network relay UE has some additional information to broadcast (i.e NCGI, due to the periodic reporting or due to camping on a new serving cell), then the 5G ProSe UE-to-network relay UE:

a)	shall request the parameters from the lower layers for ProSe direct discovery announcing (see 3GPP TS 38.331 [13]). If the 5G ProSe UE-to-network relay UE in 5GMM-IDLE mode needs to request resources for sending PROSE PC5 DISCOVERY messages as specified in 3GPP TS 38.331 [13], the 5G ProSe UE-to-network relay UE shall perform a service request procedure or mobility registration procedure as specified in 3GPP TS 24.501 [11];

b)	shall obtain a valid UTC time for the discovery transmission from the lower layers and generate the UTC-based counter corresponding to this UTC time as specified in clause 11.2.5;

c)	shall generate PROSE PC5 DISCOVERY message(s) for relay discovery additional information according to clause 10.2.1. In the PROSE PC5 DISCOVERY message for relay discovery additional information, the 5G ProSe UE-to-network relay UE shall:

1)	include the relay service code used for 5G ProSe direct communication which the 5G ProSe remote UE used to request for the relay discovery additional information;

2)	set the announcer info parameter to the User info ID parameter, configured in clause 5.2.5;

3)	set the relay discovery additional information contents by the additional information to broadcast;

4)	shall set the UTC-based counter LSB parameter to include the eight least significant bits of the UTC-based counter; and

5)	shall set the ProSe direct discovery PC5 message type parameter as specified in table 10.2.1.11;

d)	shall apply the DUIK, DUSK, or DUCK with the associated Encrypted Bitmask, along with the UTC-based counter to the PROSE PC5 DISCOVERY message for whichever security mechanism(s) configured to be applied, e.g. integrity protection, message scrambling or confidentiality protection of one or more above parameters, as specified in 3GPP TS 33.XXX [TBD];

Editor’s note:	Details of security related content in d) are FFS and will be determinated by SA WG3.

e)	shall set the default destination layer-2 ID as specified in clause 5.2.5 to the destination layer-2 ID, and self-assign a source layer-2 ID for sending the UE-to-network relay discovery announcement; and

f)	shall pass the resulting PROSE PC5 DISCOVERY message for relay discovery additional information along with the source layer-2 ID, destination layer-2 ID, and an indication that the message is for 5G ProSe direct discovery to the lower layers for transmission over the PC5 interface.

The 5G ProSe UE-to-network relay UE shall ensure that it keeps on passing the PROSE PC5 DISCOVERY messages periodically to the lower layers for transmission until the corresponding timer (i.e. timer T5107 when the additional information is NCGI or TAI) expires.

NOTE 2:	The periodicity of sending the PROSE PC5 DISCOVERY messages for relay discovery additional information by the 5G ProSe UE-to-network relay UE is implementation specific and is normally lower than the one related to the additional parameters announcement request refresh timer T5016.

During the announcing operation, if one of the above conditions is no longer met, the 5G ProSe UE-to-network relay UE may instruct the lower layers to stop announcing.

* * * Next Change * * * *

###### 8.2.1.2.5.2	Monitoring procedure for relay discovery additional information

The 5G ProSe remote UE monitors relay discovery additional information:

a)	until the additional parameters announcement request refresh timer T5016 expires if the 5G ProSe remote UE has requested the 5G ProSe UE-to-network relay UE to announce the NCGI or TAI of the cell serving the 5G ProSe UE-to-network relay UE and received the ProSe additional parameters announcement response message from the 5G ProSe UE-to-network relay UE.

The UE may instruct the lower layers to start monitoring if:

a)	a request from upper layers to monitor for relay discovery additional information is still in place and either:

1)	the UE is currently authorised to perform 5G ProSe direct discovery Model A monitoring in at least one PLMN if the UE is served by NG-RAN;

2)	the UE is currently authorised to perform 5G ProSe direct discovery Model A monitoring if the UE is not served by NG-RAN; or

3)	the UE is:

i)	in 5GMM-IDLE mode, in limited service state as specified in 3GPP TS 23.122 [14], and the reason for the UE being in limited service state is one of the following:

A)	the UE is unable to find a suitable cell in the selected PLMN as specified in 3GPP TS 36.304 [15];

B)	the UE received a REGISTRATION REJECT message or a SERVICE REJECT message with the 5GMM cause #11 "PLMN not allowed" as specified in 3GPP TS 24.501 [11]; or

C)	the UE received a REGISTRATION REJECT message or a SERVICE REJECT message with the 5GMM cause #7 "5GS services not allowed " as specified in 3GPP TS 24.501 [11]; and

ii)	authorised to perform 5G ProSe direct discovery Model A monitoring when the UE is not served by NG-RAN and configured with the radio parameters to be used for 5G ProSe direct discovery when not served by NG-RAN.

If the UE is in 5GMM-CONNECTED mode, the monitoring UE shall also trigger the corresponding procedure in lower layers as specified in 3GPP TS 38.331 [13].

During the monitoring operation, if one of the above conditions is no longer met, the UE may instruct the lower layers to stop monitoring. When the UE stops monitoring, if the UE is in 5GMM-CONNECTED mode, the UE shall trigger the corresponding procedure in lower layers as specified in 3GPP TS 38.331 [13].

Upon reception of a PROSE PC5 DISCOVERY message for relay discovery additional information according to clause 10.2.1, for the target relay service code to be monitored, the UE shall use the associated DUSK, if configured, and the UTC-based counter obtained during the monitoring operation to unscramble the PROSE PC5 DISCOVERY message as described in 3GPP TS 33.XXX [TBD]. Then, if a DUCK is configured, the UE shall use the DUCK and the UTC-based counter to decrypt the configured message-specific confidentiality protected portion, as described in 3GPP TS 33.XXX [TBD]. Finally, if a DUIK is configured, the UE shall use the DUIK and UTC-based counter to verify the MIC field in the unscrambled PROSE PC5 DISCOVERY message for relay discovery additional information.

NOTE 1:	The use of an erroneous UTC-based counter for processing received PROSE PC5 DISCOVERY messages at the ProSe-enabled UE can cause MIC check failure after DUIK is used for integrity check, and malformed contents after DUSK is used for unscrambling or DUCK is used for deciphering. How a ProSe-enabled UE ensures the accuracy of the UTC-based counter is left to UE implementation.

NOTE 2:	The UE can determine the received PROSE PC5 DISCOVERY message for relay discovery additional information is for 5G ProSe direct discovery based on an indication from the lower layer.

Editor’s note:	Details of security related content in d) are FFS and will be determinated by SA WG3.

Then, if:

a)	the relay service code parameter of the PROSE PC5 DISCOVERY message for relay discovery additional information is the same as the relay service code parameter configured as specified in clause 5.2.5 for the connectivity service being monitored; and

b)	the User info ID of the UE-to-network relay UE is not configured as specified in clause 5 for the connectivity service being monitored, or the announcer info parameter of the PROSE PC5 DISCOVERY message for UE-to-network relay discovery announcement is the same as the User info ID of the UE-to-network relay UE configured as specified in clause 5.2.5 for the connectivity service being monitored,

then the UE shall consider that the relay discovery additional information it intends to monitor has been discovered. In addition, the UE can measure the signal strength of the PROSE PC5 DISCOVERY message for relay discovery additional information for relay selection or reselection.

* * * Next Change * * * *

### 8.2.8	5G ProSe additional parameters announcement procedure

#### 8.2.8.1	General

The purpose of the 5G ProSe additional parameters announcement procedure is for the 5G ProSe layer-3 remote UE to obtain NCGI or TAI of the cell serving the 5G ProSe layer-3 UE-to-network relay UE.

The remote UE in this procedure shall be a 5G ProSe-enabled UE and is authorised to act as a 5G ProSe layer-3 remote UE towards a 5G ProSe layer-3 UE-to-network relay UE based on the service authorisation procedure as specified in clause 5. The 5G ProSe layer-3 UE-to-network relay UE in this procedure shall be a 5G ProSe-enabled UE and is authorised to act as a 5G layer-3 ProSe UE-to-network relay UE based on the service authorisation procedure as specified in clause 5.

#### 8.2.8.2	5G ProSe additional parameters announcement procedure initiation by the 5G ProSe layer-3 remote UE

Before initiating the 5G ProSe additional parameters announcement procedure, a direct link has been successfully established between the 5G ProSe layer-3 remote UE and the 5G ProSe layer-3 UE-to-network relay UE.

The 5G ProSe layer-3 remote UE shall initiate a 5G ProSe additional parameters announcement procedure:

a)	when the 5G ProSe layer-3 remote UE is triggered by an upper layer application to report NCGI or TAI of the serving cell to the application server, but cannot receive the PROSE PC5 DISCOVERY message for relay discovery additional information from the 5G ProSe layer-3 UE-to-network relay UE, or the NCGI or TAI is not included in the PROSE PC5 DISCOVERY message for relay discovery additional information from the 5G ProSe layer-3 UE-to-network relay UE; or

b)	when the additional parameters announcement request refresh timer T5106 expires and the 5G ProSe layer-3 remote UE still needs to obtain NCGI or TAI of the cell serving the 5G ProSe layer-3 UE-to-network relay.

The 5G ProSe layer-3 remote UE shall generate a PROSE ADDITIONAL PARAMETERS ANNOUNCEMENT REQUEST message and pass this message to the lower layers for transmission along with the 5G ProSe layer-3 remote UE's layer-2 ID (for unicast communication) and the 5G ProSe layer-3 UE-to-network relay UE's layer-2 ID (for unicast communication).

Figure 8.2.8.2.1: 5G ProSe additional parameters announcement procedure

#### 8.2.8.3	5G ProSe additional parameters announcement procedure accepted by the 5G ProSe layer-3 UE-to-network relay UE

Upon receiving a PROSE ADDITIONAL PARAMETERS ANNOUNCEMENT REQUEST message, the 5G ProSe layer-3 UE-to-network relay UE shall allocate an additional parameters announcement request refresh timer T5106 to the remote UE, and start a timer T5107. The timer T5107 shall be longer than the additional parameters announcement request refresh timer T5106.

Then the 5G ProSe layer-3 UE-to-network relay UE shall respond a PROSE ADDITIONAL PARAMETERS ANNOUNCEMENT RESPONSE message with an additional parameters announcement request refresh timer T5106 IE set to the T5106 timer value assigned by the 5G ProSe layer-3 UE-to-network relay UE. The 5G ProSe layer-3 UE-to-network relay UE shall start announcing the NCGI or TAI of the serving cell by triggering the announcing UE procedure for relay discovery additional information as described in clause 8.2.1.2.4.

#### 8.2.8.4	5G ProSe additional parameters announcement procedure completion by the 5G ProSe layer-3 remote UE

Upon receiving a PROSE ADDITIONAL PARAMETERS ANNOUNCEMENT RESPONSE message, the UE shall start the additional parameters announcement request refresh timer T5106 with the received value.

#### 8.2.8.5	Abnormal cases

##### 8.2.8.5.1	Abnormal cases in the 5G ProSe layer-3 remote UE

If there is no response from the 5G ProSe layer-3 UE-to-network relay UE after the PROSE ADDITIONAL PARAMETERS ANNOUNCEMENT REQUEST message has been successfully delivered, the 5G ProSe layer-3 remote UE shall retransmit the PROSE ADDITIONAL PARAMETERS ANNOUNCEMENT REQUEST message.

NOTE:	The timer to trigger retransmission and the maximum number of allowed retransmissions are UE implementation specific.

* * * Next Change * * * *

### 10.3.24	ProSe additional parameters announcement request

#### 10.3.24.1	Message definition

This message is sent by the 5G ProSe layer-3 remote UE to 5G ProSe layer-3 UE-to-network relay UE to initiate the ProSe additional parameters announcement request procedure. See table 10.3.24.1.1.

Message type:	PROSE ADDITIONAL PARAMETERS ANNOUNCEMENT REQUEST

Significance:	dual

Direction:		UE to peer UE

Table 10.3.24.1.1: PROSE ADDITIONAL PARAMETERS ANNOUNCEMENT REQUEST content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE ADDITIONAL PARAMETERS ANNOUNCEMENT REQUEST identity | ProSe PC5 signalling message type 11.3.1 | M | V | 1 |
|  | Sequence Number | Sequence Number 11.3.2 | M | V | 1 |

### 10.3.25	ProSe additional parameters announcement response

#### 10.3.25.1	Message definition

This message is sent by the 5G ProSe layer-3 UE-to-network relay UE to the 5G ProSe layer-3 remote UE to acknowledge and respond to the ProSe additional parameters announcement request. See table 10.3.25.1.1.

Message type:	PROSE ADDITIONAL PARAMETERS ANNOUNCEMENT RESPONSE

Significance:	dual

Direction:		UE to peer UE

Table 10.3.25.1.1: PROSE ADDITIONAL PARAMETERS ANNOUNCEMENT RESPONSE content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE ADDITIONAL PARAMETERS ANNOUNCEMENT RESPONSE identity | ProSe PC5 signalling message type 11.3.1 | M | V | 1 |
|  | Sequence Number | Sequence number 11.3.2 | M | V | 1 |
|  | Additional parameters announcement request refresh timer T5106 | Binary 11.3.28 | M | V | 2 |

* * * Next Change * * * *

### 11.3.1	ProSe PC5 signalling message type

The purpose of the ProSe PC5 signalling message type information element is to indicate the type of messages used in ProSe PC5 signalling protocol.

The value part of the ProSe PC5 signalling message type information element used in the ProSe PC5 signalling messages is coded as shown in table 11.3.1.1.

The ProSe PC5 signalling message type is a type 3 information element, with the length of 1 octet.

Table 11.3.1.1: ProSe PC5 signalling message type

| Bits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |  |  |
| 0 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  |  |  | PROSE DIRECT LINK ESTABLISHMENT REQUEST |  |
| 0 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  |  |  | PROSE DIRECT LINK ESTABLISHMENT ACCEPT |  |
| 0 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 1 |  |  |  | PROSE DIRECT LINK ESTABLISHMENT REJECT |  |
| 0 | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 0 |  |  |  | PROSE DIRECT LINK MODIFICATION REQUEST |  |
| 0 | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 1 |  |  |  | PROSE DIRECT LINK MODIFICATION ACCEPT |  |
| 0 | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 1 |  | 0 |  |  |  | PROSE DIRECT LINK MODIFICATION REJECT |  |
| 0 | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 1 |  | 1 |  |  |  | PROSE DIRECT LINK RELEASE REQUEST |  |
| 0 | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 0 |  | 0 |  |  |  | PROSE DIRECT LINK RELEASE ACCEPT |  |
| 0 | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 0 |  | 1 |  |  |  | PROSE DIRECT LINK KEEPALIVE REQUEST |  |
| 0 | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 1 |  | 0 |  |  |  | PROSE DIRECT LINK KEEPALIVE RESPONSE |  |
| 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 1 |  | 1 |  |  |  | PROSE DIRECT LINK AUTHENTICATION REQUEST |
| 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 1 |  | 0 |  | 0 |  |  |  | PROSE DIRECT LINK AUTHENTICATION RESPONSE |
| 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 1 |  | 0 |  | 1 |  |  |  | PROSE DIRECT LINK AUTHENTICATION REJECT |
| 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 1 |  | 1 |  | 0 |  |  |  | PROSE DIRECT LINK SECURITY MODE COMMAND |
| 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 1 |  | 1 |  | 1 |  |  |  | PROSE DIRECT LINK SECURITY MODE COMPLETE |
| 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 0 |  | 0 |  | 0 |  |  |  | PROSE DIRECT LINK SECURITY MODE REJECT |
| 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 0 |  | 0 |  | 1 |  |  |  | PROSE DIRECT LINK REKEYING REQUEST |
| 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 0 |  | 1 |  | 0 |  |  |  | PROSE DIRECT LINK REKEYING RESPONSE |
| 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 0 |  | 1 |  | 1 |  |  |  | PROSE DIRECT LINK IDENTIFIER UPDATE REQUEST |
| 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 1 |  | 0 |  | 0 |  |  |  | PROSE DIRECT LINK IDENTIFIER UPDATE ACCEPT |
| 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 1 |  | 0 |  | 1 |  |  |  | PROSE DIRECT LINK IDENTIFIER UPDATE ACK |
| 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 1 |  | 1 |  | 0 |  |  |  | PROSE DIRECT LINK IDENTIFIER UPDATE REJECT |
| 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 1 |  | 1 |  | 1 |  |  |  | PROSE DIRECT LINK AUTHENTICATION FAILURE |
| 0 | 0 |  | 0 |  | 1 |  | 1 |  | 0 |  | 0 |  | 0 |  |  |  | PROSE ADDITIONAL PARAMETERS ANNOUNCEMENT REQUEST |  |
| 0 | 0 |  | 0 |  | 1 |  | 1 |  | 0 |  | 0 |  | 1 |  |  |  | PROSE ADDITIONAL PARAMETERS ANNOUNCEMENT RESPONSE |  |

* * * Next Change * * * *

## 12.7	Timers of 5G ProSe additional parameters announcement procedure

Table 12.7: 5G ProSe additional parameters announcement timers

| TIMER NUM. | TIMER VALUE | CAUSE OF START | NORMAL STOP | ON EXPIRY |
| --- | --- | --- | --- | --- |
| T5106 | NOTE 1 | Upon receiving a T5106 value in a PROSE ADDITIONAL PARAMETERS ANNOUNCEMENT RESPONSE message as described in subclause 10.3.25. | Upon receiving one or more new T5106 timer values in a PROSE ADDITIONAL PARAMETERS ANNOUNCEMENT_RESPONSE message. | Re-initiate the additional parameters announcement request procedure, if the upper layer application still needs to obtain NCGI or TAI of the cell serving the 5G ProSe layer-3 UE-to-network relay. |
| T5107 | NOTE 2 | Upon sending a PROSE ADDITIONAL PARAMETERS ANNOUNCEMENT RESPONSE message as described in subclause 10.3.25. | Upon sending a new PROSE ADDITIONAL PARAMETERS ANNOUNCEMENT RESPONSE message. | Stop the cell ID or TAI announcement in the PROSE PC5 DISCOVERY message for relay discovery additional Information. |
| NOTE 1: The value of this timer is provided by the 5G ProSe layer-3 UE-to-network relay UE during the 5G ProSe additional parameters announcement procedure. NOTE 2: The value of this timer is assigned by the 5G ProSe layer-3 UE-to-network relay UE during the 5G ProSe additional parameters announcement procedure. |  |  |  |  |

* * * End of Change * * * *