3GPP TSG-CT WG1 Meeting #133e-bis	C1-220432

- E-meeting, 17-21 January 2022

Source:	Huawei, HiSilicon, Samsung

Title:	Message Segmentation & Reassembly Procedures at client

Spec:	3GPP TS 24.538 v0.2.1

Agenda item:	17.2.30

Document for:	Agreement

1. Introduction

This pCR is to propose the procedures about Segmentation & Reassembly on client side.

2. Reason for Change

To address the Segmentation & Reassembly specifiecation in TS 23.554

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.538 v0.2.1.

* * * First Change * * * *

### 6.5.1	Procedure at MSGin5G Client

#### 6.5.1.1	General

If MSGin5G clinet is sending the segmented message, then the MSGin5G client act as Message Sender and procedure in clause 6.5.x.1 applies. If MSGin5G clinet is receiving the segmented message, then the MSGin5G client act as Message Sender and procedure in clause 6.5.x.2 applies.

### 6.5.x	Segment recovery procedure

### 6.5.x.1	Procedure at Message Sender

To support MSGin5G Message segmentation and seassembly, the Message Sender should perform the procedures specified in 6.4.1.1.2 additionally with the following procedures.

Upon receiving a CoAP POST request containing the MSGin5G service identifier and containing the Message Type with a value "segrec" which indicates the request is for messgage segment recovery, the MSGin5G Client shall send an CoAP ACK response to the request and then send each requested segmented message to the message receiver (e.g. AS, UE) as specified in 6.4.1.1.2 as requested in the received "List of Segment range" element.

#### 6.5.x.2	Procedure at Message receiver

Upon receiving an MSGin5G message, to support MSGin5G Message segmentation and seassembly, the MSGin5G Client should perform the procedures in 6.4.1.1.6 additionally with the following procedures.

#### 6.5.x.2.1	Segments recovery procedure when failed to receive all segments

If not all segments are received within expected time, the Message receiver shall send a CoAP POST request to the Message sender for recovering the segments. In the CoAP POST request, the Message receiver:

a)	shall set the "T" field in the CoAP header to 0 to indicate this request is the type of Confirmable;

b)	shall include the Message sender address in an CoAP Option, e.g. if the Message senderaddress is a URI, include a Uri-Path Option with the value of the URI;

c)	shall set the CoAP Content-Format to "50", i.e. application/json; and

d)	shall include the following information elements in the CoAP payload encoded in JSON format:

1)	a "MSGin5G service identifier" element set to indicate that this CoAP POST request is used for MSGin5G service;

2)	a "Message Type" element set with a value "segrec" to indicate this request is for segments recovery;

3)	a "Segmentation Set Identifier" element copied from one of the previous received segments; and

4)	a "List of Segment range" element to indicate the segments range which the client wants to recover, each segment range consist of start and end sequence number of missing segments e.g. (5-7, 10-10, 15-19);

If not all segments is not received within the expected time (based on configuration) then the Message receivermay consider as recovery failed or may initiate the procedure again with updated list of segment range.

NOTE:	The MSGin5G message segment recovery procedure may repeat based on the configuration.

##### 6.5.x.2.2	Segments received confirmation procedure

If the Message receiver determines that it receives all segments successfully, or the Message receiver determines it is failed (including recovery failed) to receive all segments, the Message receiver sends the message segments received confirmation to the Message sender by a CoAP POST request, in the CoAP POST request, the Message receiver:

a)	shall set the "T" field in the CoAP header to 0 to indicate this request is the type of Confirmable;

b)	shall include the Message senderaddress in an CoAP Option, e.g. if the Message senderaddress is a URI, include a Uri-Path Option with the value of the URI;

c)	shall set the CoAP Content-Format to "50", i.e. application/json; and

d)	shall include the following information elements in the CoAP payload encoded in JSON format:

1)	the "MSGin5G service identifier" element to indicate that this CoAP POST request is used for MSGin5G service;

2)	the "Message Type" element set with a value "segconfir" to indicate this request is for sending message segments received information;

3)	the "Segmentation Set Identifier" element copied from one of the previous received segments; and

4)	the "Result" element to indicate the segments are received successfull or failed;

* * * Next Change * * * *

<Proposed change in revision marks>

* * * Next Change * * * *

<Proposed change in revision marks>