3GPP TSG-CT WG1 Meeting #133e-bis	C1-220472 r1

- E-meeting, 17-21 January 2022

Source:	China Mobile

Title:	pCR on MSGin5G Server Reception of an MSGin5G delivery status report

Spec:	3GPP TS 24.538 v0.2.1

Agenda item:	17.2.30

Document for:	Agreement

1. Introduction

This pCR proposes the procedure of MSGin5G Server Reception of an MSGin5G delivery status report.

2. Reason for Change

Contribute new text

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.538 v0.2.1.

* * * First Change * * * *

##### 6.4.1.2.4	Reception of an MSGin5G delivery status report

Upon receiving an CoAP POST request containing the MSGin5G Service identifier and the "Message Type" is "imdn", if the "Number of individual messages" element and "List of individual messages" element are not be included and a "Delivery Status" element is included, the MSGin5G Server shall handle the CoAP POST request according to procedures specified in IETF RFC 7252 [5] with no additional requirement.

* * * Next Change * * * *

##### 6.4.1.2.5	Reception of an aggregated MSGin5G delivery status report

Upon receiving an CoAP POST request containing the MSGin5G Service identifier and the "Message Type" is "imdn", if a "Number of individual messages" and a "List of individual messages" are included, the MSGin5G Server determines that this message is an aggregated MSGin5G message. The MSGin5G Server shall handle the whole aggregated MSGin5G delivery status report according to procedures specified in clause 6.4.1.2.4.

* * * Next Change * * * *

<Proposed change in revision marks>