3GPP TSG-CT WG1 Meeting #133-bis-e	C1-220707

- E-meeting, 17-21 January 2022

Title:	LS on ECS provider identification in ECS address provisioning

Response to:

Release:	Rel-17

Work Item:	eEDGE_5GC, EDGEAPP

Source:	CT1

To:	SA6

Cc:	SA2, CT3

Contact person:	Lazaros Gkatzikis

Lazaros.Gkatzikis@nokia.com

+33 617 67 57 08

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

CT1 while working on stage-3 protocol implementation of the ECS Address Configuration Information provisioning to the UE (over PCO with Session Management procedures) has identified the following conflict on stage-2 specifications:

#### TS 23.548 Subclause 6.5.2.1 states in that ECS Address Configuration Information always contains an ECS provider ID:

"The ECS Address Configuration Information consists of one or more FQDN(s) and/or IP address(es) of Edge Configuration Server(s), and of an ECS Provider ID. It may be associated with spatial validity conditions. It is further described in TS 23.502 [3]. A UE may receive multiple instances of ECS Address Provisioning information (e.g. corresponding to different ECS Provider ID)."

- TS 23.558 Table 8.3.2.1-1: ECS configuration information indicates the ECS Provider ID as optional

Table 8.3.2.1-1: ECS configuration information

| Information element | Status | Description |
| --- | --- | --- |
| ECS address | M | One or more endpoint information (e.g. URI(s), FQDN(s), IP address(es)) of ECS(s) |
| ECS Provider Identifier | O | The identifier of the ECSP that provides the ECS. |

If the same PDU session is used by/for multiple ECS providers, the UE (EEC) needs to be able to identify the ECS provider that each provisioned address corresponds to.

CT1 would like to ask SA6 how is it ensured that the UE(EEC) can identify the ECS provider to be associated with each received ECS address, when the same PDU session is used by/for multiple ECS providers.

# 2	Actions

To SA6, SA2

ACTION:	CT1 kindly asks SA6 to provide answers to the question above and, if needed, to coordinate with SA2 to align their specifications on ECS address information.

# 3	Dates of next TSG CT WG1 meetings

CT1#134e	17th-25th February 2022	electronic meeting