3GPP TSG-CT WG1 Meeting #133-bis-e	C1-220468

- E-meeting, 17-21 January 2022

Source:	vivo, Nokia, Nokia Shanghai Bell

Title:	Update to QoS handling of L3 U2N relay without N3IWF to include PC5 QoS rule

Spec:	3GPP TS 24.554 v1.0.0

Agenda item:	17.2.18

Document for:	Agreement

1. Introduction

As per stage-2 spec (TS 23.304 h10) states that,

Based on signalled QoS rules (via SMF) or derived QoS rules (Uplink Uu via reflective QoS), the 5G ProSe Layer-3 UE-to-Network Relay may generate the Packet Filters used over PC5 reference point and use the L2 Link Modification procedures as defined in clause 6.4.3.6 to either update existing PC5 QoS Flow(s) or to set up new PC5 QoS Flow(s) (when the QFI to PC5 QoS Flow mapping does not exist). The 5G ProSe Layer-3 UE-to-Network Relay may also provide the PC5 QoS Rule(s) for the PC5 QoS Flow(s) to be added or modified to the 5G ProSe Layer-3 Remote UE.

So when L3 relay UE received QoS rules from SMF or derived QoS rules using reflective QoS, 5G ProSe Layer-3 UE-to-network relay UE may generate the packet filters used over PC5 reference point. And when performing L2 Link modification procedures, the relay UE may send the QoS rule(s) to remote UE, i.e. the relay UE may include the PC5 QoS rule(s) when initiating the 5G ProSe direct link modification procedure.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 24.554.

* * * First Change * * * *

##### 8.2.6.3.2	QoS flows handling initiated by the network

For QoS flows setup initiated by the network, upon reception QoS rules and QoS flow level QoS parameters from the SMF, the UE-to-network relay UE:

a)	shall determine the PQI based on the QoS mapping rules configured in clause 5.2.5;

b)	shall set the GFBR value for the PC5 GBR QoS flow to the GFBR value from the SMF, if any;

c)	shall set the MFBR value for the PC5 GBR QoS flow to the MFBR value from the SMF, if any;

d)	shall set the averaging window value for the PC5 GBR QoS flow to the averaging value from the SMF, if any; and

e)	optionally, initiate the 5G ProSe direct link modification procedure as defined in clause 7.2.3 to either update the existing PC5 QoS flow or to set up a new PC5 QoS flow. The 5G ProSe layer-3 UE-to-network relay UE may include the PC5 QoS rule(s) when initiating the 5G ProSe direct link modification procedure.

Alternatively, for dynamic QoS handling of remote UE using reflective QoS mechanism, upon the 5G ProSe layer-3 UE-to-network relay UE receiving a downlink user data packet along with the Reflective QoS Indication (RQI) as specified in 3GPP TS 24.501 [11], the 5G ProSe UE-to-network relay UE:

a)	creates a derived QoS rule by reflective QoS in the UE as specified in clause 6.2.5.1.4, 3GPP TS 24.501 [11];

b)	shall create a new derived QoS rule or update the existing derived QoS rule for the PC5 QoS flow based on the derived QoS rule from a);

c)	shall determine the corresponding PQI for the PC5 QoS flow based on the QoS mapping rules as specified in clause 5.2.5 and the 5QI value that corresponds to the QFI of the QoS rule from b);

d)	shall perform one of the following:

1)	if there is a PC5 QoS flow with the determined PQI, the 5G ProSe layer-3 UE-to-network relay UE shall perform the 5G ProSe direct link modification procedure as specified in clause 7.2.3 to associate the ProSe application on the existing PC5 QoS flow. The 5G ProSe layer-3 UE-to-network relay UE may include the PC5 QoS rule(s) associated with the updated PC5 QoS flow; or

2)	if there is no PC5 QoS flow with the determined PQI, the 5G ProSe layer-3 UE-to-network relay UE shall perform the 5G ProSe direct link modification procedure as specified in clause 7.2.3 to add a new PC5 QoS flow with the determined PQI and associate the ProSe application on the new PC5 QoS flow. The 5G ProSe layer-3 UE-to-network relay UE may include the PC5 QoS rule(s) associated with the newly added PC5 QoS flow.

When a derived QoS rule is deleted, the 5G ProSe layer-3 UE-to-network relay UE performs the 5G ProSe direct link modification procedure as specified in clause 7.2.3 to associate the ProSe application with a PC5 QoS flow such that the determined PQI maps to the 5QI of the signaled QoS rule.

* * * End of Change * * * *