3GPP TSG-CT WG1 Meeting #133e-bis	C1-22xxxx

- E-meeting, 17-21 January 2022

Source:	Nokia, Nokia Shanghai Bell, OPPO

Title:	Deriving the PC5 packet filters from the packet filters received over Uu reference point by the layer-3 Relay UE

Spec:	3GPP TS 24.554 V1.0.0

Agenda item:	17.2.18

Document for:	Agreement

1. Reason for Change

It is clarified in the agreed stage-2 CR S2-2108451 that the layer-3 Relay UE derives the PC5 packet filters (for PC QoS Rules) from the packet filters received over Uu from the network (SMF) as can be seen in the following text.

Based on signalled QoS rules (via SMF) or derived QoS rules (Uplink Uu via reflective QoS), the 5G ProSe Layer-3 UE-to-Network Relay may generate the Packet Filters used over PC5 reference point and use the L2 Link Modification procedures as defined in clause 6.4.3.6 to either update existing PC5 QoS Flow(s) or to set up new PC5 QoS Flow(s) (when the QFI to PC5 QoS Flow mapping does not exist). The 5G ProSe Layer-3 UE-to-Network Relay may also provide the PC5 QoS Rule(s) for the PC5 QoS Flow(s) to be added or modified to the 5G ProSe Layer-3 Remote UE.

This needs to be clarified in the corresponding clause in stage-3 spec, where there is a need to add a requirment for the Relay UE to do that mapping.

Also, some corrections to convert non-normative text to normative text are done in this pCR.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 24.554 V1.0.0.

* * * First Change * * * *

##### 8.2.6.3.2	QoS flows handling initiated by the network

For QoS flows setup initiated by the network, upon reception QoS rules and QoS flow level QoS parameters from the SMF, the 5G ProSe layer-3 UE-to-network relay UE:

a)	shall determine the PQI based on the QoS mapping rules configured in clause 5.2.5;

b)	shall set the GFBR value for the PC5 GBR QoS flow to the GFBR value from the SMF, if any;

c)	shall set the MFBR value for the PC5 GBR QoS flow to the MFBR value from the SMF, if any;

d)	shall set the averaging window value for the PC5 GBR QoS flow to the averaging value from the SMF, if any;

e)	may derive the packet filter(s) used for PC5 QoS rule(s) from the packet filter(s) used over Uu reference; and

f)	may initiate the 5G ProSe direct link modification procedure as defined in clause 7.2.3 to either update the existing PC5 QoS flow or to set up a new PC5 QoS flow.

Alternatively, for dynamic QoS handling of remote UE using reflective QoS mechanism, upon the 5G ProSe layer-3 UE-to-network relay UE receiving a downlink user data packet along with the Reflective QoS Indication (RQI) as specified in 3GPP TS 24.501 [11], the 5G ProSe UE-to-network relay UE:

a)	shall indicate the lower layers to create a derived QoS rule by reflective QoS in the UE as specified in clause 6.2.5.1.4, 3GPP TS 24.501 [11];

b)	shall create a new derived PC5 QoS rule or update the existing derived PC5 QoS rule for the PC5 QoS flow based on the derived QoS rule from a);

c)	shall determine the corresponding PQI for the PC5 QoS flow based on the QoS mapping rules as specified in clause 5.2.5 and the 5QI value that corresponds to the QFI of the QoS rule from b); and

d)	shall perform one of the following:

1)	if there is a PC5 QoS flow with the determined PQI, the 5G ProSe layer-3 UE-to-network relay UE shall perform the 5G ProSe direct link modification procedure as specified in clause 7.2.3 to associate the ProSe application on the existing PC5 QoS flow; or

2)	if there is no PC5 QoS flow with the determined PQI, the 5G ProSe layer-3 UE-to-network relay UE shall perform the 5G ProSe direct link modification procedure as specified in clause 7.2.3 to add a new PC5 QoS flow with the determined PQI and associate the ProSe application on the new PC5 QoS flow.

When a derived QoS rule is deleted, the 5G ProSe layer-3 UE-to-network relay UE performs the 5G ProSe direct link modification procedure as specified in clause 7.2.3 to associate the ProSe application with a PC5 QoS flow such that the determined PQI maps to the 5QI of the signaled QoS rule.

* * * End of Changes * * * *