3GPP TSG-CT WG1 Meeting #133e-bis	C1-22xxxx

- E-meeting, 17-21 January 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Clarifications on the parameters provided in the Relay Discovery Additional Information procedure and the destination layer-2 ID for Relay discovery

Spec:	3GPP TS 24.554 V1.0.0

Agenda item:	17.2.18

Document for:	Agreement

1. Reason for Change

.Stage-2 CR S2-2109160 has clarified what are the additional information that are provided in the procedure of Relay Discovery Additional Information, where the procedure is applicable for both layer-2 and layer-3 relaying, as following:

The following parameters may be used in the Relay Discovery Additional Information message (for Model A) based on the procedure defined in clause 6.5.1.3 for 5G ProSe UE-to-Network Relay where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message, and the other parameters are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-Network Relay self-selects a Source Layer-2 ID to send the Relay Discovery Additional Information message.

-	Destination Layer-2 ID: the Destination Layer-2 ID to send the Relay Discovery Additional Information message is selected based on the configuration as described in clause 5.1.4.1.

-	Relay Service Code: the Relay Service Code associated with the message. The Relay Service Code is used to identify the security parameters needed by the receiving UE to process the discovery message.

-	Announcer Info: provides information about the announcing user.

-	Additional parameters: the additional parameters for 5G ProSe Layer-3 UE-to-Network Relay (when applicable) are defined in clause 5.8.3.2.

(…)

The following additional parameters may be used in the Relay Discovery Additional Information message (for Model A) for 5G ProSe Layer-3 UE-to-Network Relay:

-	NCGI: indicates the NCGI of the serving cell of the 5G ProSe Layer-3 UE-to-Network Relay. This parameter may be requested by application running on 5G ProSe Layer-3 Remote UE.

-	TAI: indicates the Tracking Area Identity of the serving cell of the 5G ProSe Layer-3 UE-to-Network Relay. This parameter may be used by 5G ProSe Layer-3 Remote UE to select a N3IWF.

It is worth to note that NCGI and TAI are provided only for layer-3 relaying case.

Hence in this pCR some clarifications are done in stage-3 spec regarding the purpose and parameters provided in the Relay Discovery Addiotional Information procedure.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 24.554 V1.0.0.

* * * First Change * * * *

###### 8.2.1.2.4.1	General

The purpose of the announcing UE procedure for relay discovery additional information is to announce to the 5G ProSe remote UEs additional information about:

a)	the relay service code of the 5G ProSe UE-to-network relay UE;

b)	the announcer info of the announcing user;

c)	the NCGI of the cell serving the 5G ProSe layer-3 UE-to-network relay UE; and

d)	the TAI of the cell serving the 5G ProSe layer-3 UE-to-network relay UE;

as defined in 3GPP TS 23.304 [2].

* * * Next Change * * * *

###### 8.2.1.2.5.1	General

The purpose of the monitoring UE procedure for relay discovery additional information is to enable a 5G ProSe remote UE to become aware of the additional information of the 5G ProSe UE-to-network relay UE as  described in clause 8.2.1.2.4.1.

* * * End of Changes * * * *