3GPP TSG-CT WG1 Meeting #133e-bis	C1-22xxxx

- E-meeting, 17-21 January 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Defining the Metadata IE that is used in the 5G ProSe direct discovery messages

Spec:	3GPP TS 24.554 V1.0.0

Agenda item:	17.2.18

Document for:	Agreement

1. Reason for Change

The Metadata information is used in the 5G ProSe direct discovery message but it is not defined in the IEs clauses. Moreover, stage-2 CR S2-2109170 clarified the usage of the Metadata information for all discovery cases (5G ProSe direct discovery announcement, restricted 5G ProSe direct discovery response, group member discovery announcement…etc). Also it is clarified that the metadata with reasonable message size can be included within PC5 discovery messages, and this is satisfied for the application layer discovery message.

Hence this pCR proposes the following changes:

1- Describing the usage of Metadata information in all possible PC5 discovery messages in the messages tables.

2- Removing the Metadata IE from the "PROSE PC5 DISCOVERY message for restricted 5G ProSe direct discovery solicitation", as ths informaton is provided only by the Announcer UE as per CR S2-2109170.

3- Defining the Metadata IE, where the format of the content of Metadata information is out of scope of the specification.

4- Resolving the Editor's note that is related to how to transmit the application layer discovery message and indicating that this is UE implementation specific.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 24.554 V1.0.0.

* * * First Change * * * *

##### 7.4.2.1.1	Initiation of forming a group

Before the UE is requested by upper layers to send data unit(s) of a ProSe application identified by a ProSe identifier using 5G ProSe communication over PC5, for the users sharing the same application layer group ID as configured in clause 5.2.4 may perform a group member discovery procedure as specified in clause 6.2.15.

When the group is formed, the following information may be exchanged within the group members included in the application layer discovery message:

a)	group size; and

b)	member ID.

NOTE:	Whether to transmit the application layer discovery message as metadata in a 5G ProSe direct discovery message or as user traffic over PC5 is UE implementation specific.

* * * Next Change * * * *

### 10.2.1	Message definition

This message is sent by the UE over the PC5 interface for open 5G ProSe direct discovery and restricted 5G ProSe direct discovery. See table 10.2.1.1, table 10.2.1.2, table 10.2.1.3, table 10.2.1.4, table 10.2.1.5, table 10.2.1.6, table 10.2.1.7, table 10.2.1.8, table 10.2.1.9, table 10.2.1.10 and table 10.2.1.11.

Message type:	PROSE PC5 DISCOVERY

Significance:	dual

Direction:		UE to peer UE

Editor's note:	The IEs related to security are FFS and depend on SA3 requirements.

Editor's note:	The condition of the optional IEs is FFS.

Editor's note:	Whether Metadata IE is subject to security protection is FFS and depends on SA3 requirements.

Table 10.2.1.1: PROSE PC5 DISCOVERY message content for open 5G ProSe direct discovery announcement

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe application code | ProSe application code 11.2.2 | M | V | 23 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | UTC-based counter LSB | Binary 11.2.5 | M | V | 1 |
| XY | Metadata | Metadata 11.2.B | O | TLV | 3-257 |
| NOTE: The discovery type is set to "Open discovery" and the content type is set to "Announcement". |  |  |  |  |  |

Table 10.2.1.2: PROSE PC5 DISCOVERY message content for restricted 5G ProSe direct discovery announcement

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe restricted code | ProSe restricted code 11.2.3 | M | V | 23 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | UTC-based counter LSB | Binary 11.2.5 | M | V | 1 |
| XY | Metadata | Metadata 11.2.B | O | TLV | 3-257 |
| NOTE: The discovery type is set to "Restricted discovery" and the content type is set to "Announcement". |  |  |  |  |  |

Table 10.2.1.3: PROSE PC5 DISCOVERY message content for restricted 5G ProSe direct discovery solicitation

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe query code | ProSe restricted code 11.2.3 | M | V | 23 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | UTC-based counter LSB | Binary 11.2.5 | M | V | 1 |
| NOTE: The discovery type is set to "Restricted discovery" and the content type is set to "Solicitation". |  |  |  |  |  |

Table 10.2.1.4: PROSE PC5 DISCOVERY message content for restricted 5G ProSe direct discovery response

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe response code | ProSe restricted code 11.2.3 | M | V | 23 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | UTC-based counter LSB | Binary 11.2.5 | M | V | 1 |
| XY | Metadata | Metadata 11.2.B | O | TLV | 3-257 |
| NOTE: The discovery type is set to "Restricted discovery" and the content type is set to "response". |  |  |  |  |  |

Table 10.2.1.5: PROSE PC5 DISCOVERY message for group member discovery announcement

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | Application layer group ID | Application layer group ID 11.2.6 | M | LV | 2-256 |
|  | Announcer info | User info ID 11.2.7 | M | V | 6 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | UTC-based counter LSB | Binary 11.2.5 | M | V | 1 |
| XY | Metadata | Metadata 11.2.B | O | TLV | 3-257 |
| NOTE: The discovery type is set to "Restricted discovery", the content type is set to "Group member discovery announcement/group member discovery response" and the discovery model is set to "Model A". |  |  |  |  |  |

Table 10.2.1.6: PROSE PC5 DISCOVERY message for group member discovery solicitation

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | Application layer group ID | Application layer group ID 11.2.6 | M | LV | 2-256 |
|  | Discoverer info | User info ID 11.2.7 | M | V | 6 |
|  | GMDS composition | GMDS composition 11.2.8 | M | V | 1 |
|  | Target user info | Target user info 11.2.9 | C (NOTE 2) | V | 6 |
|  | Target group info | Target group info 11.2.10 | C (NOTE 2) | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | UTC-based counter LSB | Binary 11.2.5 | M | V | 1 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Group member discovery solicitation" and the discovery model is set to "Model B". NOTE 2: Presence of the target user info and of target group info is indicated by the GMDS composition. |  |  |  |  |  |

Table 10.2.1.7: PROSE PC5 DISCOVERY message for group member discovery response

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | Application layer group ID | Application layer group ID 11.2.6 | M | LV | 2-256 |
|  | Discoveree info | User info ID 11.2.7 | M | V | 6 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | UTC-based counter LSB | Binary 11.2.5 | M | V | 1 |
| XY | Metadata | Metadata 11.2.B | O | TLV | 3-257 |
| NOTE: The discovery type is set to "Restricted discovery", the content type is set to "Group member discovery announcement/group member discovery response" and the discovery model is set to "Model B". |  |  |  |  |  |

Table 10.2.1.8: PROSE PC5 DISCOVERY message for UE-to-network relay discovery announcement

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | Announcer info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code (NOTE 2) | Relay service code 11.2.11 | M | V | 3 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
|  | Status indicator | Status indicator 11.2.12 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
| yy | NCGI | NCGI 11.2.15 | O | TV | 9 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "UE-to-network relay discovery announcement/UE-to-network relay discovery response" and the discovery model is set to "Model A". NOTE 2: If the announcing UE works as a 5G ProSe Layer-3 UE-to-network relay UE, the S-NSSAI associated with the relay service code belongs to the allowed NSSAI of the UE. |  |  |  |  |  |

Table 10.2.1.9: PROSE PC5 DISCOVERY message for UE-to-network relay discovery solicitation

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | Discoverer info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.11 | M | V | 3 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
| NOTE: The discovery type is set to "Restricted discovery", the content type is set to "UE-to-network relay discovery solicitation" and the discovery model is set to "Model B". |  |  |  |  |  |

Table 10.2.1.10: PROSE PC5 DISCOVERY message for UE-to-network relay discovery response

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | Discoveree info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code (NOTE 2) | Relay service code 11.2.11 | M | V | 3 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
|  | Status indicator | Status indicator 11.2.12 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
| yy | NCGI | NCGI 11.2.15 | O | TV | 9 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "UE-to-network relay discovery announcement/UE-to-network relay discovery response" and the discovery model is set to "Model B". NOTE 2: If the discoveree UE works as a 5G ProSe Layer-3 UE-to-network relay UE, the S-NSSAI associated with the relay service code belongs to the allowed NSSAI of the UE. |  |  |  |  |  |

Table 10.2.1.11: PROSE PC5 DISCOVERY message for relay discovery additional information

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | Relay service code | Relay service code 11.2.11 | M | V | 3 |
|  | Announcer info | User info ID 11.2.7 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
| yy | NCGI | NCGI 11.2.15 | O | TV | 9 |
| xx | Relay TAI | TAI 11.2.13 | O | TV | 4 |
| NOTE: The discovery type is set to "Restricted discovery", the content type is set to "Relay discovery additional information" and the discovery model is set to "Model A". |  |  |  |  |  |

* * * Next Change * * * *

### 10.2.C	Metadata

The Metadata information element may be included:

a)	to provide the application layer metadata information in the following messages:

1)	the PROSE PC5 DISCOVERY message for open 5G ProSe direct discovery announcement as in table 10.2.1.1;

2)	the PROSE PC5 DISCOVERY message for restricted 5G ProSe direct discovery announcement as in table 10.2.1.2, and

3)	the PROSE PC5 DISCOVERY message for restricted 5G ProSe direct discovery response as in table 10.2.1.4; or

b)	to provide the application layer discovery message in the following messages:

1)	the PROSE PC5 DISCOVERY message for group member discovery announcement as in table 10.2.1.5; and

2)	the PROSE PC5 DISCOVERY message for group member discovery response as in table 10.2.1.7.

NOTE:	The format of the application layer metadata information and the application layer discovery message are out of scope of this specification.

* * * Next Change * * * *

### 11.2.B	Metadata

The Metadata parameter carries the application layer metadata information.

The Metadata information element is coded as shown in figure 11.2.B.1 and table 11.2.B.1.

The Metadata is a type 4 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Metadata IEI |  |  |  |  |  |  |  |  | octet 1 |
| Length of Metadata contents |  |  |  |  |  |  |  |  | octet 2 |
| Metadata contents |  |  |  |  |  |  |  |  | octet 3 |
|  |  |  |  |  |  |  |  |  | octet m |

Figure 11.2.B.1: Metadata information element

Table 11.2.B.1: Metadata information element

The length of Metadata contents field contains the binary coded representation of the length of the Metadata contents field. The Metadata contents field contains the octets indicating the Metadata parameter. The format of the Metadata parameter is out of scope of this specification.

* * * End of Changes * * * *