3GPP TSG-CT WG1 Meeting #133e-bis	C1-22xxxx

- E-meeting, 17-21 January 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Modifications for Security parameters related to Restricted 5G ProSe Direct Discovery (Model A and Model B)

Spec:	3GPP TS 24.554 V1.0.0

Agenda item:	17.2.18

Document for:	Agreement

1. Reason for Change

The 5G ProSe security aspects has been specified in TS 33.503. Multiple corrections/modifications are needed for Security parameters that are defined in TS 24.554 and related to Restricted 5G ProSe Direct Discovery (Model A and Model B) to match the security aspects defined in TS 33.503. The following modifications are done within this pCR:

The PC5 security policies can be indicated from the 5G DDNMF to the UE during the Discovery Response message over PC3a, as specified in TS 33.503 as following:

-	During the discovery request procedure, 5G DDNMF may optionally provide the PC5 security policies to the UEs.

And this needs to be implemented in the related Discovery messages in TS 24.554, where the XML schema and the semantics of the DISCOVERY_RESPONSE are updated to cover this requirement.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 24.554 V1.0.0.

* * * First Change * * * *

### 10.5.3	XML schema

Implementations in compliance with the present document shall implement the XML schema defined below for messages used in 5G ProSe direct discovery procedures over PC3a interface.

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema"

xmlns="urn:3GPP:ns:5GProSe:Discovery:2021"

elementFormDefault="qualified"

targetNamespace="urn:3GPP:ns:5GProSe:Discovery:2021">

<xs:annotation>

<xs:documentation>

Info for 5G ProSe Discovery Control Messages Syntax

</xs:documentation>

</xs:annotation>

<!-- Complex types defined for parameters with complicated structure -->

<xs:complexType name="AppID-info">

<xs:sequence>

<xs:element name="OS-ID">

<xs:simpleType>

<xs:restriction base="xs:hexBinary">

<xs:length value="16"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="OS-App-ID" type="xs:string"/>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="PLMN-info">

<xs:sequence>

<xs:element name="mcc" type="xs:integer"/>

<xs:element name="mnc" type="xs:integer"/>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="SUPI-info">

<xs:sequence>

<xs:element name="MCC" type="xs:integer"/>

<xs:element name="MNC" type="xs:integer"/>

<xs:element name="MSIN" type="xs:integer"/>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="DiscFilter-info">

<xs:sequence>

<xs:element name="ProSe-Application-Code" type="xs:hexBinary"/>

<xs:element name="ProSe-Application-Mask" type="xs:hexBinary" maxOccurs="unbounded"/>

<xs:element name="TTL-timer-T5064" type="xs:integer"/>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="MatchingFilter-info">

<xs:sequence>

<xs:element name="Code" type="xs:hexBinary"/>

<xs:element name="Mask" type="xs:hexBinary" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="DUCK-info">

<xs:sequence>

<xs:element name="discovery-user-confidentiality-key" type="xs:hexBinary"/>

<xs:element name="encrypted-bitmask" type="xs:hexBinary"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="RestrictedDiscFilter-info">

<xs:sequence>

<xs:element name="filter" type="MatchingFilter-info" maxOccurs="unbounded"/>

<xs:element name="TTL-timer-T5066" type="xs:integer"/>

<xs:element name="restricted-security" type="Restricted-Security-info" minOccurs="0" />

<xs:element name="RPAUID" type="xs:string" minOccurs="0" />

<xs:element name="DUSK" type="xs:hexBinary" minOccurs="0" />

<xs:element name="DUIK" type="xs:hexBinary" minOccurs="0" />

<xs:element name="MIC-check-indicator" type="xs:boolean" minOccurs="0" />

<xs:element name="DUCK" type="DUCK-info" minOccurs="0" />

<xs:element name="metadata-indicator" type="xs:integer" minOccurs="0"/>

<xs:element name="metadata" type="xs:string" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="RestrictedCodeSuffixRange-info">

<xs:sequence>

<xs:element name="beginning-suffix-code" type="xs:hexBinary" />

<xs:element name="ending-suffix-code" type="xs:hexBinary" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="RestrictedMonitoringUpdate-info">

<xs:sequence>

<xs:element name="updated-filter" type="RestrictedDiscFilter-info" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="RestrictedAnnouncingUpdate-info">

<xs:sequence>

<xs:element name="ProSe-Restricted-Code" type="xs:hexBinary" />

<xs:element name="validity-timer-T5062" type="xs:integer" />

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="MonitoringUpdate-info">

<xs:sequence>

<xs:element name="updated-filter" type="DiscFilter-info" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="AnnouncingUpdate-info">

<xs:sequence>

<xs:element name="ProSe-Application-Code" type="xs:hexBinary" />

<xs:element name="validity-timer-T5060" type="xs:integer" />

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="Update-Option-info">

<xs:choice>

<xs:element name="update-info-restricted-announce" type="RestrictedAnnouncingUpdate-info" />

<xs:element name="update-info-restricted-monitor" type="RestrictedMonitoringUpdate-info" />

<xs:element name="update-info-open-annnounce" type="AnnouncingUpdate-info" />

<xs:element name="update-info-open-monitor" type="MonitoringUpdate-info"/>

<xs:element name="anyExt" type="anyExtType" />

<xs:any namespace="##other" processContents="lax"/>

</xs:choice>

</xs:complexType>

<xs:complexType name="Restricted-Code-Option-info">

<xs:choice>

<xs:element name="ProSe-Restricted-Code" type="xs:hexBinary" />

<xs:element name="ProSe-Response-Code" type="xs:hexBinary" />

<xs:element name="anyExt" type="anyExtType" />

<xs:any namespace="##other" processContents="lax"/>

</xs:choice>

</xs:complexType>

<xs:complexType name="Subquery-info">

<xs:sequence>

<xs:element name="ProSe-Rquery-Code" type="xs:hexBinary" />

<xs:element name="response-filter" type="MatchingFilter-info" maxOccurs="unbounded"/>

<xs:element name="validity-timer-T5070" type="xs:integer"/>

<xs:element name="restricted-security" type="Restricted-Security-info" minOccurs="0" />

<xs:element name="RPAUID" type="xs:string" minOccurs="0" />

<xs:element name="metadata" type="xs:string" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="Restricted-Security-info">

<xs:sequence>

<xs:element name="DUSK" type="xs:hexBinary" minOccurs="0" />

<xs:element name="DUIK" type="xs:hexBinary" minOccurs="0" />

<xs:element name="DUCK" type="DUCK-info" minOccurs="0" />

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="ApplicationCodeSuffixRange-info">

<xs:sequence>

<xs:element name="beginning-suffix-code" type="xs:hexBinary" />

<xs:element name="ending-suffix-code" type="xs:hexBinary" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="ProSeApplicationCodeACE-info">

<xs:sequence>

<xs:element name=" ProSe-Application-Code-Prefix" type="xs:hexBinary" />

<xs:element name=" ProSe-Application-Code-Suffix-Range" type="ApplicationCodeSuffixRange-info" maxOccurs="unbounded" />

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="PC5-Security-Policies-info">

<xs:sequence>

<xs:element name="signalling-integrity-protection-policy" type="xs:integer"/>

<xs:element name="signalling-ciphering-policy" type="xs:integer"/>

<xs:element name="user-plane-integrity-protection-policy" type="xs:integer"/>

<xs:element name="user-plane-ciphering-policy" type="xs:integer"/>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- Complex types defined for transaction-level -->

<xs:complexType name="AnnounceRsp-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="ProSe-Application-Code" type="xs:hexBinary" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="ProSe-Application-Code-ACE" type="ProSeApplicationCodeACE-info" minOccurs="0"/>

<xs:element name="validity-timer-T5060" type="xs:integer" minOccurs="0" />

<xs:element name="discovery-key" type="xs:hexBinary" minOccurs="0" />

<xs:element name="discovery-entry-ID" type="xs:integer" minOccurs="0" />

<xs:element name="ACE-enabled-indicator" type="xs:integer" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="MonitorRsp-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="discovery-filter" type="DiscFilter-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="discovery-entry-ID" type="xs:integer" minOccurs="0" />

<xs:element name="ACE-enabled-indicator" type="xs:integer" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="DiscReq-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="command" type="xs:integer"/>

<xs:element name="UE-identity" type="SUPI-info"/>

<xs:element name="ProSe-Application-ID" type="xs:string"/>

<xs:element name="application-identity" type="AppID-info"/>

<xs:element name="discovery-entry-ID" type="xs:integer" minOccurs="0" />

<xs:element name="Requested-Timer" type="xs:integer" minOccurs="0" />

<xs:element name="metadata" type="xs:string" minOccurs="0"/>

<xs:element name="Announcing-PLMN-ID" type="PLMN-info" minOccurs="0" />

<xs:element name="ACE-enabled-indicator" type="xs:integer" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="RestrictedDiscReq-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="command" type="xs:integer"/>

<xs:element name="UE-identity" type="SUPI-info"/>

<xs:element name="RPAUID" type="xs:string"/>

<xs:element name="application-identity" type="AppID-info"/>

<xs:element name="discovery-type" type="xs:integer"/>

<xs:element name="ACE-enabled-indicator" type="xs:integer" minOccurs="0"/>

<xs:element name="announcing-type" type="xs:integer" minOccurs="0"/>

<xs:element name="application-level-container" type="xs:hexBinary" minOccurs="0"/>

<xs:element name="discovery-model" type="xs:integer" minOccurs="0"/>

<xs:element name="Announcing-PLMN-ID" type="PLMN-info" minOccurs="0" />

<xs:element name="discovery-entry-ID" type="xs:integer"/>

<xs:element name="Requested-Timer" type="xs:integer" minOccurs="0" />

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="RestrictedAnnounceRsp-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="ProSe-Restricted-Code" type="xs:hexBinary" minOccurs="0"/>

<xs:element name="ProSe-Restricted-Code-Suffix-Range" type="RestrictedCodeSuffixRange-info" minOccurs="0"/>

<xs:element name="validity-timer-T5062" type="xs:integer" minOccurs="0"/>

<xs:element name="ACE-enabled-indicator" type="xs:integer" minOccurs="0" />

<xs:element name="restricted-code-security-material" type="Restricted-Security-info" minOccurs="0" />

<xs:element name="on-demand-announcing-enabled-indicator" type="xs:boolean" minOccurs="0" />

<xs:element name="discovery-entry-ID" type="xs:integer"/>

<xs:element name="PC5-security-policies" type="xs:PC5-Security-Policies-info" minOccurs="0" />

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="RestrictedMonitorRsp-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="restricted-discovery-filter" type="RestrictedDiscFilter-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="ACE-enabled-indicator" type="xs:integer" minOccurs="0" />

<xs:element name="application-level-container" type="xs:hexBinary"/>

<xs:element name="discovery-entry-ID" type="xs:integer"/>

<xs:element name="PC5-security-policies" type="xs:PC5-Security-Policies-info" minOccurs="0" />

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="RestrictedDiscovereeRsp-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="ProSe-Response-Code" type="xs:hexBinary" />

<xs:element name="query-filter" type="MatchingFilter-info" maxOccurs="unbounded"/>

<xs:element name="validity-timer-T5068" type="xs:integer"/>

<xs:element name="restricted-code-security-material " type="Restricted-Security-info" minOccurs="0" />

<xs:element name="discovery-entry-ID" type="xs:integer"/>

<xs:element name="PC5-security-policies" type="xs:PC5-Security-Policies-info" minOccurs="0" />

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="RestrictedDiscovererRsp-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="subquery-result" type="Subquery-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="discovery-entry-ID" type="xs:integer"/>

<xs:element name="PC5-security-policies" type="xs:PC5-Security-Policies-info" minOccurs="0" />

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="RejectRsp-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="PC3a-control-protocol-cause-value" type="xs:integer"/>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="UE-RejectRsp-info">

<xs:sequence>

<xs:element name="DDNMF-transaction-ID" type="xs:integer"/>

<xs:element name="PC3a-control-protocol-cause-value" type="xs:integer"/>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="MatchRep-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="ProSe-Application-Code" type="xs:hexBinary"/>

<xs:element name="UE-identity" type="SUPI-info"/>

<xs:element name="Monitored-PLMN-ID" type="PLMN-info"/>

<xs:element name="VPLMN-ID" type="PLMN-info" minOccurs="0"/>

<xs:element name="MIC" type="xs:hexBinary"/>

<xs:element name="UTC-based-counter" type="xs:hexBinary"/>

<xs:element name="Metadata-flag" type="xs:boolean"/>

<xs:element name="MessageType" type="xs:hexBinary" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="RestrictedMatch-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="UE-identity" type="SUPI-info"/>

<xs:element name="discovery-type" type="xs:integer"/>

<xs:element name="application-identity" type="AppID-info"/>

<xs:element name="RPAUID" type="xs:string"/>

<xs:element name="Restricted-Code-Discovered" type="Restricted-Code-Option-info" />

<xs:element name="MIC" type="xs:hexBinary" minOccurs="0"/>

<xs:element name="MessageType" type="xs:hexBinary" minOccurs="0"/>

<xs:element name="UTC-based-counter" type="xs:hexBinary" minOccurs="0"/>

<xs:element name="Metadata-flag" type="xs:boolean" />

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="MatchAck-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="ProSe-Application-ID" type="xs:string"/>

<xs:element name="validity-timer-T5072" type="xs:integer"/>

<xs:element name="metadata" type="xs:string" minOccurs="0"/>

<xs:element name="metadata-index-mask" type="xs:hexBinary" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="match-report-refresh-timer-T5074" type="xs:integer"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="RestrictedMatchAck-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="application-identity" type="AppID-info"/>

<xs:element name="RPAUID" type="xs:string"/>

<xs:element name="validity-timer-T5076" type="xs:integer"/>

<xs:element name="metadata" type="xs:string" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="match-report-refresh-timer-T5077" type="xs:integer"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="MatchReject-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="PC3a-control-protocol-cause-value" type="xs:integer"/>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="DiscUpdateReq-info">

<xs:sequence>

<xs:element name="DDNMF-transaction-ID" type="xs:integer"/>

<xs:element name="UE-identity" type="SUPI-info"/>

<xs:element name="discovery-entry-ID" type="xs:integer"/>

<xs:element name="update-info" type="Update-Option-info" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="DiscUpdateRsp-info">

<xs:sequence>

<xs:element name="DDNMF-transaction-ID" type="xs:integer"/>

<xs:element name="discovery-entry-ID" type="xs:integer"/>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="AnnouncingAlertReq-info">

<xs:sequence>

<xs:element name="DDNMF-transaction-ID" type="xs:integer"/>

<xs:element name="RPAUID" type="xs:string"/>

<xs:element name="UE-identity" type="SUPI-info"/>

<xs:element name="discovery-entry-ID" type="xs:integer"/>

<xs:element name="ProSe-Restricted-Code" type="xs:hexBinary"/>

<xs:element name="ProSe-Restricted-Code-Suffix-Range" type="RestrictedCodeSuffixRange-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="AnnouncingAlertRsp-info">

<xs:sequence>

<xs:element name="DDNMF-transaction-ID" type="xs:integer"/>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- Complex types defined for Message-level -->

<xs:complexType name="prose-direct-discovery-request">

<xs:sequence>

<xs:element name="discovery-request" type="DiscReq-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="restricted-discovery-request" type="RestrictedDiscReq-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="network-initiated-transaction-method" type="xs:integer"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="prose-direct-discovery-response">

<xs:sequence>

<xs:element name="Current-Time" type="xs:dateTime"/>

<xs:element name="Max-Offset" type="xs:integer"/>

<xs:element name="response-announce" type="AnnounceRsp-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="response-monitor" type="MonitorRsp-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="restricted-announce-response" type="RestrictedAnnounceRsp-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="restricted-monitor-response" type="RestrictedMonitorRsp-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="restricted-discoveree-response" type="RestrictedDiscovereeRsp-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="restricted-discoverer-response" type="RestrictedDiscovererRsp-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="response-reject" type="RejectRsp-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="network-initiated-transaction-method" type="xs:integer"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="prose-direct-discovery-update-request">

<xs:sequence>

<xs:element name="discovery-update-request" type="DiscUpdateReq-info" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="prose-direct-discovery-update-response">

<xs:sequence>

<xs:element name="response-update" type="DiscUpdateRsp-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="response-reject" type="UE-RejectRsp-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="prose-direct-discovery-match-report">

<xs:sequence>

<xs:element name="match-report" type="MatchRep-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="restricted-match" type="RestrictedMatch-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="prose-direct-discovery-match-report-ack">

<xs:sequence>

<xs:element name="Current-Time" type="xs:dateTime"/>

<xs:element name="match-ack" type="MatchAck-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="match-reject" type="MatchReject-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="restricted-match-ack" type="RestrictedMatchAck-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="prose-direct-discovery-announcing-alert-request">

<xs:sequence>

<xs:element name="announcing-alert-request" type="AnnouncingAlertReq-info" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="prose-direct-discovery-announcing-alert-response">

<xs:sequence>

<xs:element name="announcing-alert-response" type="AnnouncingAlertRsp-info" maxOccurs="unbounded"/>

<xs:element name="response-reject" type="UE-RejectRsp-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!--  extension allowed -->

<xs:complexType name="DiscMsgExtType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!--  XML attribute for any future extensions  -->

<xs:complexType name="anyExtType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<!--  Top levelDiscovery Message definition  -->

<xs:element name="prose-discovery-message">

<xs:complexType>

<xs:choice>

<xs:element name="DISCOVERY_REQUEST" type="prose-direct-discovery-request"/>

<xs:element name="DISCOVERY_RESPONSE" type="prose-direct-discovery-response"/>

<xs:element name="MATCH_REPORT" type="prose-direct-discovery-match-report"/>

<xs:element name="MATCH_REPORT_ACK" type="prose-direct-discovery-match-report-ack"/>

<xs:element name="DISCOVERY_UPDATE_REQUEST" type="prose-direct-discovery-update-request"/>

<xs:element name="DISCOVERY_UPDATE_RESPONSE" type="prose-direct-discovery-update-response"/>

<xs:element name="ANNOUNCING_ALERT_REQUEST" type="prose-direct-discovery-announcing-alert-request"/>

<xs:element name="ANNOUNCING_ALERT_RESPONSE" type="prose-direct-discovery-announcing-alert-response"/>

<xs:element name="message-ext" type="DiscMsgExtType"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:choice>

</xs:complexType>

</xs:element>

</xs:schema>

An entity receiving the XML body ignores any unknown XML element and any unknown XML attribute.

* * * Next Change * * * *

#### 10.2.4.3	Semantics of <DISCOVERY_RESPONSE>

The <DISCOVERY_RESPONSE> element contains one or more of the following elements:

a)	a <Current-Time> element containing the parameter defined in clause 11.4.2.16;

b)	a <Max-Offset> element containing the parameter defined in clause 11.4.2.17;

c)	zero, one or more <response-announce> element which contains transactions sent from the 5G DDNMF to the UE as a response to an announcing request if the 5G DDNMF accepts the request. Each <response-announce> consists of:

1)	a <transaction-ID> element containing the parameter defined in clause 11.4.2.1;

2)	zero, one or more <ProSe-Application-Code> elements containing the parameter defined in clause 11.4.2.6;c)	zero or one <ProSe application code-ACE> element containing the parameter defined in clause 11.4.2.45;

3)	zero, or one <validity-timer-T5060> element containing the parameter defined in 11.4.2.7;

4)	zero, or, one <discovery-key> element containing the parameter defined in clause 11.4.2.9;

5)	a <discovery-entry-ID> element containing the parameter defined in clause 11.4.2.26;

6)	zero or one <ACE-enabled-indicator> element containing the parameter defined in clause 11.4.2.31;

7)	 zero or one <anyExt> element containing elements defined in future releases;

8)	zero, one or more elements from other namespaces defined in future releases; and

9)	zero, one or more attributes defined in future releases;

d)	zero, one or more <response-monitor> element which contains transactions sent from the 5G DDNMF to the UE as a response to a monitoring request if the 5G DDNMF accepts the request. Each <response-monitor> consists of:

1)	a <transaction-ID> element containing the parameter defined in clause 11.4.2.1;

2)	zero, one or more <discovery-filter> elements containing the parameter defined in clause 11.4.2.9;

3)	a <discovery-entry-ID> element containing the parameter defined in clause 11.4.2.26;

4)	zero or one <ACE-enabled-indicator> element containing the parameter defined in clause 11.4.2.31;

5)	zero or one <anyExt> element containing elements defined in future releases;

6)	zero, one or more elements from other namespaces defined in future releases; and

7)	zero, one or more attributes defined in future releases;

e)	zero, one or more <restricted-announce-response> element which contains transactions sent from the 5G DDNMF to the UE as a response to an announcing request for restricted ProSe direct discovery model A if the 5G DDNMF accepts the request. Each <restricted-announce-response> consists of:

1)	a <transaction-ID> element containing the parameter defined in clause 11.4.2.1;

2)	zero or one <ProSe-Restricted-Code> element containing the parameter defined in clause 11.4.2.27;

3)	zero, one or more <ProSe-Restricted-Code-Suffix-Range> element containing the parameter defined in clause 11.4.2.28;

4)	zero or one <validity-timer-T5062> element containing the parameter defined in 11.4.2.32;

5)	zero or one <ACE-enabled-indicator> element containing the parameter defined in clause 11.4.2.31;

6)	zero or one <restricted-code-security-material> element containing the parameter defined in clause 11.4.2.33;

7)	zero or one <on-demand-announcing-enabled-indicator> element containing the parameter defined in clause 11.4.2.29;

8)	a <discovery-entry-id> element containing the parameter defined in clause 11.4.2.26;

9)	zero or one <PC5-security-policies> element containing the parameter defined in clause 11.4.2.BB;

10)	zero or one <anyExt> element containing elements defined in future releases;

11)	zero, one or more elements from other namespaces defined in future releases; and

12)	zero, one or more attributes defined in future releases;

f)	zero, one or more <restricted-monitor-response> element which contains transactions sent from the 5G DDNMF to the UE as a response to a monitoring request for restricted ProSe direct discovery model A if the 5G DDNMF accepts the request. Each <restricted-monitor-response> consists of:

1)	a <transaction-ID> element containing the parameter defined in clause 11.4.2.1;

2)	zero, one or more <restricted-discovery-filter> elements containing the parameter defined in clause 11.4.2.30;

3)	zero or one <ACE-enabled-indicator> element containing the parameter defined in clause 11.4.2.31

4)	a <discovery-entry-id> element containing the parameter defined in clause 11.4.2.26;

5)	an <application-level-container> element containing the parameter defined in clause 11.4.2.25;

6)	zero or one <PC5-security-policies> element containing the parameter defined in clause 11.4.2.BB;

7)	zero or one <anyExt> element containing elements defined in future releases;

8)	zero, one or more elements from other namespaces defined in future releases; and

9)	zero, one or more attributes defined in future releases;

g)	zero, one or more <restricted-discoveree-response> element which contains transactions sent from the 5G DDNMF to the UE as a response to a discoveree UE's request for restricted ProSe direct discovery model B if the 5G DDNMF accepts the request. Each <restricted-discoveree-response> consists of:

1)	a <transaction-ID> element containing the parameter defined in clause 11.4.2.1;

2)	a <ProSe-Response-Code> element containing the element defined in clause11.4.2.35;

3)	one or more <query-filter> elements containing the parameter defined in clause 11.4.2.36;

4)	a <validity-timer-T5068> element containing the parameter defined in clause 11.4.2.37;

5)	zero or one <restricted-code-security-material> element containing the parameter defined in clause 11.4.2.33;

6)	a <discovery-entry-id> element containing the parameter defined in clause 11.4.2.26;

7)	zero or one <PC5-security-policies> element containing the parameter defined in clause 11.4.2.BB;

8)	zero or one <anyExt> element containing elements defined in future releases;

9)	zero, one or more elements from other namespaces defined in future releases; and

10)	zero, one or more attributes defined in future releases;

h)	zero, one or more <restricted-discoverer-response> element which contains transactions sent from the 5G DDNMF to the UE as a response to a discoverer UE's request for restricted ProSe direct discovery model B if the 5G DDNMF accepts the request. Each <restricted-discoverer-response> consists of:

1)	a <transaction-ID> element containing the parameter defined in clause 11.4.2.1;

2)	one or more <subquery-result> elements containing the parameter defined in clause 11.4.2.38;

3)	a <discovery-entry-id> element containing the parameter defined in clause 11.4.2.26;

4)	zero or one <PC5-security-policies> element containing the parameter defined in clause 11.4.2.BB;

5)	zero or one <anyExt> element containing elements defined in future releases;

6)	zero, one or more elements from other namespaces defined in future releases; and

7)	zero, one or more attributes defined in future releases;

i)	zero, one or more <response-reject> element which contains transactions sent from the 5G DDNMF to the UE as a response to an announcing or monitoring requests if the 5G DDNMF  cannot accept the request. Each <response-reject> consists of:

1)	a <transaction-ID> element containing the parameter defined in clause 11.4.2.1;

2)	a <PC3a-control-protocol-cause-value> element containing the parameter defined in clause 11.4.2.8.

3)	zero, one or more elements defined in future releases; and

4)	zero, one or more attributes defined in future releases;

j)	 zero or one <anyExt> element containing elements defined in future releases;

k)	zero, one or more elements from other namespaces defined in future releases;

l)	an optional "network-initiated transaction method" attribute containing the parameter defined in clause 11.4.2.42; and

m)	zero, one or more attributes defined in future releases.

* * * Next Change * * * *

#### 11.4.2.BB	PC5 security policies

This parameter is used to indicate the PC5 security policies as integer values, where the PC5 signalling security policies are defined in table 11.3.12.1 and the PC5 user plane security policies are defined in table 11.3.23.1.

* * * End of Changes * * * *