3GPP TSG-CT WG1 Meeting #133e-bis	C1-220559

- E-meeting, 17-21 January 2022

Title:	LS on Deletion of "ME support of SOR-CMCI" indicator during Nudm_SDM_Get

Response to:	LS in C4-216607/C1-220090

Release:	Rel-17

Work Item:	eCPSOR_CON

Source:	CT1

To:	CT4, SA2

Cc:

Contact person:	Ban Al-Bakri

ban.albakri@meadowcom.com

NTT DOCOMO

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	C1-220558

# 1	Overall description

CT1 thanks CT4 for the LS in C4-216607/C1-220090 on Deletion of "ME support of SOR-CMCI" indicator during Nudm_SDM_Get. CT1 provides the following answer to Q1:

Question 1: CT4 would like to highlight that Nudm_SDM_Get is a read-only operation on UDM, and should not cause changes to server state. Hence, CT4 would like to request CT1 to explore if the deletion of "ME support of SOR-CMCI" indicator during initial/emergency registrations can be performed via some alternate procedures (e.g. before/after Nudm_SDM_Get) instead, or an alternative that does not require deletion of “ME support of CMCI” indicator upon specific registration types by the UE.

CT1 answer to Q1:

It should be possible for the AMF to provide the registration type to the UDM using Nudm_UECM_Registration service operation when a registration request is performed by the UE. As a consequence, the UDM updates the UDR using the Nudr_DR_Update service operation. In the case of initial registration and emergency registration, the UDR shall delete the stored "ME support of SOR-CMCI" indicator, if any.

Therefore, it is required that the AMF provides the UDM with the registration type for every "initial registration " and "emergency registration". CT1 has agreed C1-220558 that implements this solution.

CT1 notices that the current TS23.502 and TS 29.503 allow only to indicate "initial registration" over the Nudm_UECM_Registration service operation, where there is a need to indicate "emergency registration" as well.

Therefore CT1 kindly asks SA2 and CT4 to update the spec to allow the "emergency registration" to be provided over the Nudm_UECM_Registration service operation, and make it mandatory for the AMF to provide the UDM with the "initial registration" and the "emergency registration".

CT1 kindly asks SA2 and CT4 to confirm if the solution agreed in C1-220558 is acceptable.

# 2	Actions

To SA2 and CT4

ACTION:

CT1 kindly asks SA2 and CT4 to confirm if the solution agreed in C1-220558 is acceptable.

CT1 kindly asks SA2 and CT4 to update the spec to allow the "emergency registration" to be provided over the Nudm_UECM_Registration service operation, and make it mandatory for the AMF to provide the UDM with the "initial registration" and the "emergency registration".

# 3	Dates of next TSG CT WG1 meetings

CT1#134e	17th-25th February 2022	electronic meeting