- 3GPP TSG-CT WG1 Meeting #133e-bis	C1-220232

- E-meeting, 17-21 January 2022

Title:	LS on lists of 5GS forbidden tracking area for non-3GPP access

Response to:	-

Release:	Rel-17

Work Item:	TEI17_N3SLICE

Source:	CT1

To:	SA2

Cc:	-

Contact person:	ZHOU Xingyue (Joy)

zhou.xingyue@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

Two facts are identified in SA2 and CT1 respectively as below:

- In SA2, TEI17_N3SLICE introduces a new requirement on TAI configuration for non-3GPP access as excerpted below in clause 5.3.2.3 of TS 23.501:

"TAI(s) dedicated to Non-3GPP access may be defined in a PLMN and apply within this PLMN. Each N3IWF, TNGF, TWIF and W-AGF is locally configured with one TAI value. Each N3IWF, TNGF, TWIF and W-AGF may be configured with a different TAI value or with the same TAI value as other N3IWFs, TNGFs, TWIFs and/or W-AGFs. The TAI is provided to the AMF during N2 interface setup as described in TS 38.413 [34]."

In addition, a NOTE as excerpted below in clause 5.3.4.1.1 of TS 23.501 clarifies:

"NOTE 1:	If a TAI (see clause 5.3.2.3) is configured in a N3IWF, TNGF, TWIF or W-AGF and this TAI is forbidden, access via that N3IWF, TNGF, TWIF or W-AGF is forbidden. If a single TAI is configured in all N3IWF, TNGF, TWIF and W-AGF and if this TAI is forbidden in a PLMN, non-3GPP Access is forbidden altogether in this PLMN."

- In CT1, for lists of 5GS forbidden tracking areas, there is a requirement for the UE that "Within the 5GS, these lists are managed independently per access type, i.e., 3GPP access or non-3GPP access." as specified in clause 5.3.13 of TS 24.501.

Based on the above, when each N3IWF, TNGF, TWIF and W-AGF is configured with different TAI values , and the UE needs to update the list of 5GS forbidden tracking areas if receiving the 5GMM cause #12 "tracking area not allowed" or #13 "roaming not allowed in this tracking area", it is not clear that how the UE gets known of the current TAI for non-3GPP access in order to add this TAI into the list of 5GS forbidden tracking areas.

Therefore, CT1 would like to ask SA2 when each TAI has different values how the UE gets known of the TAI of the N3IWF, TNGF, TWIF or W-AGF in the case mentioned above..

# 2	Actions

To SA2 group:

ACTION: 	CT1 kindly asks SA2 to provide the answer to the question above.

# 3	Dates of next TSG CT WG1 meetings

CT1#134e	17th-25th February 2022	electronic meeting