- 3GPP TSG-CT WG1 Meeting #133e-bis	C1-220232

- E-meeting, 17-21 January 2022

Title:	LS on lists of 5GS forbidden tracking area for non-3GPP access

Response to:	-

Release:	Rel-17

Work Item:	TEI17_N3SLICE

Source:	CT1

To:	SA2

Cc:	-

Contact person:	ZHOU Xingyue (Joy)

zhou.xingyue@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

Two facts are identified in SA2 and CT1 respectively as below:

- In SA2, TEI17_N3SLICE introduces a new requirement on TAI configuration for non-3GPP access as excerpted below in clause 5.3.2.3 of TS 23.501:

"TAI(s) dedicated to Non-3GPP access may be defined in a PLMN and apply within this PLMN. Each N3IWF, TNGF, TWIF and W-AGF is locally configured with one TAI value. Each N3IWF, TNGF, TWIF and W-AGF may be configured with a different TAI value or with the same TAI value as other N3IWFs, TNGFs, TWIFs and/or W-AGFs. The TAI is provided to the AMF during N2 interface setup as described in TS 38.413 [34]."

In addition, a NOTE as excerpted below in clause 5.3.4.1.1 of TS 23.501 clarifies:

"NOTE 1:	If a TAI (see clause 5.3.2.3) is configured in a N3IWF, TNGF, TWIF or W-AGF and this TAI is forbidden, access via that N3IWF, TNGF, TWIF or W-AGF is forbidden. If a single TAI is configured in all N3IWF, TNGF, TWIF and W-AGF and if this TAI is forbidden in a PLMN, non-3GPP Access is forbidden altogether in this PLMN."

- In CT1, for lists of 5GS forbidden tracking areas, there is a requirement for the UE that "Within the 5GS, these lists are managed independently per access type, i.e., 3GPP access or non-3GPP access." as specified in clause 5.3.13 of TS 24.501.

Based on the above, CT1 would like to ask SA2 following questions:

When each N3IWF, TNGF, TWIF and W-AGF is configured with different TAI values, and the UE needs to update the list of 5GS forbidden tracking areas if receiving the 5GMM cause #12 "tracking area not allowed" or #13 "roaming not allowed in this tracking area",

a)	how the UE gets to know the current TAI for non-3GPP access in order to add this TAI into the list of 5GS forbidden tracking areas; and

b)	whether the UE needs to keep track of multiple non-3GPP TAIs.

# 2	Actions

To SA2 group:

ACTION: 	CT1 kindly asks SA2 to provide the answer to the questions above.

# 3	Dates of next TSG CT WG1 meetings

CT1#134e	17th-25th February 2022	electronic meeting