3GPP TSG-CT WG1 Meeting #133-bis-e	C1-220585

- E-meeting, 17-21 January 2022

Title:	LS on MINT and Higher priority PLMN Selection

Response to:	- -

Release:	Release 17

Work Item:	MINT

Source:	CT1

To:	SA1

Cc:

Contact person:	Lalith Kumar

lalith.kumar@samsung.com

+918095301357

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	NA

# 1	Overall description

CT1 has discussed below case.

1) UE is registered on Forbidden PLMN(FPLMN) to receive disaster roaming service.

2) Higher priority scan timer (T) expires.

3) UE scans for available PLMNs

4) UE finds that there are PLMNs available, which are allowable for the UE i.e. those are not part of forbidden PLMNs list. But these PLMNs are not HPLMN and neither part of the EHPLMN list, "User Controlled PLMN Selector with Access Technology" data file(UPLMN list) in the SIM (in priority order)/ME; or "Operator Controlled PLMN Selector with Access Technology" data file(OPLMN list) in the SIM (in priority order)/ME configured by the network.

CT1 kindly requests SA1 to answer whether UE is allowed to attempt on such PLMNs to receive normal service or continue to use disaster roaming service on the FPLMN.

# 2	Actions

To SA2

ACTION:	CT1 kindly request SA1 to answer for discussed scenario in description:

a)	if UE is allowed to attempt and receive normal service on PLMNs which are not HPLMN/EHPLMN and neither configured in OPLMN/UPLMN list by the network; or

b)	Continue to remain registered with FPLMN to receive disaster roaming service.

# 3	Dates of next TSG CT WG1 meetings

CT1#133-bis-e	17th-21st January 2022	electronic meeting

CT1#134e	17th-25th February 2022	electronic meeting