3GPP TSG-CT WG1 Meeting #133-bis-e	C1-220585

- E-meeting, 17-21 January 2022

Title:	LS on MINT and Higher priority PLMN Selection

Response to:	- -

Release:	Release 17

Work Item:	MINT

Source:	CT1

To:	SA1

Cc:

Contact person:	Lalith Kumar

lalith.kumar@samsung.com

+918095301357

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	NA

# 1	Overall description

CT1 has discussed below cases.

Case 1:

The UE is registered on a Forbidden PLMN (FPLMN) to receive disaster roaming service. The UE is not in the home country and there is no PLMN of the same country as the current serving VPLMN stored in the User Controlled PLMN Selector with Access Technology" data file (UPLMN list) in the SIM/ME or the "Operator Controlled PLMN Selector with Access Technology" data file (OPLMN list) in the SIM/ME.

Question 1) Is the UE allowed or be mandated to scan for all available PLMNs in order to obtain service on an allowable PLMN?

Case 2:

1) 	The UE is registered on a Forbidden PLMN (FPLMN) to receive disaster roaming service. 
The UE is in the home country (i.e. HPLMN might be available) or there are PLMN of the same country as the current serving VPLMN stored in the User Controlled PLMN Selector with Access Technology" data file (UPLMN list) in the SIM/ME or the "Operator Controlled PLMN Selector with Access Technology" data file (OPLMN list) in the SIM/ME.

2) 	Higher priority scan timer (T) expires.

3)	As there are potential higher priority PLMNs available, the UE initiates the scan (current UE behaviour is assumed to also be applicable when registered on PLMN for disaster roaming).

4) 	The UE might find allowable PLMNs which are neither the HPLMN nor PLMNs of the same country which are configured in the OPLMN or UPLMN lists.

Question 2) Is the UE allowed or even mandated to attempt service on such other allowable PLMNs or shall the UE continue to use disaster roaming service on the FPLMN?

Question 3) The interval of time between searches for a higher priority PLMN is configured by HPLMN in the. Can SA1 kindly provide the reasoning whether or not this interval of time needs be different/adjusted for the UE in disaster roaming condition?

CT1 kindly requests SA1 to answer the above questions.

NOTE : “allowable PLMNs” above are the PLMNs which are not part of the list of forbidden PLMNs in the UE.

# 2	Actions

To SA2

ACTION:	CT1 kindly request SA1 to answer the questions in the description section.

# 3	Dates of next TSG CT WG1 meetings

CT1#134e	17th-25th February 2022	electronic meeting