3GPP TSG-CT WG1 Meeting #133-bis-e	C1-220585

- E-meeting, 17-21 January 2022

Title:	LS on MINT and Higher priority PLMN Selection

Response to:	- -

Release:	Release 17

Work Item:	MINT

Source:	CT1

To:	SA1

Cc:

Contact person:	Lalith Kumar

lalith.kumar@samsung.com

+918095301357

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	NA

# 1	Overall description

CT1 has discussed below cases.

Case 1:

1)	The UE is registered on a Forbidden PLMN (FPLMN) to receive disaster roaming service. 
The UE is not in the home country (i.e. HPLMN is not available) and there are no PLMN of the same country as the current serving VPLMN stored in the User Controlled PLMN Selector with Access Technology" data file (UPLMN list) in the SIM/ME or the "Operator Controlled PLMN Selector with Access Technology" data file (OPLMN list) in the SIM/ME.

2) 	Higher priority scan timer (T) expires.

Question 1) Shall the UE be mandated to scan for all available PLMNs in order to obtain service on an allowable PLMN?

Please note that this new requirement will cause additional power consumption and is not required to detect that the PLMN under disaster has recovered, as the recovery will be indicated by the PLMN providing disaster roaming. Thus it would only be justified if there is a justification that the UE prioritize an allowable non-preferred PLMN over a PLMN providing disaster roaming.

Question 2) Shall such attempts to obtain service on a allowable (i.e. not on a PLMN offering disaster roaming) but non-preferred PLMN be done with the same, higher or lower periodicity than the timer which is currently defined and configured on USIM for attempts to obtain service of higher priority PLMNs?

Case 2:

1) 	The UE is registered on a Forbidden PLMN (FPLMN) to receive disaster roaming service. 
The UE is the home country (i.e. HPLMN might be available) or there are PLMN of the same country as the current serving VPLMN stored in the User Controlled PLMN Selector with Access Technology" data file (UPLMN list) in the SIM/ME or the "Operator Controlled PLMN Selector with Access Technology" data file (OPLMN list) in the SIM/ME.

2) 	Higher priority scan timer (T) expires.

3)	As there are potential higher priority PLMNs available, the UE initiates the scan (current UE behaviour is assumed to also be applicable when registered on PLMN for disaster roaming).

4) 	The UE might find allowable PLMNs which are neither the HPLMN nor PLMNs of the same country which are configured in the OPLMN or UPLMN lists.

Question 3) Is the UE allowed or even mandated to attempt service on such other allowable PLMNs or shall the UE continue to use disaster roaming service on the FPLMN?

CT1 kindly requests SA1 to answer the above questions.

NOTE : “allowable PLMNs” above are the PLMNs which are not part of the list of forbidden PLMNs in the UE.

# 2	Actions

To SA2

ACTION:	CT1 kindly request SA1 to answer the questions in the description section.

# 3	Dates of next TSG CT WG1 meetings

CT1#134e	17th-25th February 2022	electronic meeting