3GPP TSG-CT WG1 Meeting #133-bis-e	C1-220585

- E-meeting, 17-21 January 2022

Title:	LS on MINT and Higher priority PLMN Selection

Response to:	- -

Release:	Release 17

Work Item:	MINT

Source:	CT1

To:	SA1

Cc:

Contact person:	Lalith Kumar

lalith.kumar@samsung.com

+918095301357

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	NA

# 1	Overall description

CT1 has discussed below cases.

Case 1:

1) UE is registered on Forbidden PLMN(FPLMN) to receive disaster roaming service.

2) Higher priority scan timer (T) expires.

Question: Is UE allowed to scan for all available PLMNs in the area even if such PLMNs are not configured in "User Controlled PLMN Selector with Access Technology" data file(UPLMN list) in the SIM/ME; or "Operator Controlled PLMN Selector with Access Technology" data file(OPLMN list) in the SIM/ME.

Case 2:

1) A situation occurs that UE can only register to a FPLMN disaster roaming service.

2) Higher priority scan timer (T) expires.

3) UE is configured with at least one PLMN in OPLMN or UPLMN list of the same country. Thus UE initiates the scan (current UE behaviour).

4) UE may find another allowable PLMNs which are not configured in OPLMN or UPLMN list.

Question

Is UE allowed to make an access attempt on such other allowable PLMNs to receive normal service or continue to use disaster roaming service on the FPLMN? and if UE continues on FPLMN, then for how long?

CT1 kindly requests SA1 to answer above questions.

NOTE : “allowable PLMNs” above are the PLMNs which are not part of forbidden PLMN list in the UE.

# 2	Actions

To SA2

ACTION:	CT1 kindly request SA1 to answer the questions in the description section.

# 3	Dates of next TSG CT WG1 meetings

CT1#133-bis-e	17th-21st January 2022	electronic meeting

CT1#134e	17th-25th February 2022	electronic meeting