3GPP TSG-CT WG1 Meeting #133-bis-e	C1-22abcd

E-meeting, 17-21 January 2022	was C1-220434

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.282 | CR | 0286 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | functional alias as a target user for 1-1 SDS/FD request using media plane |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData3 |  |  |  |  |  | Date: |  |  | 2022-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per TS 23.282 subclauses 7.4.2.3, 7.4.2.4 & 7.4.2.5 “MCData Functional alias resolution response” operation can be invoked by the MCData server on receiving the request targeted for the functional alias. This results in to single target MCData ID returned to back to the requesting MCData user to further invoke the procedure using the resolved MCData ID. The MCData client and MCData server needs to be updated to address the newly introduced functionality in stage 2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | In 9.2.3.2.3 & 9.2.4.2.3, included the procedure to include the target functional alias with indication and handling of 300 Multiple Choice response from the server. In 9.2.3.4.4 & 9.2.4.4.4, Added the handling of incoming request with functional alias used as a target and send the response with selected MCData id after resolving the functional alias D.1.2, D.1.3: Schema and defination are updated for the call-to-functional-alias-ind element |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 3 spec TS 24.282 won’t be able to support the newly introduced functionality in stage 2, TS 23.282 (subclauses 7.4.2.3, 7.4.2.4 & 7.4.2.5). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.2.3, 9.2.3.4.4, 9.2.4.2.3, 9.2.4.4.4, 22.2.2.2.8 (NEW), 22.2.2.2.9(NEW), 22.2.2.3.7(NEW), 22.2.2.3.8(NEW), D.1.2, D.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

##### 9.2.3.2.3	MCData client originating procedures

If a group standalone SDS message is to be sent, the MCData client shall determine whether the group document contains a <list-service> element that contains a <preconfigured-group-use-only> element. If a <preconfigured-group-use-only> element exists and is set to the value "true", then the MCData client:

1)	should indicate to the MCData user that a group standalone SDS message is not allowed on the indicated group; and

2)	shall skip the remainder of this procedure.

The MCData client shall generate a SIP INVITE request in accordance with 3GPP TS 24.229 [5] with the clarifications given below.

The MCData client:

1)	shall include the g.3gpp.mcdata.sds media feature tag and the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [16];

2)	shall include an Accept-Contact header field containing the g.3gpp.mcdata.sds media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

3)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

4)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7] in the SIP INVITE request;

5)	should include the "timer" option tag in the Supported header field;

6)	should include the Session-Expires header field according to IETF RFC 4028 [38]. It is recommended that the "refresher" header field parameter is omitted. If included, the "refresher" header field parameter shall be set to "uac";

7)	if a one-to-one standalone SDS message is to be sent:

a)	shall insert in the SIP INVITE request a MIME resource-lists body with the MCData ID of the invited MCData user or the URI of the functional alias to be called, according to rules and procedures of IETF RFC 5366 [18];

NOTE 1:	The MCData client indicates whether an MCData ID or a functional alias is to be called as specified in step 7) b) below.

b)	shall contain an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with:

i)	the <request-type> element set to a value of "one-to-one-sds";

ii)	with the <call-to-functional-alias-ind> set to "true" if the MCData client is aware of active functional aliases and an active functional alias is to be called; and

iii)	if the MCData client is aware of active functional aliases and if an active functional alias is to be included in the SIP INVITE request, the <functional-alias-URI> element set to the URI of the used functional alias; and

NOTE 2:	The MCData client learns the functional aliases that are activated for an MCData ID from procedures specified in clause 22.2.1.3.

c)	if an end-to-end security context needs to be established and the security context does not exist or if the existing security context has expired, then:

i)	if necessary, shall instruct the key management client to request keying material from the key management server as described in 3GPP TS 33.180 [26];

ii)	shall use the keying material to generate a PCK as described in 3GPP TS 33.180 [26];

iii)	shall use the PCK to generate a PCK-ID with the four most significant bits set to "0001" to indicate that the purpose of the PCK is to protect one-to-one communications and with the remaining twenty eight bits being randomly generated as described in 3GPP TS 33.180 [26];

iv)	shall encrypt the PCK to a UID associated to the MCData client using the MCData ID of the invited user and a time related parameter as described in 3GPP TS 33.180 [26];

v)	shall generate a MIKEY-SAKKE I_MESSAGE using the encapsulated PCK and PCK-ID as specified in 3GPP TS 33.180 [26];

vi)	shall add the MCData ID of the originating MCData to the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [26]; and

vii)	shall sign the MIKEY-SAKKE I_MESSAGE using the originating MCData user's signing key provided in the keying material together with a time related parameter, and add this to the MIKEY-SAKKE payload, as described in 3GPP TS 33.180 [26];

8)	if a group standalone SDS message is to be sent:

a)	if the "/<x>/<x>/Common/MCData/AllowedSDS" leaf node present in the group document of the requested MCData group, configured on the group management client as specified in 3GPP TS 24.483 [42] is set to "false", shall reject the request to send SDS and not continue with the rest of the steps in this clause; and

b)	shall contain in an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with:

i)	the <request-type> element set to a value of "group-sds";

ii)	the <mcdata-request-uri> element set to the MCData group identity;

iii)	the <mcdata-client-id> element set to the MCData client ID of the originating MCData client; and

NOTE 3:	The MCData client does not include the MCData ID of the originating MCData user in the body, as this will be inserted into the body of the SIP INVITE request that is sent from the originating participating MCData function.

iv)	if the MCData client is aware of active functional aliases and if an active functional alias is to be included in the SIP INVITE request, may include the <functional-alias-URI> element set to the URI of the used functional alias;

9)	shall set the Request-URI of the SIP INVITE request to the public service identity identifying the participating MCData function serving the MCData user;

NOTE 4:	The MCData client is configured with public service identity identifying the participating MCData function serving the MCData user.

10)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [5];

11)	shall include an SDP offer according to 3GPP TS 24.229 [5] with the clarifications given in clause 9.2.3.2.1; and

12)	shall send the SIP INVITE request towards the MCData server according to 3GPP TS 24.229 [5].

On receipt of a SIP 2xx response to the SIP INVITE request, the MCData client:

1)	shall send a SIP ACK request as specified in 3GPP TS 24.229 [5];

2)	shall start the SIP Session timer according to rules and procedures of IETF RFC 4028 [38]; and

3)	shall interact with the media plane as specified in 3GPP TS 24.582 [15] clause 6.1.1.2.

Upon receiving a SIP 300 (Multiple Choices) response to the SIP INVITE request the MCData client shall use the MCData ID of MCData user contained in the <mcdata-request-uri> element of an application/vnd.3gpp.mcdata-info MIME body as the MCData ID of the invited MCData user and shall generate an initial SIP MCData request by following the UE originating session procedures specified in 3GPP TS 24.229 [5], with the clarifications given in this clause and with the following additional clarifications:

1)	shall insert in the SIP INVITE request a MIME resource-lists body with the MCData ID of the invited MCData user returned in the SIP 300 (Multiple Choices) response to the initial MCData INVITE request for establishing session for sending one-to-one standalone SDS message, according to rules and procedures of IETF RFC 5366 [18];

2)	shall not include a <call-to-functional-alias-ind> element into the <mcdata-Params> element of the <mcdatainfo> element of an application/vnd.3gpp.mcdata-info+xml MIME body; and

3)	shall include a <called-functional-alias-URI> element into the <mcdata-Params> element of the <mcdatainfo> element of an application/vnd.3gpp.mcdata-info+xml MIME body with the target functional alias URI used in the initial SIP INVITE request for establishing a session for sending one-to-one standalone SDS message.

On receipt of a SIP 4xx response, a SIP 5xx response or a SIP 6xx response to the SIP INVITE request:

1)	shall indicate to the MCData user that the SDS message could not be sent; and

2)	shall send a SIP ACK request as specified in 3GPP TS 24.229 [5].

On receipt of an indication from the media plane indicating that the standalone SDS message was not sent successfully, the MCData client shall:

1)	shall generate a SIP BYE request according to 3GPP TS 24.229 [5] with:

a)	Reason code set to "SIP";

b)	cause set to "480"; and

c)	text set to "transmission failed";

2)	shall set the Request-URI to the MCData session identity to release; and

3)	shall send a SIP BYE request towards MCData server according to 3GPP TS 24.229 [5].

On receipt of an indication from the media plane indicating that the standalone SDS message has been successfully transferred, the MCData client shall:

1)	shall generate a SIP BYE request according to 3GPP TS 24.229 [5] with:

a)	Reason code set to "SIP";

b)	cause set to "200"; and

c)	text set to "transmission succeeded";

2)	shall set the Request-URI to the MCData session identity to release; and

3)	shall send a SIP BYE request towards MCData server according to 3GPP TS 24.229 [5].

Upon receiving a SIP 200 (OK) response to the SIP BYE request, the MCData client shall interact with the media plane and indicate to terminate the session, as specified in 3GPP TS 24.582 [15].

* * * * * * * NEXT CHANGE * * * * * * *

##### 9.2.3.4.4	Terminating controlling MCData function procedures

In the procedures in this clause:

1)	MCData ID in an incoming SIP INVITE request refers to the MCData ID of the originating user from the <mcdata-calling-user-id> element of the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SIP INVITE request;

2)	group identity in an incoming SIP INVITE request refers to the group identity from the <mcdata-request-uri> element of the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SIP INVITE request; and

3)	MCData ID in an outgoing SIP INVITE request refers to the MCData ID of the called user in the <mcdata-request-uri> element of the application/vnd.3gpp.mcdata-info+xml MIME body of the outgoing SIP INVITE request;

Upon receipt of a "SIP INVITE request for controlling MCData function for standalone SDS over media plane", the controlling MCData function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP INVITE request with a SIP 500 (Server Internal Error) response. The controlling MCData function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [4] and skip the rest of the steps;

1A)	if the group document contains a <list-service> element that contains a <preconfigured-group-use-only> element that is set to the value "true", shall reject the SIP INVITE request with a SIP 403 (Forbidden) response with the warning text set to "167 call is not allowed on the preconfigured group" as specified in clause 4.9 "Warning header field" and shall skip the rest of this procedure;

2)	shall determine if the media parameters are acceptable and the MSRP URI is offered in the SDP offer and if not reject the request with a SIP 488 (Not Acceptable Here) response and skip the rest of the steps;

3)	shall reject the SIP request with a SIP 403 (Forbidden) response and not process the remaining steps if:

a)	an Accept-Contact header field does not include the g.3gpp.mcdata.sds media feature tag; or

b)	an Accept-Contact header field does not include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds";

4)	shall cache SIP feature tags, if received in the Contact header field and if the specific feature tags are supported;

5)	shall start the SIP Session timer according to rules and procedures of IETF RFC 4028 [38];

6)	if the <request-type> element in the application/vnd.3gpp.mcdata-info+xml MIME body of the SIP INVITE request is set to a value of "one-to-one-sds" and the SIP INVITE request:

a)	does not contain an application/resource-lists MIME body or contains an application/resource-lists MIME body with more than one <entry> element, shall return a SIP 403 (Forbidden) response with the warning text set to "204 unable to determine targeted user for one-to-one SDS" in a Warning header field as specified in clause 4.9, and skip the rest of the steps below; and

a1)	if the <mcdata-Params> element of the <mcdatainfo> element of an application/vnd.3gpp.mcdata-info+xml MIME body contains an <call-to-functional-alias-ind> element set to "true":

I)	shall identify the MCData ID(s) of the MCData user(s) that have activated the received called functional alias in the MIME resource-lists body of the SIP INVITE request by performing the actions specified in clause 22.2.2.2.8;

II)	if unable to determine the MCData ID(s) of the MCData user(s) that have activated the received called functional alias in the MIME resource-lists body of the SIP INVITE by performing the actions specified in clause 22.2.2.2.8, shall reject the SIP INVITE request with a SIP 403 (Forbidden) response including warning text set to "145 unable to determine called party" in a Warning header field as specified in clause 4.9, and shall not continue with the rest of the steps; and

III)	upon receipt of a SIP NOTIFY request with application/pidf+xml MIME body containing the MCData ID(s) of the MCData user(s) that have activated the called functional alias as specified in clause 22.2.2.3.8, shall return a SIP 300 (Multiple Choices) response to the SIP INVITE request populated according to 3GPP TS 24.229 [5], IETF RFC 3261 [24] with:

A)	a Contact header field containing a SIP URI for the MCData session identity; and

B)	an application/vnd.3gpp.mcdata-info MIME body with a <mcdata-request-uri> element set to the MCData ID of one MCData user from the listed MCData user(s) in the application/pidf+xml MIME body of the SIP NOTIFY request that have activated the identified called functional alias and shall not continue with the rest of the steps in this clause;

NOTE 1:	The controlling MCData function determines the appropriate MCData ID to be selected based on implementation-specific selection criteria when the functional alias used as a target of the one-to-one standalone SDS message request is simultaneously active for multiple MCData users.

b)	contains an application/resource-lists MIME body with exactly one <entry> element, shall invite the MCData user identified by the <entry> element of the MIME body, as specified in clause 9.2.3.4.3; and

c)	shall interact with the media plane as specified in 3GPP TS 24.582 [15] clause 6.3.1;

7)	if the <request-type> element in the application/vnd.3gpp.mcdata-info+xml MIME body of the SIP INVITE request is set to a value of "group-sds":

a)	shall retrieve the necessary group document(s) from the group management server for the group identity contained in the SIP INVITE request and carry out initial processing as specified in clause 6.3.3, and shall continue with the remaining steps if the procedures in clause 6.3.3 were successful;

b)	if the <on-network-disabled> element is present in the group document, shall send a SIP 403 (Forbidden) response with the warning text set to "115 group is disabled" in a Warning header field as specified in clause 4.9 and shall not continue with the rest of the steps;

c)	if the <entry> element of the <list> element of the <list-service> element in the group document does not contain an <mcdata-mcdata-id> element with a "uri" attribute matching the MCData ID of the originating user contained in the <mcdata-calling-user-id> element of the application/vnd.3gpp.mcdata-info+xml MIME body in the SIP INVITE request, shall send a SIP 403 (Forbidden) response with the warning text set to "116 user is not part of the MCData group" in a Warning header field as specified in clause 4.9 and shall not continue with the rest of the steps;

d)	if the <list-service> element contains a <mcdata-allow-short-data-service> element in the group document set to a value of "false", shall send a SIP 403 (Forbidden) response with the warning text set to "206 short data service not allowed for this group" in a Warning header field as specified in clause 4.x and shall not continue with the rest of the steps;

e)	if the <supported-services> element is not present in the group document or is present and contains a <service> element containing an "enabler" attribute which is not set to the value "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds", shall send a SIP 488 (Not Acceptable) response with the warning text set to "207 SDS services not supported for this group" in a Warning header field as specified in clause 4.9 and shall not continue with the rest of the steps;

f)	if the MCData server group SDS procedures in clause 11.1 indicate that the user identified by the MCData ID is not allowed to send group MCData communications on this group identity as determined by step 2) of clause 11.1, shall reject the SIP INVITE request with a SIP 403 (Forbidden) response, with warning text set to "201 user not authorised to transmit data on this group identity" in a Warning header field as specified in clause 4.9, and shall not continue with the rest of the steps in this clause;

g)	the originating user identified by the MCData ID is not affiliated to the group identity contained in the SIP INVITE request, as specified in clause 6.3.5, shall return a SIP 403 (Forbidden) response with the warning text set to "120 user is not affiliated to this group" in a Warning header field as specified in clause 4.9, and skip the rest of the steps below;

h)	shall determine targeted group members for MCData communications by following the procedures in clause 6.3.4;

i)	if the procedures in clause 6.3.4 result in no affiliated members found in the selected MCData group, shall return a SIP 403 (Forbidden) response with the warning text set to "198 no users are affiliated to this group" in a Warning header field as specified in clause 4.9, and skip the rest of the steps below; and

j)	shall invite each group member determined in step h) above, to the group session, as specified in clause 9.2.3.4.3; and

k)	shall interact with the media plane as specified in 3GPP TS 24.582 [15] clause 6.3.1.

Upon receiving a SIP 200 (OK) response for a SIP INVITE request as specified in clause 9.2.3.4.3 and if the MCData ID in the SIP 200 (OK) response matches to the MCData ID in the corresponding SIP INVITE request. the controlling MCData function:

1)	shall generate SIP 200 (OK) response to the SIP INVITE request according to 3GPP TS 24.229 [5];

2)	shall include the option tag "timer" in a Require header field;

3)	shall include the Session-Expires header field and start supervising the SIP session according to rules and procedures of IETF RFC 4028 [38], "UAS Behavior". The "refresher" parameter in the Session-Expires header field shall be set to "uac";

4)	shall include a P-Asserted-Identity header field with the public service identity of the controlling MCData function;

5)	shall include a SIP URI for the MCData session identity in the Contact header field identifying the MCData session at the controlling MCData function;

6)	shall include the following in the Contact header field:

a)	the g.3gpp.mcdata.sds media feature tag;

b)	the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds"; and

c)	the isfocus media feature tag;

7)	shall include Warning header field(s) received in incoming responses to the SIP INVITE request;

8)	shall include in the SIP 200 (OK) response an SDP answer to the SDP offer in the incoming SIP INVITE request as specified in the clause 9.2.3.4.2;

9)	shall interact with the media plane as specified in 3GPP TS 24.582 [15] clause 6.3.1; and

10)	shall send a SIP 200 (OK) response to the inviting MCData client according to 3GPP TS 24.229 [5].

* * * * * * * NEXT CHANGE * * * * * * *

##### 9.2.4.2.3	MCData client originating procedures

The MCData client shall generate a SIP INVITE request in accordance with 3GPP TS 24.229 [5] with the clarifications given below.

The MCData client:

1)	shall include the g.3gpp.mcdata.sds media feature tag and the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [16];

2)	shall include an Accept-Contact header field containing the g.3gpp.mcdata.sds media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

3)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

4)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7] in the SIP INVITE request;

5)	should include the "timer" option tag in the Supported header field;

6)	should include the Session-Expires header field according to IETF RFC 4028 [38]. It is recommended that the "refresher" header field parameter is omitted. If included, the "refresher" header field parameter shall be set to "uac";

7)	if a one-to-one SDS session is requested:

a0)	if the MCData user has requested the origination of an MCData emergency one-to-one communication or is originating an MCData one-to-one communication and the MCData emergency state is already set, then:

i)	if this is an authorised request for an MCData emergency one-to-one communication as determined by the procedures of clause 6.2.8.3.1.1, shall comply with the procedures in clause 6.2.8.3.2; or

ii)	if this is an unauthorised request for an MCData emergency one-to-one communication as determined in step i) above, should indicate to the MCData user that initiation of an MCData emergency one-to-one communication is not authorized and shall release the generated SIP INVITE request and end the procedure;

a)	shall insert in the SIP INVITE request a MIME resource-lists body with the MCData ID of the invited MCData user or the URI of the functional alias to be called, according to rules and procedures of IETF RFC 5366 [18];

NOTE 0:	The MCData client indicates whether an MCData ID or a functional alias is to be called as specified in step 7) b) below.

b)	shall contain an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with:

i)	the <request-type> element set to a value of "one-to-one-sds-session";

ii)	with the <call-to-functional-alias-ind> set to "true" if the MCData client is aware of active functional aliases and an active functional alias is to be called; and

iii)	if the MCData client is aware of active functional aliases and if an active functional alias is to be included in the SIP INVITE request, the <functional-alias-URI> element set to the URI of the used functional alias;

NOTE 1:	The MCData client learns the functional aliases that are activated for an MCData ID from procedures specified in clause 22.2.1.3.

c)	if an end-to-end security context needs to be established and the security context does not exist or if the existing security context has expired, then:

i)	if necessary, shall instruct the key management client to request keying material from the key management server as described in 3GPP TS 33.180 [26];

ii)	shall use the keying material to generate a PCK as described in 3GPP TS 33.180 [26];

iii)	shall use the PCK to generate a PCK-ID with the four most significant bits set to "0001" to indicate that the purpose of the PCK is to protect one-to-one communications and with the remaining twenty eight bits being randomly generated as described in 3GPP TS 33.180 [26];

iv)	shall encrypt the PCK to a UID associated to the MCData client using the MCData ID of the invited user and a time related parameter as described in 3GPP TS 33.180 [26];

v)	shall generate a MIKEY-SAKKE I_MESSAGE using the encapsulated PCK and PCK-ID as specified in 3GPP TS 33.180 [26];

vi)	shall add the MCData ID of the originating MCData user to the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [26]; and

vii)	shall sign the MIKEY-SAKKE I_MESSAGE using the originating MCData user's signing key provided in the keying material together with a time related parameter, and add this to the MIKEY-SAKKE payload, as described in 3GPP TS 33.180 [26]; and

d)	if the MCData emergency private communication state is set to either "MDEPC 2: emergency-pc-requested" or "MDEPC 3: emergency-pc-granted" or if the MCData emergency private priority state of this one-to-one communication is set to a value other than "MDEPP 2: in-progress" or "MDEPP 3: confirm-pending", shall execute the procedures in clause 6.2.8.3.3 to include the Resource-Priority header field;

8)	if a group SDS session is requested:

a)	if the "/<x>/<x>/Common/MCData/AllowedSDS" leaf node present in the group document of the requested MCData group, configured on the group management client as specified in 3GPP TS 24.483 [42] is set to "false", shall reject the request to send SDS and not continue with the rest of the steps in this clause;

a1)	if the group document contains a <list-service> element that contains a <preconfigured-group-use-only> element that is set to the value "true":

i)	should notify the MCData user that an SDS session is not allowed on this preconfigured group; and

ii)	shall skip the rest of this procedure;

b)	shall contain in an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with:

i)	the <request-type> element set to a value of "group-sds-session";

ii)	the <mcdata-request-uri> element set to the MCData group identity; and

iii)	the <mcdata-client-id> element set to the MCData client ID of the originating MCData client;

NOTE 2:	The MCData client does not include the MCData ID of the originating MCData user in the body, as this will be inserted into the body of the SIP INVITE request that is sent from the originating participating MCData function.

iv)	if the MCData client is aware of active functional aliases, and an active functional alias is to be included in the SIP INVITE request, the <functional-alias-URI> set to the URI of the used functional alias;

c)	if the MCData user has requested the origination of an MCData emergency group communication or is originating an MCData pre-arranged group communication and the MCData emergency state is already set, the MCData client shall execute the procedures in clause 6.2.8.1.1;

d)	if the MCData user has requested the origination of an MCData imminent peril group communication, the MCData client shall execute the procedures in clause 6.2.8.1.9;

e)	if the MCData client emergency group state for this group is set to "MDEG 2: in-progress" or "MDEG 4: confirm-pending", the MCData client shall execute the procedures in clause 6.2.8.1.2 to include the Resource-Priority header field; and

f)	if the MCData client imminent peril group state for this group is set to "MDIG 2: in-progress" or "MDIG 4: confirm-pending", shall execute the procedures in clause 6.2.8.1.12 to include the Resource-Priority header field;

9)	shall set the Request-URI of the SIP INVITE request to the public service identity identifying the participating MCData function serving the MCData user;

NOTE 3:	The MCData client is configured with public service identity identifying the participating MCData function serving the MCData user.

10)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [5];

11)	shall include an SDP offer according to 3GPP TS 24.229 [5] with the clarifications given in clause 9.2.4.2.1; and

12)	shall send the SIP INVITE request towards the MCData server according to 3GPP TS 24.229 [5].

Upon receiving a SIP 183 (Session Progress) response to the SIP INVITE request, the MCData client:

1)	may indicate the progress of the session establishment to the inviting MCData user.

On receipt of a SIP 2xx response to the SIP INVITE request, the MCData client:

0)	if the response is to a SIP INVITE request for an MCData emergency group communication or if an MCData imminent peril group communication shall perform the actions specified in clause 6.2.8.1.4;

1)	if the response is to a SIP INVITE request for an MCData emergency one-to-one communication, shall perform the actions specified in clause 6.2.8.3.4;

2)	shall send a SIP ACK request as specified in 3GPP TS 24.229 [5];

3)	shall start the SIP Session timer according to rules and procedures of IETF RFC 4028 [38]; and

4)	shall interact with the media plane as specified in 3GPP TS 24.582 [15] clause 6.1.2.2.

Upon receiving a SIP 300 (Multiple Choices) response to the SIP INVITE request the MCData client shall use the MCData ID of MCData user contained in the <mcdata-request-uri> element of an application/vnd.3gpp.mcdata-info MIME body as the MCData ID of the invited MCData user and shall generate an initial SIP MCData request by following the UE originating session procedures specified in 3GPP TS 24.229 [5], with the clarifications given in this clause and with the following additional clarifications:

1)	shall insert in the SIP INVITE request a MIME resource-lists body with the MCData ID of the invited MCData user returned in the SIP 300 (Multiple Choices) response to the initial MCData INVITE request for establishing session for sending one-to-one standalone SDS message, according to rules and procedures of IETF RFC 5366 [18];

2)	shall not include a <call-to-functional-alias-ind> element into the <mcdata-Params> element of the <mcdatainfo> element of an application/vnd.3gpp.mcdata-info+xml MIME body; and

3)	shall include a <called-functional-alias-URI> element into the <mcdata-Params> element of the <mcdatainfo> element of an application/vnd.3gpp.mcdata-info+xml MIME body with the target functional alias URI used in the initial SIP INVITE request for establishing a session for sending one-to-one standalone SDS message.

On receipt of a SIP 4xx response, a SIP 5xx response or a SIP 6xx response to the SIP INVITE request, the MCData client:

0)	if the response is to a SIP INVITE request for an MCData emergency group communication or an MCData imminent peril group communication, shall perform the actions specified in clause 6.2.8.1.5;

1)	if the response is to a SIP INVITE request for an MCData emergency one-to-one communication, shall perform the actions specified in clause 6.2.8.3.5;

2)	shall indicate to the MCData user that the SDS message could not be sent; and

3)	shall send a SIP ACK request as specified in 3GPP TS 24.229 [5].

On receipt of a SIP INFO request where the Request-URI contains an MCData session ID identifying an ongoing group session, the MCData client shall follow the actions specified in clause 6.2.8.1.13.

On receipt of a SIP INFO request where the Request-URI contains an MCData session ID identifying an ongoing one-to-one session, the MCData client shall follow the actions specified in clause 6.2.8.3.7.

On receipt of an indication from the media plane indicating that the SDS message was not sent successfully, the MCData client:

1)	shall generate a SIP BYE request according to 3GPP TS 24.229 [5] with:

a)	Reason code set to "SIP";

b)	cause set to "480"; and

c)	text set to "transmission failed";

2)	shall set the Request-URI to the MCData session identity to release; and

3)	shall send a SIP BYE request towards MCData server according to 3GPP TS 24.229 [5].

* * * * * * * NEXT CHANGE * * * * * * *

##### 9.2.4.4.4	Terminating controlling MCData function procedures

In the procedures in this clause:

1)	MCData ID in an incoming SIP INVITE request refers to the MCData ID of the originating user from the <mcdata-calling-user-id> element of the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SIP INVITE request;

2)	group identity in an incoming SIP INVITE request refers to the group identity from the <mcdata-request-uri> element of the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SIP INVITE request; and

3)	MCData ID in an outgoing SIP INVITE request refers to the MCData ID of the called user in the <mcdata-request-uri> element of the application/vnd.3gpp.mcdata-info+xml MIME body of the outgoing SIP INVITE request;

Upon receipt of a "SIP INVITE request for controlling MCData function for SDS session", the controlling MCData function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP INVITE request with a SIP 500 (Server Internal Error) response. The controlling MCData function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [4] and skip the rest of the steps;

NOTE:	If the SIP INVITE request contains an emergency indication or an imminent peril indication set to a value of "true" and this is an authorised request originating an MCData emergency group communication as determined by clause 6.3.7.2.6, or for originating an MCData imminent peril group communication as determined by clause 6.3.7.2.4, the controlling MCData function can, according to local policy, choose to accept the request.

2)	shall determine if the media parameters are acceptable and the MSRP URI is offered in the SDP offer and if not reject the request with a SIP 488 (Not Acceptable Here) response and skip the rest of the steps;

3)	shall reject the SIP request with a SIP 403 (Forbidden) response and not process the remaining steps if:

a)	an Accept-Contact header field does not include the g.3gpp.mcdata.sds media feature tag; or

b)	an Accept-Contact header field does not include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds";

3A)	if the received SIP INVITE request includes an application/vnd.3gpp.mcdata-info+xml MIME body with an <emergency-ind> element included or an <imminentperil-ind> element included, shall validate the request as described in clause 6.3.7.1.9;

3B)	if the SIP INVITE request contains an unauthorised request for an MCData emergency communication as determined by clause 6.3.7.2.6:

a)	shall reject the SIP INVITE request with a SIP 403 (Forbidden) response as specified in clause 6.3.7.2.7; and

b)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [5] and skip the rest of the steps;

3C)	if the SIP INVITE request contains an unauthorised request for an MCData imminent peril group communication as determined by clause 6.3.7.2.4, shall reject the SIP INVITE request with a SIP 403 (Forbidden) response with the following clarifications:

a)	shall include in the SIP 403 (Forbidden) response an application/vnd.3gpp.mcdata-info+xml MIME body as specified in clause D.1 with the <mcdatainfo> element containing the <mcdata-Params> element with the <imminentperil-ind> element set to a value of "false"; and

b)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [5] and skip the rest of the steps;

3D)	if a Resource-Priority header field is included in the SIP INVITE request:

a)	if the Resource-Priority header field is set to the value indicated for emergency communications and the SIP INVITE request does not contain an emergency indication and the in-progress emergency state of the group is set to a value of "false", shall reject the SIP INVITE request with a SIP 403 (Forbidden) response and skip the rest of the steps; or

b)	if the Resource-Priority header field is set to the value indicated for imminent peril communications and the SIP INVITE request does not contain an imminent peril indication and the in-progress imminent peril state of the group is set to a value of "false", shall reject the SIP INVITE request with a SIP 403 (Forbidden) response and skip the rest of the steps;

4)	shall cache SIP feature tags, if received in the Contact header field and if the specific feature tags are supported;

5)	void;

6)	shall start the SIP Session timer according to rules and procedures of IETF RFC 4028 [38];

7)	if the <request-type> element in the application/vnd.3gpp.mcdata-info+xml MIME body of the SIP INVITE request is set to a value of "one-to-one-sds-session" and the SIP INVITE request:

a)	does not contain an application/resource-lists MIME body or contains an application/resource-lists MIME body with more than one <entry> element, shall return a SIP 403 (Forbidden) response with the warning text set to "204 unable to determine targeted user for one-to-one SDS" in a Warning header field as specified in clause 4.9, and skip the rest of the steps below;

a1)	if the <mcdata-Params> element of the <mcdatainfo> element of an application/vnd.3gpp.mcdata-info+xml MIME body contains an <call-to-functional-alias-ind> element set to "true":

I)	shall identify the MCData ID(s) of the MCData user(s) that have activated the received called functional alias in the MIME resource-lists body of the SIP INVITE request by performing the actions specified in clause 22.2.2.2.8;

II)	if unable to determine the MCData ID(s) of the MCData user(s) that have activated the received called functional alias in the MIME resource-lists body of the SIP INVITE by performing the actions specified in clause 22.2.2.2.8, shall reject the SIP INVITE request with a SIP 403 (Forbidden) response including warning text set to "145 unable to determine called party" in a Warning header field as specified in clause 4.9, and shall not continue with the rest of the steps; and

III)	upon receipt of a SIP NOTIFY request with application/pidf+xml MIME body containing the MCData ID(s) of the MCData user(s) that have activated the called functional alias as specified in clause 22.2.2.3.8, shall return a SIP 300 (Multiple Choices) response to the SIP INVITE request populated according to 3GPP TS 24.229 [5], IETF RFC 3261 [24] with:

A)	a Contact header field containing a SIP URI for the MCData session identity; and

B)	an application/vnd.3gpp.mcdata-info MIME body with a <mcdata-request-uri> element set to the MCData ID of one MCData user from the listed MCData user(s) in the application/pidf+xml MIME body of the SIP NOTIFY request that have activated the identified called functional alias and shall not continue with the rest of the steps in this clause;

NOTE 1:	The controlling MCData function determines the appropriate MCData ID to be selected based on implementation-specific selection criteria when the functional alias used as a target of the one-to-one standalone SDS message request is simultaneously active for multiple MCData users.

b)	contains an application/resource-lists MIME body with exactly one <entry> element, shall invite the MCData user identified by the <entry> element of the MIME body, as specified in clause 9.2.4.4.3; and

c)	shall interact with the media plane as specified in 3GPP TS 24.582 [15] clause 6.3.2;

8)	if the <request-type> element in the application/vnd.3gpp.mcdata-info+xml MIME body of the SIP INVITE request is set to a value of "group-sds-session":

a)	shall retrieve the necessary group document(s) from the group management server for the group identity contained in the SIP INVITE request and carry out initial processing as specified in clause 6.3.3, and shall continue with the remaining steps if the procedures in clause 6.3.3 were successful;

b)	if the <on-network-disabled> element is present in the group document, shall send a SIP 403 (Forbidden) response with the warning text set to "115 group is disabled" in a Warning header field as specified in clause 4.9 and shall not continue with the rest of the steps;

b1)	if the group document contains a <list-service> element that contains a <preconfigured-group-use-only> element that is set to the value "true", shall reject the SIP INVITE request with a SIP 403 (Forbidden) response with the warning text set to "167 call is not allowed on the preconfigured group" as specified in clause 4.9 "Warning header field" and shall skip the rest of this procedure;

c)	if the <entry> element of the <list> element of the <list-service> element in the group document does not contain an <mcdata-mcdata-id> element with a "uri" attribute matching the MCData ID of the originating user contained in the <mcdata-calling-user-id> element of the application/vnd.3gpp.mcdata-info+xml MIME body in the SIP INVITE request, shall send a SIP 403 (Forbidden) response with the warning text set to "116 user is not part of the MCData group" in a Warning header field as specified in clause 4.9 and shall not continue with the rest of the steps;

d)	if the <list-service> element contains a <mcdata-allow-short-data-service> element in the group document set to a value of "false", shall send a SIP 403 (Forbidden) response with the warning text set to "206 short data service not allowed for this group" in a Warning header field as specified in clause 4.9 and shall not continue with the rest of the steps;

e)	if the <supported-services> element is not present in the group document or is present and contains a <service> element containing an "enabler" attribute which is not set to the value "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds", shall send a SIP 488 (Not Acceptable) response with the warning text set to "207 SDS services not supported for this group" in a Warning header field as specified in clause 4.9 and shall not continue with the rest of the steps;

f)	if the MCData server group SDS procedures in clause 11.1 indicate that the user identified by the MCData ID is not allowed to send group MCData communications on this group identity as determined by step 2) of clause 11.1, shall reject the SIP INVITE request with a SIP 403 (Forbidden) response, with warning text set to "222 user not authorised to initiate group SDS session on this group identity" in a Warning header field as specified in clause 4.9, and shall not continue with the rest of the steps in this clause;

g)	if the originating user identified by the MCData ID is not affiliated to the group identity contained in the SIP INVITE request, as specified in clause 6.3.5, shall return a SIP 403 (Forbidden) response with the warning text set to "120 user is not affiliated to this group" in a Warning header field as specified in clause 4.9, and skip the rest of the steps below;

h)	shall determine targeted group members for MCData communications by following the procedures in clause 6.3.4;

i)	if the procedures in clause 6.3.4 result in no affiliated members found in the selected MCData group, shall return a SIP 403 (Forbidden) response with the warning text set to "198 no users are affiliated to this group" in a Warning header field as specified in clause 4.9, and skip the rest of the steps below; and

j)	shall invite each group member determined in step g) above, to the group session, as specified in clause 9.2.4.4.3; and

k)	shall interact with the media plane as specified in 3GPP TS 24.582 [15] clause 6.3.2.

Upon receiving a SIP 200 (OK) response for a SIP INVITE request as specified in clause 9.2.4.4.3 and if the MCData ID in the SIP 200 (OK) response matches to the MCData ID in the corresponding SIP INVITE request, the controlling MCData function:

1)	shall generate SIP 200 (OK) response to the SIP INVITE request according to 3GPP TS 24.229 [5];

2)	shall include the option tag "timer" in a Require header field;

3)	shall include the Session-Expires header field and start supervising the SIP session according to rules and procedures of IETF RFC 4028 [38], "UAS Behavior". The "refresher" parameter in the Session-Expires header field shall be set to "uac";

4)	shall include a P-Asserted-Identity header field with the public service identity of the controlling MCData function;

5)	shall include a SIP URI for the MCData session identity in the Contact header field identifying the MCData session at the controlling MCData function;

6) shall include the following in the Contact header field:

a)	the g.3gpp.mcdata.sds media feature tag;

b)	the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds"; and

c)	the isfocus media feature tag;

7)	shall include Warning header field(s) received in incoming responses to the SIP INVITE request;

8)	shall include in the SIP 200 (OK) response an SDP answer to the SDP offer in the incoming SIP INVITE request as specified in the clause 9.2.4.4.2;

8A)	if the SIP INVITE request contains an alert indication set to a value of "true" and this is an unauthorised request for an MCData emergency alert as specified in clause 6.3.7.2.1, shall include in the SIP 200 (OK) response the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in clause 4.9;

8B)	if the received SIP INVITE request contains an application/vnd.3gpp.mcdata-info+xml MIME body with the <imminentperil-ind> element set to a value of "true" and if the in-progress emergency state of the group is set to a value of "true", shall include in the SIP 200 (OK) response the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in clause 4.9;

9)	shall interact with the media plane as specified in 3GPP TS 24.582 [15] clause 6.3.2; and

10)	shall send a SIP 200 (OK) response to the inviting MCData client according to 3GPP TS 24.229 [5].

* * * * * * * NEXT CHANGE * * * * * * *

##### 22.2.2.2.8	Functional alias resolution from MCData server owning the functional alias procedure

In order to discover the MCData users that have successfully activated a handled functional alias in the MCData server owning the functional alias, the MCData server shall generate an initial SIP SUBSCRIBE request according to 3GPP TS 24.229 [5], IETF RFC 3856 [39], and IETF RFC 6665 [36].

In the SIP SUBSCRIBE request, the MCData server:

1)	shall set the Request-URI to the public service identity of the controlling MCData function associated with the handled functional alias;

2)	shall include an application/vnd.3gpp.mcdata-info+xml MIME body. In the application/vnd.3gpp.mcdata -info+xml MIME body, the MCData server shall include the <mcdata-request-uri> element set to the handled functional alias ID;

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata" (coded as specified in 3GPP TS 24.229 [5]), in a P-Asserted-Service header field according to IETF RFC 6050 [7];

4)	shall set the Expires header field according to IETF RFC 6665 [36] to zero;

NOTE:	if the MCData server wants to receive the current status and later notification, can set the Expires header field according to IETF RFC 6665 [36], to 4294967295, which is the highest value defined for Expires header field in IETF RFC 3261 [24].

5)	shall include an Accept header field containing the application/pidf+xml MIME type;

6)	shall include an Events header field set to "presence"; and

7)	shall include an application/simple-filter+xml MIME body indicating per-functional alias restrictions of presence event package notification information indicating the served functional alias.

##### 22.2.2.2.9	Forwarding subscription to functional alias status towards another MCData server procedure

Upon receiving a SIP SUBSCRIBE request such that:

1)	Request-URI of the SIP SUBSCRIBE request contains the public service identity identifying the originating participating MCData function serving the MCData user;

2)	the SIP SUBCRIBE request contains an application/vnd.3gpp.mcdata-info MIME body containing the<mcdata-request-uri> element which identifies an MCData ID not served by MCData server;

3)	the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata" (coded as specified in 3GPP TS 24.229 [5]), in a P-Asserted-Service header field according to IETF RFC 6050 [7]; and

4)	the Event header field of the SIP SUBSCRIBE request contains the "presence" event type;

then the MCData server:

1)	shall identify the target MCData ID in the <mcdata-request-uri> element of the application/vnd.3gpp.mcdata-info MIME body of the SIP SUBSCRIBE request;

2)	shall identify the originating MCData ID from public user identity in the P-Asserted-Identity header field of the SIP SUBSCRIBE request;

3)	shall generate a SIP SUBSCRIBE request from the received SIP SUBSCRIBE request. In the generated SIP SUBSCRIBE request, the MCData server:

a)	shall set the Request-URI to the public service identity identifying the terminating participating MCData function serving the target MCData ID;

b)	shall include a P-Asserted-Identity header field containing the public service identity identifying the originating participating MCData function serving the MCData user;

c)	shall include an application/vnd.3gpp.mcdata-info+xml MIME body. In the application/vnd.3gpp.mcdata-info+xml MIME body, the MCData server:

A)	shall include the <mcdata-request-uri> element set to the target MCData ID; and

B)	shall include the <mcdata-calling-user-id> element set to the originating MCData ID; and

d)	shall include other signalling elements from the received SIP SUBSCRIBE request; and

4)	shall send the generated SIP SUBSCRIBE request according to 3GPP TS 24.229 [5].

The MCData server shall forward to the originating MCData ID any received SIP responses to the SIP SUBSCRIBE request, and for the duration of the subscription any received SIP NOTIFY requests and any received SIP responses to the SIP NOTIFY request according to 3GPP TS 24.229 [5].

* * * * * * * NEXT CHANGE * * * * * * *

##### 22.2.2.3.7	Receiving subscription to functional alias resolution procedure

Upon receiving a SIP SUBSCRIBE request such that:

1)	Request-URI of the SIP SUBSCRIBE request contains the public service identity of the controlling MCData function associated with the requested functional alias;

2)	the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata" (coded as specified in 3GPP TS 24.229 [5]), in a P-Asserted-Service header field according to IETF RFC 6050 [7];

3)	the Event header field of the SIP SUBSCRIBE request contains the "presence" event type; and

4)	the SIP SUBSCRIBE request contains an application/simple-filter+xml MIME body indicating per-functional alias restrictions of presence event package notification information according to clause 22.3.2;

then the MCData server:

1)	shall identify the requested functional alias ID in the <mcdata-request-uri> element of the application/vnd.3gpp.mcdata-info+xml MIME body of the SIP SUBSCRIBE request;

2)	if the Expires header field of the SIP SUBSCRIBE request is not included or has nonzero value lower than 4294967295, shall send a SIP 423 (Interval Too Brief) response to the SIP SUBSCRIBE request, where the SIP 423 (Interval Too Brief) response contains a Min-Expires header field set to 4294967295, and shall not continue with the rest of the steps;

3)	if the requested functional alias does not exist in the MCData server, shall reject the SIP SUBSCRIBE request with SIP 403 (Forbidden) response to the SIP PUBLISH request according to 3GPP TS 24.229 [5], IETF RFC 3903 [34] and IETF RFC 3856 [39] and skip the rest of the steps; and

4)	shall generate a SIP 200 (OK) response to the SIP SUBSCRIBE request according to 3GPP TS 24.229 [5], IETF RFC 6665 [36].

For the duration of the subscription, the MCData server shall notify subscriber about changes of the information of the requested functional alias, as described in clause 22.2.2.2.4.

##### 22.2.2.3.8	Sending notification to functional alias resolution procedure

In order to notify the subscriber about the MCData users that have successfully activated the functional alias corresponding to the requested functional alias ID, the MCData server:

1)	shall consider a functional alias information entry such that:

a)	the functional alias information entry is in the list of functional alias information entries described in clause 22.2.2.3.2; and

b)	the functional alias ID of the functional alias information entry is equal to the requested functional alias ID;

2)	shall consider any MCData user information entry such that the MCData user information entry is in the list of the MCData user information entries of the served functional alias information entry, as the served MCData user information entry;

3)	shall generate an application/pidf+xml MIME body indicating per-functional alias information according to clause 22.3.1 and the served list of the served MCData user information entry of the functional alias information entry

4)	send a SIP NOTIFY request according to 3GPP TS 24.229 [5], and IETF RFC 6665 [36] for the subscription created in clause 22.2.2.3.7. In the SIP NOTIFY request, the MCData server shall include the generated application/pidf+xml MIME body indicating per-functional alias information.

* * * * * * * NEXT CHANGE * * * * * * *

## D.1.2	XML schema

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="urn:3gpp:ns:mcdataInfo:1.0"

xmlns:mcdatainfo="urn:3gpp:ns:mcdataInfo:1.0"

elementFormDefault="qualified"

attributeFormDefault="unqualified"

xmlns:xenc="http://www.w3.org/2001/04/xmlenc#">

<xs:import namespace="http://www.w3.org/2001/04/xmlenc#" schemaLocation="http://www.w3.org/TR/xmlenc-core/xenc-schema.xsd"/>

<!-- root XML element -->

<xs:element name="mcdatainfo" type="mcdatainfo:mcdatainfo-Type" id="info"/>

<xs:complexType name="mcdatainfo-Type">

<xs:sequence>

<xs:element name="mcdata-Params" type="mcdatainfo:mcdata-ParamsType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcdatainfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="mcdata-ParamsType">

<xs:sequence>

<xs:element name="mcdata-access-token" type="mcdatainfo:contentType" minOccurs="0"/>

<xs:element name="request-type" type="xs:string" minOccurs="0"/>

<xs:element name="mcdata-request-uri" type="mcdatainfo:contentType" minOccurs="0"/>

<xs:element name="mcdata-calling-user-id" type="mcdatainfo:contentType" minOccurs="0"/>

<xs:element name="mcdata-called-party-id" type="mcdatainfo:contentType" minOccurs="0"/>

<xs:element name="mcdata-calling-group-id" type="mcdatainfo:contentType" minOccurs="0"/>

<xs:element name="alert-ind" type="mcdatainfo:contentType" minOccurs="0"/>

<xs:element name="originated-by" type="mcdatainfo:contentType" minOccurs="0"/>

<xs:element name="mcdata-client-id" type="mcdatainfo:contentType" minOccurs="0"/>

<xs:element name="mcdata-controller-psi" type="mcdatainfo:contentType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcdatainfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!--    anyExt elements for MCData-Params-->

<xs:element name="emergency-alert-area-ind" type="xs:boolean"/>

<xs:element name="group-geo-area-ind" type="xs:boolean"/>

<xs:element name="pre-established-session-ind" type="xs:boolean"/>

<xs:element name="call-to-functional-alias-ind" type="xs:boolean"/>

<xs:element name="mcdata-communication-state" type="mcdatainfo:mcdataCommunicationStateType"/>

<xs:simpleType name="mcdataCommunicationStateType">

<xs:restriction base="xs:string">

<xs:enumeration value="establish-request"/>

<xs:enumeration value="establish-success"/>

<xs:enumeration value="establish-fail"/>

<xs:enumeration value="terminate-request"/>

<xs:enumeration value="terminated"/>

</xs:restriction>

</xs:simpleType>

<xs:element name="emergency-ind" type="xs:boolean"/>

<xs:element name="alert-ind-rcvd" type="xs:boolean"/>

<xs:element name="mc-org" type="xs:string"/>

<xs:element name="functional-alias-URI" type="mcdatainfo:contentType"/>

<xs:element name="multiple-devices-ind" type="mcdatainfo:contentType"/>

<xs:element name="imminentperil-ind" type="xs:boolean"/>

<xs:element name="emergency-ind-rcvd" type="xs:boolean"/>

<xs:element name="binding-ind" type="xs:boolean"/>

<xs:element name="binding-fa-uri" type="xs:anyURI"/>

<xs:element name="unbinding-fa-uri" type="xs:anyURI"/>

<xs:element name="store-all-private-comms-in-msgstore" type="xs:boolean"/>

<xs:element name="store-all-group-comms-in-msgstore" type="xs:boolean"/>

<xs:element name="store-specific-private-comms-in-msgstore" type="mcdatainfo:storageCtrlType"/>

<xs:element name="store-specific-group-comms-in-msgstore" type="mcdatainfo:storageCtrlType"/>

<xs:simpleType name="storageCtrlType">

<xs:restriction base="xs:string">

<xs:enumeration value="enable"/>

<xs:enumeration value="disable"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="protectionType">

<xs:restriction base="xs:string">

<xs:enumeration value="Normal"/>

<xs:enumeration value="Encrypted"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="contentType">

<xs:choice>

<xs:element name="mcdataURI" type="xs:anyURI"/>

<xs:element name="mcdataString" type="xs:string"/>

<xs:element name="mcdataBoolean" type="xs:boolean"/>

<xs:any namespace="##other" processContents="lax"/>

<xs:element name="anyExt" type="mcdatainfo:anyExtType" minOccurs="0"/>

</xs:choice>

<xs:attribute name="type" type="mcdatainfo:protectionType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="anyExtType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:schema>

## D.1.3	Semantic

The <mcdatainfo> element is the root element of the XML document. The <mcdatainfo> element can contain subelements.

NOTE 1:	The subelements of the <mcdata-info> are validated by the <xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> particle of the <mcdata-info> element

If the <mcdatainfo> contains the <mcdata-Params> element then:

1)	the <mcdata-access-token>, <mcdata-request-uri>, <mcdata-controller-psi>, <mcdata-calling-user-id>, <mcdata-called-party-id>, <mcdata-calling-group-id>, <alert-ind>, <originated-by>, <mcdata-client-id> and <functional-alias-URI> can be included with encrypted content;

2)	for each element in 1) that is included with content that is not encrypted:

a)	the element has the "type" attribute set to "Normal";

b)	if the element is the <mcdata-request-uri>, <mcdata-calling-user-id>, <mcdata-called-party-id>, <mcdata-calling-group-id>, <originated-by> or <functional-alias-URI> then the <mcdataURI> element is included;

c)	if the element is the <mcdata-access-token> or <mcdata-client-id>, then the <mcdataString> element is included; and

d)	if the element is <alert-ind>, then the <mcdataBoolean> element is included; and

3)	for each element in 1) that is included with content that is encrypted:

a)	the element has the "type" attribute set to "Encrypted";

b)	the <xenc:EncryptedData> element from the "http://www.w3.org/2001/04/xmlenc#" namespace is included and:

i)	can have a "Type" attribute can be included with a value of "http://www.w3.org/2001/04/xmlenc#Content";

ii)	can include an <EncryptionMethod> element with the "Algorithm" attribute set to value of "http://www.w3.org/2009/xmlenc11#aes128-gcm";

iii)	can include a <KeyInfo> element with a <KeyName> element containing the base 64 encoded XPK-ID; and

iv)	includes a <CipherData> element with a <CipherValue> element containing the encrypted data.

NOTE 2:	When the optional attributes and elements are not included within the <xenc:EncryptedData> element, the information they contain is known to sender and the receiver by other means.

If the <mcdatainfo> contains the <mcdata-Params> element then:

1)	the <mcdata-access-token> can be included with the access token received during authentication procedure as described in 3GPP TS 24.482 [24];

2)	the <request-type> can be included with:

a)	a value of "one-to-one-sds" to indicate that the MCData client wants to initiate a one-to-one SDS request;

b)	a value of "group-sds" to indicate the MCData client wants to initiate a group SDS request;

c)	a value of "one-to-one-fd" to indicate that the MCData client wants to initiate a one-to-one FD request;

d)	a value of "group-fd" to indicate that the MCData client wants to initiate a group FD request;

e)	a value of "msf-disc-req" to indicate that the MCData client wishes to discover the absoluteURI of the media storage function for HTTP requests;

f)	a value of "msf-disc-res" when the participating MCData function sends the absolute URI to the MCData client;

g)	a value of "notify" when the controlling MCData function needs to send a notification to the MCData client;

h)	a value of "one-to-one-sds-session" to indicate that the MCData client wants to initiate a one-to-one SDS session;

i)	a value of "group-sds-session" to indicate the MCData client wants to initiate a group SDS session;

j)	a value of "functional-alias-status-determination" when a client initiates a subscription request to FA status;

k)	"fa-group-binding-req" when a client initiates a request for binding of a functional alias with the MCData group(s) for the MCData user; or

l)	a value of "store-comms-in-msgstore-ctrl-req" when an MCData client initiates a request to control the storage of MCData communications (private and group) into MCData message store;

3)	the <mcdata-request-uri> can be included with an MCData group ID or an MCData user ID;

4)	the <mcdata-calling-user-id> can be included, set to MCData ID of the originating user;

5)	the <mcdata-called-party-id> can be included, set to the MCData ID of the terminating user;

6)	the <mcdata-calling-group-id> can be included to indicate the MCData group identity to the terminating user;

7)	the <alert-ind> can be:

a)	set to "true" to indicate that an alert is to be sent; or

b)	set to "false" to indicate that an alert is to be cancelled;

8)	the <originated-by> can be included, set to the MCData ID of the originating user of an MCData emergency alert when being cancelled by another authorised MCData user;

9)	the <mcdata-client-id> can be included, set to the MCData client ID of the MCData client that originated a SIP INVITE request, SIP REFER request, SIP REGISTER request, SIP PUBLISH request or SIP MESSAGE request;

10)	the <mcdata-controller-psi> can be included, set to the PSI of the controlling MCData function that handled the one-to-one or group MCData data request; and

11)	the <anyExt> can be included with the following elements:

a)	a <pre-established-session-ind> element :

i)	set to the value "true" by the MCData client in a pre-established session setup request to indicate to the MCData participating function about initiation of a pre-established session;

b)	an <mcdata-communication-state> element can be included to indicate the state of MCData communication within a pre-established session. The <mcdata-communication-state> can be set to:

i)	the value "establish-request" by the MCData participating function to indicate to the MCData client about an MCData communication establishment request within a pre-established session;

ii)	the value "establish-success" by the MCData participating function or the MCData client to indicate that the MCData communication is established successfully;

iii)	the value "establish-fail" by the MCData participating function or the MCData client to indicate that the MCData communication establishment is failed or rejected;

iv)	the value "terminate-request" by the MCData participating function to indicate to the MCData client about an MCData communication termination request within a pre-established session; or

v)	the value "terminated" by the MCData participating function or the MCData client to indicate that the MCData communication is terminated;

c)	an <emergency-ind> element can be included and set to:

i)	"true" to indicate that the communication that the MCData client is initiating is an emergency MCData communication; or

ii)	"false" to indicate that the MCData client is cancelling an emergency MCData communication (i.e. converting it back to a non-emergency communication);

d)	an <alert-ind-rcvd> element:

i)	may be set to "true" and included in a SIP MESSAGE to indicate that the emergency alert or cancellation was received successfully;

e)	an <mc-org> element may be:

i)	set to the MCData user's Mission Critical Organization and included in an emergency alert sent by the MCData server to terminating MCData clients;

f)	a <functional-alias-URI> element set to the value of the functional alias that is used together with the "mcdata-calling-user-id";

g)	an <emergency-alert-area-ind> element:

i)	set to the value "true" when the MCData client has entered an emergency alert area; or

ii)	set to the value "false" when the MCData client has exited an emergency alert area;

h)	a <group-geo-area-ind> element:

i)	set to the value "true" when the MCData client has entered a group geographic area; or

ii)	set to the value "false" when the MCData client has exited a group geographic area;

i)	an <imminentperil-ind> element can be included if the <mcdata-request-uri> is also included and set to an MCData group ID, in which case the <imminentperil-ind> element is to be set to:

i)	"true" to indicate that the communication that the MCData client is initiating is an imminent peril MCData communication; or

ii)	"false" to indicate that the MCData client requests that the communication should no longer be considered an imminent peril MCData communication;

j)	an <emergency-ind-rcvd> element:

i)	can be set to "true" and included in a SIP MESSAGE to indicate that the in-progress emergency cancellation request was received successfully;

k)	a <multiple-devices-ind> element can be included and set to:

i)	"true" to indicate to the client that multiple clients are registered for the MCData user; or

ii)	"false" to indicate to the client that no other clients are registered for the MCData user;

l)	a <binding-ind> element set to:

i)	"true" when the user wants to create a binding of a particular functional alias with the specified list of MCData groups for the MCData client; or

ii)	"false" when the user wants to remove a binding of a particular functional alias from the specified list of MCData groups for the MCData client;

m)	a <binding-fa-uri> element set to:

i)	a URI of a functional alias that shall be bound with the specified list of MCData groups for the MCData client;

n)	a <unbinding-fa-uri> element set to:

i)	a URI of a functional alias that shall be unbound from the specified list of MCData groups for the MCData client;

o)	a <store-all-private-comms-in-msgstore> element can be included and set to:

i)	"true" when the user wants to store his/her MCData private communications into his/her MCData message store account; or

ii)	"false" when the user do not store his/her MCData private communications into his/her MCData message store account;

p)	a <store-all-group-comms-in-msgstore> element can be included and set to:

i)	"true" when the user wants to store his/her MCData group communications into his/her MCData message store account; or

ii)	"false" when the user do not store his/her MCData group communications into his/her MCData message store account;

q)	a <store-specific-private-comms-in-msgstore> element can be included and set to:

i)	set to a value of "enable" when the user wants to store the specified MCData private communications for which user is authorized to store the communication into the MCData message store; or

ii)	set to a value of "disable" when the user do not wants to store the specified MCData private communications for which user is authorized to store the communication into the MCData message store;

r)	a <store-specific-group-comms-in-msgstore> element can be included and set to:

i)	"enable" when the user wants to store the specified MCData group communications for which user is authorized to store the communication into the MCData message store; or

ii)	"disable" when the user do not wants to store the specified MCData group communications for which user is authorized to store the communication into the MCData message store; and

s)	an <call-to-functional-alias-ind> element can be included and set to:

i)	"true" when the MCData client is using a functional alias to identify the MCData IDs of the potential target MCData users; or

ii)	"false" when the MCData client is using MCData IDs to identify the potential target MCData users.

Absence of the <emergency-ind>, <alert-ind> and <imminentperil-ind> in a SIP INVITE request indicates that the MCData client is initiating a non-emergency communication.

Absence of the <call-to-functional-alias-ind> in a SIP INVITE or a SIP REFER request indicates the use of the MCData IDs of the potential target MCData users.

The recipient of the XML ignores any unknown element and any unknown attribute.

* * * * * * * END CHANGE * * * * * * *