3GPP TSG-CT WG1 Meeting #133e-bis	C1-220327

- E-meeting, 17-21 January 2022  										    was C1-217287, C1-216883,

- C1-216212, C1-215962

Source:	Samsung, AT&T, Huawei, HiSilicon

Title:	Eees_EASDiscovery_UpdateSubscription operation for Eees_EASDiscovery API

Spec:	3GPP TS 24.558 v1.0.0

Agenda item:	17.2.16

Document for:	Agreement

1. Introduction

This pCR specifies Eees_EASDiscovery_UpdateSubscription service operation.

2. Reason for Change

The Eees_EASDiscovery_UpdateSubscription operation is defined in clause 8.5.4.5 of 3GPP TS 23.558. It is required to provide service operations details in stage#3.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.558 v1.0.0.

* * * First Change * * * *

#### 5.3.2.5	Eees_EASDiscovery_UpdateSubscription

##### 5.3.2.5.1	General

This service operation is used by the EEC to update its subscription at the EES for reporting of EAS discovery information.

##### 5.3.2.5.2	EEC updating EAS discovery information subscription at EES using Eees_EASDiscovery_UpdateSubscription operation

To request modification of an existing Individual EAS Discovery Subscription, the EEC shall send an HTTP PATCH request (for partial modification) or PUT request (for fully replacement) message to the EES using the resource URI identifying the concerned "Individual EAS Discovery Subscription" resource as specified in clause 6.3.2.3.3.3 for an HTTP PATCH message and in clause 6.3.2.3.3.1 for an HTTP PUT message.

The PATCH message includes the parameters (EAS discovery filters, EAS dynamic information filters, Service continuity support, or proposed expiry time) that need to be replaced in the existing subscription resource.

The PUT message shall replace all  properties of the existing resource with the EAS Discovery subscription information in the request. The values of the easId and ueId provided during the subscription creation shall not be changed.

Upon receiving the HTTP PATCH or PUT message from the EEC, the EES:

a)	shall check the update subscription message from the EEC to see if the EEC is authorized to update the requested subscription resource;

b)	if the EEC is authorized to update the EAS discovery subscription and if the eecId and the ueId of the request match the eecId and the ueId in the resource, then the EES;

1)	shall update the resource identified by Resource URI of the EAS discovery subscription with the updated information received in the HTTP PATCH or PUT request message;

2)	on success, shall either return an HTTP "200 OK" response with the payload body of the HTTP PATCH or PUT response containing the representation of the replaced resource or an HTTP "204 No Content" response.

c)	if the EAS discovery subscription update operation is unsuccessful, shall return an appropriate error response as specified in Table 5.2.6-1 of 3GPP TS 29.122 [3];

d)	may include an expiration time.

If the expiration time is provided, the EEC shall send the EAS discovery subscription update request prior to the expiration time if the EEC wants to maintain the subscription. If the EAS discovery subscription update request is not received prior to the expiration time, the EES shall treat the EEC as implicitly unsubscribed and remove the corresponding EAS discovery subscription resource.

* * * End of Change * * * *