3GPP TSG-CT WG1 Meeting #133-bis-e	C1-220247

- E-meeting, 17-21 January 2022

Source:	ZTE

Title:	Registration for constrained devices

Spec:	3GPP TS 24.538 v0.2.1

Agenda item:	17.2.30

Document for:	Agreement

1. Introduction

Registration procedure for MSGin5G UE in MSGin5G-1 has been introduced in TS 24.538. The registration procedure for the constrained devices which do not have enough capability to communicate with MSGin5G Server has not been covered.

2. Reason for Change

The constrained devices may not have enough capability to communicate with the MSGin5G Server. If allowed by configuration, the contrained devices communicates with the MSGin5G Server via a Gateway MSGin5G UE:

- over the MSGin5G-5 interface between the Application Client of the constrained device and the MSGin5G Client of the Gateway MSGin5G UE; or

- over the MSGin5G-6 interface between the MSGin5G Client of the constrained device and the MSGin5G Client of the Gateway MSGin5G UE.

The MSGin5G-6 reference is based on NR PC5. If the MSGin5G-6 interface is used, the constrained device supports the registration procedure defined for the MSGin5G Client in subclause 6.3.1.1.1. The Gateway MSGin5G UE maintains the association between the UE Service ID of the constrained device and the PC5 link identifier defined in TS 24.554. Such association is used for distribute downlink data correctly. The Gateway MSGin5G UE replaces the Gateway MSGin5G UE address in the Option header of the received CoAP message with the MSGin5G Server address.

3. Conclusions

Complete registration procedure for constrained devices.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.538 v0.2.1.

* * * First Change * * * *

### 6.3.2	Constrained device registration to use Gateway MSGin5G UE

This clause covers the procedures in MSGin5G-5 and/or MSGin5G-6.

#### 6.3.2.1	Procedure at Gateway MSGin5G UE

##### 6.3.2.1.1	Constrained device registration to use Gateway MSGin5G UE

Upon reception of the CoAP POST request message containing MSGin5G service identifier indicating that the received request message is for MSGin5G service and Message Type indicating that the received request message is for registration from the constrained device:

a)	the MSGin5G Client of the Gateway MSGin5G UE shall store the UE Service ID included in the received CoAP POST request message and the PC5 link identifier of the constrained device as defined in TS 24.554[X], if unicast mode communitcation as defined in TS 24.554[X] is used between the constrained device and the Gateway MSGin5G UE;

b)	the MSGin5G Client of the Gateway MSGin5G UE which has been configured one or more destination layer-2 ID(s) for reception of data unit(s) over PC5 shall store the UE Service ID included in the received CoAP POST request message with the configured destination layer-2 ID, if broadcast mode communitcation as defined in TS 24.554[X] is used between the constrained device and the Gateway MSGin5G UE; and

c)	the MSGin5G Client of the Gateway MSGin5G UE shall generate a new CoAP POST request message and send it to the MSGin5G Server. The MSGin5G Client of the Gateway MSGin5G UE:

1)	shall construct the CoAP POST request message as specified in subclause 6.3.1.1.1 with the corresponding value in the recevied CoAP POST request message from the constrained device except the Option header; and

2)	shall include the MSGin5G Server address in the Option header of the CoAP POST request message and set the Option header to a corresponding value, e.g. if the MSGin5G Server address is a URI, the Uri-Path Option is set to the value of such URI.

Upon reception of the CoAP 2.01 (Created) response or CoAP 2.04 (Change) response from the MSGin5G Server, the Gateway MSGin5G UE checks whether the UE service identifier included in the "UE Service ID" identifies a stored constrained device as followe:

a)	If the the UE service identifier included in the "UE Service ID" identifies a stored constrained device, the MSGin5G Client of Gateway MSGin5G UE shall generate a new CoAP 2.01 (Created) response or CoAP 2.04 (Change) response and construct the CoAP 2.01 (Created) response or CoAP 2.04 (Change) response as specified in bullet b) of subclause 6.3.1.2.1 with the corresponding value in the recevied the CoAP 2.01 (Created) response or CoAP 2.04 (Change) response message from the MSGin5G Server. The MSGin5G Client of Gateway MSGin5G UE sends it to the MSGin5G Client of constrained device via unicast mode or broadcast mode over PC5 as defined in TS 24.554[X].

b)	Otherwise, the Gateway MSGin5G UE shall discard the received CoAP 2.01 (Created) response or CoAP 2.04 (Change) response.

* * * Next Change * * * *

#### 6.3.2.2	Procedure at Constrained device

##### 6.3.2.2.1	Constrained device registration to use Gateway MSGin5G UE

In order to register constrained device to the MSGin5G Server, the MSGin5G Client of constrained device sends a CoAP POST request message to the Gateway MSGin5G UE via unicast mode or broadcast mode over PC5 as defined in TS 24.554[X]. The CoAP POST request message is constructed as specified in subclause 6.3.1.1.1 except the Option header of the CoAP POST request message. The MSGin5G Client of constrained device includes the MSGin5G Client address of Gateway MSGin5G UE in the Option header of the CoAP POST request message and set the Option header to a corresponding value, e.g. if the MSGin5G Client address of Gateway MSGin5G UE is a URI, the Uri-Path Option is set to the value of such URI.

* * * End of Change * * * *