3GPP TSG-CT WG1 Meeting #133-bis-e	C1-220250

- E-meeting, 17-21 January 2022

Source:	ZTE

Title:	Correction of registration procedure

Spec:	3GPP TS 24.538 v0.2.1

Agenda item:	17.2.30

Document for:	Agreement

1. Introduction

Registration and other procedures have been introduced in TS 24.538. This pCR is proposed to add the missing IE of "MSGin5G service identifier" and make some correction.

2. Reason for Change

1. During the message delivery, the element of "MSGin5G service identifier" has been introduced in the CoAP POST request to indicate that this CoAP POST request message is used for MSGin5G service. However, it is missing in the CoAP messages for registration procedure.

2. Remove the description of the registration request message. In the last meeting, it was replaced of CoAP POST request message.

3. Remove the redundant condition.

4. The MSGin5G Client address is not included in the CoAP POST request message. Thus the MSGin5G Server cannot retrieves such address and fill it in the Option header of the CoAP response message.

3. Conclusions

Add the missing IE and make some correction.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.538 v0.2.1.

* * * First Change * * * *

##### 6.3.1.1.1	MSGin5G UE registration

After the MSGin5G UE receives the UE Service ID, in order to register MSGin5G UE to the MSGin5G Server, the MSGin5G Client shall send a CoAP POST request message to the MSGin5G Server to according to procedures specified in IETF RFC 7252 [5]. In the CoAP POST request message, the MSGin5G Client:

a)	shall set the "T" field in the CoAP header to 0 to indicate acknowledge message required;

b)	shall include the MSGin5G Server address in the Option header of the CoAP POST request message and set the Option header to a corresponding value, e.g. if the MSGin5G Server address is a URI, the Uri-Path Option is set to the value of such URI;

c)	shall set the "Content-Format" element to "50" to indicate the format of the CoAP payload is "application/json"; and

d)	shall include the following information elements in the CoAP payload encoded in JSON format:

1)	the "MSGin5G service identifier" element to indicate that this CoAP POST request message is used for MSGin5G service;

2)	the "Message Type" element to indicate that the CoAP POST request message is used for registration;

3)	the "UE Service ID" element to indicate the MSGin5G UE initiating registration procedure;

4)	the "UE Credential Information" element to indicate the authentication type and related information;

- Editor's note:	The details of authentication type and related information are FFS.

5)	optionally, the "MSGin5G Client Profile" element to include a set of parameters describing the MSGin5G Client. This element may include the "MSGin5G Client Triggering Information" element and the "MSGin5G Client Communication Availability" element. The "MSGin5G Client Triggering Information" element shall include the "MSGin5G UE ID" element to indicate the MSGin5G UE hosting the MSGin5G Client and the "MSGin5G Client Ports" element to indicate that the MSGin5G Client listens on for device triggers from the MSGin5G Server.

)	he "MSGin5G Client Communication Availability" element tnforms the MSGin5G Server whether the client has a specific application-level schedule/periodicity to its MSGin5G communications, which may be used in conjunction with UE reachability monitoring to determine whether and when MSGin5G communications are attempted. This element:

i)	shall include the "Scheduled communication time" element to indicate the time when the UE becomes available for communication;

ii)	shall include the "Communication duration time" element to indicate the duration time of periodic communication;

iii)	may include the "Periodic communication indicator" element to identify whether the client communicates periodically or not;

iv)	shall include the "Periodic communication interval" element to indicate the interval Time of periodic communication if "Periodic communication indicator" element is included;

v)	may include the "Data size indication" element to indicate the expected data size to be exchanged during the communication duration; and

vi)	may include the "Store and forward option" element to indicate the UE does not request store and forward services for incoming MSGin5G requests.

* * * Next Change * * * *

##### 6.3.1.2.1	MSGin5G UE registration

Upon reception of the CoAP POST request message containing MSGin5G service identifier indicating that the received request message for MSGin5G service and Message Type indicating that the received request message for registration, the MSGin5G Server shall verifies the security credentials according to "UE Credential Information" element. After a successful verification, the MSGin5G Server:

a)	shall store the UE Service ID and the MSGin5G Client Profile information included in the received CoAP POST request; and

b)	shall generate a CoAP 2.01 (Created) response or CoAP 2.04 (Change) response including the following parameters:

1)	the CoAP "Message ID" element and the "Token" element with the same values with those in the CoAP POST request message for registration;

2)

the "Content-Format" element with "50" to indicate the format of the CoAP payload is "application/json" and the CoAP payload including:

i)	the "UE Service ID" element to indicate the MSGin5G UE initiating registration procedure; and

ii)	the "Registration result" element to indicate whether the registration is success or failure.

- Editor's note:	How to verify the security credentials is FFS.

* * * End of Change * * * *