3GPP TSG-CT WG1 Meeting #133-bis-e	C1-220269

- E-meeting, 17-21 January 2022

Source:	ZTE

Title:	The structure of the CoAP message for deregistration

Spec:	3GPP TS 24.538 v0.2.1

Agenda item:	17.2.30

Document for:	Agreement

1. Introduction

De-registration procedure for MSGin5G UE in MSGin5G-1 has been introduced in TS 24.538. This pCR proposes to define the structure of the CoAP message for de-registration

2. Reason for Change

The schema for the body of CoAP request/response is based on JSON Schema Draft-07. In order to reduce the overhead of MSGin5G message, the properties are defined as shorten form and the relationship between the properties and IEs are described in the description of the properties.

3. Conclusions

The schema based on JSON Schema Draft-07 is introduced for the body of CoAP request/response for de-registration.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.538 v0.2.1.

* * * First Change * * * *

## 7.3	MSGin5G message structure

### 7.3.X	MSGin5G UE De-registration structure

The schema is based on JSON Schema Draft-07. For reducing the overhead of the message used in MSGin5G service, the properties are defined as shorten form and the relationships between the properties and elements used in subclause 6.3.1.1.2 are described in the description of the properties. The JSON schema of the CoAP POST request for the MSGin5G de-registration is defined below:

{

"$schema": "http://json-schema.org/draft-07/schema#",

"$id": "http://www.3gpp.org/MSGin5G/MSGin5G_Deregistration_request_schema",

"title": "MSGin5G Deregistration Request",

"type": "object",

"properties": {

"msgIden": {

"type": "string",

"format": "uri",

"description": "Refer to Service identifier of MSGin5G service"

},

"msgType": {

"type": "string",

"enum": [

"DEREG"

],

"description": "Refer to the usage of this message. The value DEREG refers to MSGin5G De-registration"

},

"oriAddr": {

"type": "object",

"properties": {

"oriAddrType": {

"enum": [

"UE"

]

},

"addr": {

"type": "string"

}

},

"description": "Refer to Originating UE Service ID"

},

"secCred": {

"type": "object",

"description": "Refer to Security Credentials"

}

},

"required": [

"msgIden",

"oriAddr ",

"secCred"

]

}

The schema is based on JSON Schema Draft-07. For reducing the overhead of the message used in MSGin5G service, the properties are defined as shorten form and the relationships between the properties and elements used in subclause 6.3.1.2.2 are described in the description of the properties. The JSON schema of CoAP 2.04 (Change) response for the MSGin5G de-registration is defined below:

{

"$schema": "http://json-schema.org/draft-07/schema#",

"$id": "http://www.3gpp.org/MSGin5G/MSGin5G_ Deregistration_response_schema",

"title": "MSGin5G Deregistration Response",

"type": "object",

"properties": {

"oriAddr": {

"type": "object",

"properties": {

"oriAddrType": {

"enum": [

"UE"

]

},

"addr": {

"type": "string"

}

},

"description": "Refer to Originating UE Service ID"

},

"result": {

"type": "boolean",

"default": true,

"description": "Refer to De-registration result. The value true refers to succcess"

}

},

"required": [

"oriAddr",

"result"

]

}

* * * End of Change * * * *