3GPP TSG-CT WG1 Meeting #133-e	C1-216883

- E-meeting, 11-19 November 2021  										was C1-216212, C1-215962

Source:	Samsung

Title:	Eees_EASDiscovery_UpdateSubscription operation for Eees_EASDiscovery API

Spec:	3GPP TS 24.558 v0.5.0

Agenda item:	17.2.16

Document for:	Agreement

1. Introduction

This pCR specifies Eees_EASDiscovery_UpdateSubscription service operation.

2. Reason for Change

The Eees_EASDiscovery_UpdateSubscription operation is defined in clause 8.5.4.5 of 3GPP TS 23.558. It is required to provide service operations details in stage#3.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.558 v0.5.0.

* * * First Change * * * *

#### 5.3.2.5	Eees_EASDiscovery_UpdateSubscription

##### 5.3.2.5.1	General

This service operation is used by the EEC to update its subscription at EES for reporting of EAS discovery information.

##### 5.3.2.5.2	EEC updating EAS discovery information subscription at EES using Eees_EASDiscovery_UpdateSubscription operation

To update an EAS discovery information subscription at the EES by mean of a complete resource replacement, the EEC shall send an HTTP PUT message to the EES using the resource URI identifying the concerned Individual EAS Discovery Subscription resource representation, as specified in clause 6.3.2.3.3.1.

Upon receiving the HTTP PUT message from the EEC, the EES:

a)	shall check the update subscription message from the EEC to see if the EEC is authorized to update the requested subscription resource;

b)	if the EEC is authorized to update the EAS discovery subscription and if the eecId the ueId of the request match the eecId and the ueId in the resource, then the EES;

1)	shall update the resource identified by Resource URI of the EAS discovery subscription with the updated  information received in the HTTP PUT request message;

2)	on success, shall either return an HTTP "200 OK" response with the payload body of the PUT response containing the representation of the replaced resource or an HTTP “204 (No Content)” response.

c)	if the EAS discovery subscription update operation is unsuccessful, shall return an appropriate error response as specified in Table 5.2.6-1 of TS 29.122 [3].

If the expiration time is provided, then to maintain the subscription, the EEC shall send the EAS discovery subscription update request prior to the expiration time. If the EAS discovery subscription update request is not received prior to the expiration time, the EES shall treat the EEC as implicitly unsubscribed and remove the corresponding EAS discovery subscription resource.

Editor’s note:	Whether PATCH method will be used to partially update the individual subscription resource is FFS.

* * * End of Change * * * *