3GPP TSG-CT WG1 Meeting #133-e	C1-216944 rev1

- E-meeting, 11-19 November 2021

Source:	China Mobile

Title:	pCR on MSGin5G Client sending an MSGin5G message

Spec:	3GPP TS 24.538 v0.1.0

Agenda item:	17.2.30

Document for:	Agreement

1. Introduction

This pCR proposes the the procedure of MSGin5G Client sending an MSGin5G message.

2. Reason for Change

Contribute new text.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.538 v0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.554: "Application architecture for MSGin5G Service; Stage 2;".

[3]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals".

[y]	IETF RFC 7252: "The Constrained Application Protocol (CoAP)".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next Change * * * *

##### 6.4.1.1.2	Sending of an MSGin5G message

In order to send an MSGin5G message, the MSGin5G Client shall compares the size of the received message from the application client to the maximum allowed MSGin5G message segmentation size. If the size exceeds, the MSGin5G Client shall segment the MSGin5G message into a set of segmented MSGin5G messages such that each segmented MSGin5G message can fit within the maximum allowed MSGin5G message segmentation size. For each segmented MSGin5G message, the steps listed below shall be processed individually.

The MSGin5G Client shall send the MSGin5G message in an CoAP POST request message according to procedures specified in IETF RFC 7252 [y]. In the CoAP POST request message, The MSGin5G Client:

a)	shall set the "T" field in the CoAP header to 0 if delivery status report from the recipient is requested, i.e. indicates this message is the type of Confirmable, to ensure the application layer delivery status report;

b)	shall include the MSGin5G Server address in an CoAP Option, e.g. if the MSGin5G Server address is a URI, include a Uri-Path Option with the value of the URI;

c)	shall set the CoAP Content-Format to "50", i.e. application/json;

d)	shall include the information elements specified in 3GPP TS 23.554[2] in the CoAP payload encoded in JSON format:

Editor’s note:	the JSON CoAP payload is to be specified in clause 7.

1)	shall include an "MSGin5G service identifier" element to indicate that this CoAP POST request message is used for MSGin5G service;

2)	shall include an "Originating UE Service ID" element set to the UE which requests the sending of the MSGin5G message;

3)	shall include a "Recipient UE Service ID/AS Service ID" element if the recipient(s) is(are) MSGin5G UE/Non-MSGin5G UE or Application Server;

4)	 shall include a "Group Service ID" element if the recipient is an MSGin5G Group;

5)	shall include a "Broadcast Area ID" element if the message needs to be broadcast;

6)	shall include a "Messaging Topic" element if this message will be distributed based on message topic. This element shall not present in other message scenarios;

NOTE:	In a MSGin5G Message request, only one of these IEs listed in 3) to 6) shall be included.

7)	may include one or more "Application ID" element(s) to indicate the application(s) for which the payload is intended;

8)	shall included a "Message ID" which is globally unique within the MSGin5G service;

9)	shall include a "Security credentials" element which is required by the MSGin5G Server;

Editor's note:	the Security credentials element is a placeholder for SA3 security information.

10)	may include a "Delivery status required" element if delivery acknowledgement from the recipient is requested;

11)	may include a "Priority type" element to indicate the application priority level requested for this message;

12)	may include a "Message is segmented" element to indicate that this message is part of a segmented message;

13)if "Message is segmented" element is included, shall include a "Segmentation set identifier" element to indicate that this segmented message is associated within a set of segmented messages , all segmented messages associated with the same MSGin5G message are assigned the same unique identifier;

14)	if "Message is segmented" element is included and this message is the first segment of the MSGin5G message, shall include a "Total number of message segments" element to indicate the total number of segments for the MSGin5G message;

15)	if "Message is segmented" element is included, shall include a "Message segment number" element to indicate segmented message number of each segmented message within a set of segmented messages;

16)		if "Message is segmented" element is included and this message is the last segment of the MSGin5G message, shall include a "Last segment flag" element to indicate that this segmented message is the last segment in the set of segmented messages;

17)shall include a "Store and forward flag" element to indicate whether store and forward services are requested for this message;

18)	if store and forward services are requested, may include a "Store and forward parameters" element to carry the parameters used by MSGin5G Server for providing store and forward services. The "Store and forward parameters":

i)	may include a "Message expiration time" element to indicate message expiration time used for providing store and forward services if the destination is not available for communications;

ii)	may include a "Application specific store and forward information" element to carry the information used by MSGin5G Server for handling store and forward, e.g. a delivery time/date; and

19)	may include a "Payload" element specified in 3GPP TS 23.554[2] in the CoAP payload and located it after the MSGin5G header to carry the payload of this message.

* * * Next Change * * * *

##### 6.4.1.1.3	Sending of an aggregated MSGin5G message

Before the sending of a MSGin5G message, the MSGin5G Client shall check if aggregation is allowed for this message, check the message data size, and the priority level to determine if the message can be aggregated. For example, the MSGin5G Client finds that the messages have small payload size when compared to the maximum segment size that can be transmitted over CoAP and are not high priority messages, which could be sent as per scheduling policy towards a selected target. The MSGin5G Client can decide to continue aggregating messages until optimal use of segment size before sending message towards MSGin5G Server.

If the message can be aggregated, the MSGin5G Client aggregates multiple MSGin5G message requests intended for a selected target and sends the aggregated message in a single CoAP POST request message. The sending of the CoAP POST request message shall follow the procedures specified in clause 6.4.1.1.2 with the clarifications listed below:

1)	The MSGin5G Client should not segment the aggregated message, so in step d) of clause 6.4.1.1.2, the "Message is segmented", "Segmentation set identifier", "Total number of message segments", "Message segment number" and "Last segment flag" elements should not be included.

2)	In addition to the step d) of clause 6.4.1.1.2, the MSGin5G Client should include a "Number of individual messages" element in this message to indicate the total number of messages which are aggregated into this single message.

3)	In addition to the step d) of clause 6.4.1.1.2, the MSGin5G Client should include a "List of individual messages" element in this message. Each child element in this "List of individual messages" element contains information elements listed below:

i)	"Message ID" of the individual message;

ii)	"Payload";

iii)	one of more optional "Application ID" element(s);

iv)	an optional "Delivery status required" element; and

v)	an optional "Priority type" element.

4)	The MSGin5G Client should not include the "Payload" element, i.e. the 19) in step e) of clause 6.4.1.1.2 shall not be processed.