3GPP TSG-CT WG1 Meeting #133-e	C1-21xxxx

- E-meeting, 11-19 November 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Evaluation and conclusion on Scenario 3 in Key Issue 1

Spec:	3GPP TR 23.700-10 v1.1.0

Agenda item:	17.3.3

Document for:	Agreement

1. Introduction

There is neither evaluation nor conclusion on Scenario 3 in Key Issue 1.

2. Reason for Change

Evaluation and conclusion are necessary.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-10 v1.1.0.

* * * First Change * * * *

# 7	Overall Evaluation

For Key Issue #1:

Scenario 1

Editor's note:	The evaluation for Scenario 1 is FFS.

Scenario 2

-	Only Solution 1 was proposed in this scenario and the solution does not qualify as a basis for the normative work. See clause 6.1.3.

Scenario 3

-	Solution 2 and Solution 4 address how P-CSCFs in different IMS networks can be selected by an SMF.

-	Solution 2 requires enhancement in the 5GSM protocol.

The only benefit provided by Solution 2 is that it supports the case where all parameters available in the SMF (e.g., SUPI, UE location information, local operator policies, UE IP address, DNN) do not provide any meaningful differentiation and the IMS networks have different IMS home network domain names.

-	Solution 4 does not require change in any protocol but requires that a different DNN is assigned for each of the IMS networks which can be accessed by a single 5GC network slice of a PLMN.

For Key Issue #2, there is no solution to evaluate.

For Key Issue #3, there is no solution to evaluate.

For Key Issue #4, there is no solution to evaluate.

* * * Next Change * * * *

# 8	Conclusions

For Key Issue #1 / Scenario 2, no work in the normative phase will happen.

For Key Issue #1 / Scenario 3, no conclusion is made during the study phase.

Editor's note:	CT1 has sent an LS to SA2 for feedback. Based on feedback from SA2, CT1 will make a progress with normative work.

For Key Issue #2, no solutions within CT1 responsibility was provided, i.e. no work in the normative phase will happen.

For Key Issue #3, no solutions within CT1 responsibility was provided, i.e. no work in the normative phase will happen.

For Key Issue #4, no solutions within CT1 responsibility was provided, i.e. no work in the normative phase will happen.