3GPP TSG-CT WG1 Meeting #133-e	C1-217156

- E-meeting, 11-19 November 2021

Title:	Response to reply LS on UAC enhancements and system information extensions for minimization of service interruption

Response to:	Reply LS R2-2111599/C1-217140 on UAC enhancements and system information extensions for minimization of service interruption from RAN2

Release:	Rel-17

Work Item:	MINT

Source:	CT1

To:	RAN2

Cc:	-

Contact person:	Sung Hwan WON

sung.won@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

CT1 appreciates RAN2 agreements and questions in R2-2111599/C1-217140. CT1 would like to provide answers to the questions as below.

Question 1: Can a UE that performs disaster roaming be configured with any other Access Identities than Access Identity 3? And if so, which Access Identities should be considered by the UE when performing access barring evaluation? For example, should a UE performing disaster roaming which is configured with Access Identity 1, 2 or 11 to 15 and 3, only consider Access Identity 3?

Yes, a UE performing disaster roaming can be configured with one or more access identities other than Access Identity 3. Such a UE can be configured with Access Identity 1, 2, or 11 – 15.

Question 2: RAN2 does not expect there is impact on cell selection/reselection and would like to confirm whether CT1 foresee any impact on cell selection/reselection due to MINT?

CT1 does not foresee any impact on cell selection or reselection due to MINT.

Question 3: RAN2 understood that disaster roaming is not supported for SNPNs, but is disaster roaming supported for PNI-NPNs?

With respect to SNPN, CT1 agrees with RAN2. CT1 agreed that disaster roaming is not supported for PNI-NPNs, i.e., a PNI-NPN does not accept a UE performing disaster roaming if the UE is not allowed to access the PNI-NPN.

# 2	Actions

To RAN2

ACTION: 	CT1 kindly asks RAN2 to take the above answers into account for the future work.

# 3	Dates of next TSG CT WG1 meetings

CT1#133e	11th-19th November 2021	electronic meeting

CT1#133-bis-e	17th-21st January 2022	electronic meeting

CT1#134e	17th-25th February 2022	electronic meeting