3GPP TSG-CT WG1 Meeting #134-e	C1-221799

- E-Meeting, 17th – 25th February 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.390 | CR | 0058 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 3GPP PS data off and UE in SNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2022-02-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Rel-17, the UE can use IMS in SNPN, using selected entry of "list of subscriber data" or selected PLMN subscription. It is not specified how the UE and the AS can be configured with 3GPP PS data off exempt services in this situation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 3GPP PS data off exempt services can be configured for situation when the UE selects an SNPN using the selected entry of "list of subscriber data" or the selected PLMN subscription. Text of clause C.2.1.1.1 and C.2.2.1 is reused and adapted for situation when the UE is in SNPN, using selected entry of "list of subscriber data" or selected PLMN subscription. Changes are indicated by a different MS Word change marks. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are no 3GPP PS data off exempt services when 3GPP PS data off is activated and the UE is in an SNPN using the selected entry of "list of subscriber data" or the selected PLMN subscription. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | E.2.1.1.1, E.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Corrected "indications whether" to "indications of whether" added " using the subscribed SNPN's credentials" where applicable Moved the change to annex E which is for 5GS. |  |  |  |  |  |  |  |  |

***** change *****

#### E.2.1.1.1	General

The requirements in subclause C.2.1.1.1 apply.

Additionally, the following requirements apply.

In case of SNPN, if the UE supports the 3GPP PS data off:

1)	the UE can be configured with:

a)	up to two indications of whether the USSI is a 3GPP PS data off exempt service for each subscribed SNPN whose entry exists in the "list of subscriber data", one indication is valid for the UE camping in the subscribed SNPN, and the other indication is valid for any non-subscribed SNPN the UE is camping in using the subscribed SNPN's credentials; and

b)	an indication whether USSI is a 3GPP PS data off exempt service for PLMN subscription, valid for any non-subscribed SNPN the UE is camping in using the subscribed SNPN's credentials; and

2)	for a subscribed SNPN whose entry exists in the "list of subscriber data", the UE may support being configured with the indication whether the USSI is a 3GPP PS data off exempt service using one or more of the following methods:

a)	the USSI_exempt node of 3GPP TS 24.391 [13], if the UE is in the subscribed SNPN; and

b)	the USSI_exempt_non_subscribed_SNPN node of 3GPP TS 24.391 [13], if the UE is in the non-subscribed SNPN.

For PLMN subscription, the UE may support being configured with the indication whether the USSI is a 3GPP PS data off exempt service using the following method:

1)	the USSI_exempt_non_subscribed_SNPN node of 3GPP TS 24.391 [13], if the UE is in the non-subscribed SNPN.

When the UE is only configured with the indication valid for the UE camping in the subscribed SNPN, the UE shall use this indication also when the UE is in the non-subscribed SNPN.

***** change *****

### E.2.2.1	3GPP PS data off

The requirements in subclause C.2.2.1 apply.

Additionally, the following requirements apply.

When serving a UE in an SNPN using an entry of "list of subscriber data", an USSI AS supporting the 3GPP PS data off can be configured with up to two values of whether USSI is a 3GPP PS data off exempt service, one value is valid for the UE camping in the subscribed SNPN, and the other value is valid for the UE camping in the non-subscribed SNPNs with whom service agreements exist.

When the USSI AS is only configured with the indication whether USSI is a 3GP PS data off exempt service for the UE camping in the subscribed SNPN, the USSI AS shall use this indication also when the UE is in the non-subscribed SNPN.

When serving a UE in an SNPN using PLMN subscription, an USSI AS supporting the 3GPP PS data off can be configured with a value of whether USSI is a 3GPP PS data off exempt service, valid for the UE camping in the non-subscribed SNPNs with whom service agreements exist.