3GPP TSG-CT WG1 Meeting #135-e	C1-222543

# E-Meeting, 6th – 12th April 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.587 | CR | 0234 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requesting V2XP, ProSeP or both during registration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC_Ph2, 5G_ProSe |  |  |  |  |  | Date: |  |  | 2022-04-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | If the UE includes the UE POLICY PROVISIONING REQUEST message (with the requested UE policies) in the REGISTRATION REQUEST message, retransmission mechanisms specified for the REGISTRATION REQUEST message ensure that the REGISTRATION REQUEST message (carrying the UE POLICY PROVISIONING REQUEST message) is reliably transported to the AMF. Thus, there is no need to retransmit the UE POLICY PROVISIONING REQUEST message transported in REGISTRATION REQUEST message. However, total duration of the procedure needs to be adjusted to maximum duration of the registration procedure + some time for PCF to react. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If the UE includes the UE POLICY PROVISIONING REQUEST message (with the requested UE policies) in the REGISTRATION REQUEST message, the UE POLICY PROVISIONING REQUEST message is not retransmitted, the total duration of the procedure is adjusted to maximum duration of the registration procedure + some time for PCF to react. If there is no response to the UE POLICY PROVISIONING REQUEST message transported in the REGISTRATION REQUEST message, the UE can re-initiate standalone V2X policy provisioning request procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE will try to re-transmit the UE POLICY PROVISIONING REQUEST during registration procedure which cannot be transported as the registration procedure is ongoing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.6, 10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** change *****

#### 5.3.2.6	Abnormal cases on the UE

The following abnormal cases can be identified:

a)	T5040 expired.

If the UE POLICY PROVISIONING REQUEST message was transported in the REGISTRATION REQUEST message, the UE shall abort the procedure, release the allocated PTI and may re-initiate UE-requested V2X policy provisioning procedure, if still needed.

If the UE POLICY PROVISIONING REQUEST message was not transported in the REGISTRATION REQUEST message, the UE shall, on the first expiry of the timer T5040, retransmit the UE POLICY PROVISIONING REQUEST message and shall reset and start timer T5040. This retransmission is repeated four times, i.e. on the fifth expiry of timer T5040, the UE shall abort the procedure and release the allocated PTI.

***** change *****

## 10.2	Timers of provisioning of parameters for V2X configuration procedures

Timers of provisioning of parameters for V2X configuration are shown in table 10.2.1.

Table 10.2.1: Timers of provisioning of parameters for V2X configuration – UE side

| TIMER NUM. | TIMER VALUE | CAUSE OF START | NORMAL STOP | ON THE 1st, 2nd, 3rd, 4th EXPIRY |  |
| --- | --- | --- | --- | --- | --- |
| T5040 | NOTE 4 | Transmission of UE POLICY PROVISIONING REQUEST message | MANAGE UE POLICY COMMAND with UE policies for V2X communication or UE POLICY PROVISIONING REJECT message received | NOTE 5 |  |
| T5041 | NOTE 1 | Start using the new UE policies for V2X communication over PC5 received in MANAGE UE POLICY COMMAND message | Stop using the old UE policies for V2X communication over PC5 | Initiate the UE-requested V2X policy provisioning procedure (NOTE 3) |  |
| T5042 | NOTE 2 | Start using the new UE policies for V2X communication over Uu received in MANAGE UE POLICY COMMAND message | Stop using the old UE policies for V2X communication over Uu | Initiate the UE-requested V2X policy provisioning procedure (NOTE 3) |  |
| NOTE 1: The value of this timer is the validity timer value which is one of the configuration parameters for V2X communication over PC5 (see clause 5.2) and it is specified in 3GPP TS 24.588 [7] clause 5.3. NOTE 2: The value of this timer is the validity timer value which is one of the configuration parameters for V2X communication over Uu (see clause 5.2) and it is specified in 3GPP TS 24.588 [7] clause 5.4. NOTE 3: The timers expire only once. NOTE 4: If the UE POLICY PROVISIONING REQUEST message is transported in the REGISTRATION REQUEST message, the value of this timer is the value of timer T3510 as specified in 3GPP TS 24.501 [6] increased with 16s, otherwise the value of this timer is 16s. NOTE 5: If the UE POLICY PROVISIONING REQUEST message is transported in the REGISTRATION REQUEST message, the timer expire only once and expiration of the timer results into procedure being considered as aborted and UE-requested V2X policy provisioning procedure can be re-initiated, if still needed, otherwise expiration of the timer results into retransmission of UE POLICY PROVISIONING REQUEST message. |  |  |  |  |  |