3GPP TSG-CT WG1 Meeting #136-e	C1-22abcd

- E-Meeting, 12th – 20th May 2022

Source:	Huawei, HiSilicon

Title:	Pseudo CR on correction to the Eees_AppContextRelocation service

Spec:	3GPP TS 24.558 v1.3.0

Agenda item:	17.2.16

Document for:	Agreement

1. Introduction

The specification was updated in its latest version with the Eees_AppContextRelocation API but some corrections are needed.

2. Reason for Change

The Eees_AppContextRelocation AP I service needs update to correct a number of things:

(a)	linkage to the Edge Enabler Server API definitions under clause 6;

(b)	use of “<apiVersion>” rather than “v1” as per the rest of the specification;

(c)	keep consistentcy on structure of bullet items in the specification and some editorials.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.558 v1.3.0.

* * * First Change * * * *

#### 5.5.2.1	Introduction

The service operation defined for Eees_AppContextRelocation API is shown in the table 5.5.2.1-1.

Table 5.5.2.1-1: Operations of the Eees_AppContextRelocation API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_AppContextRelocation_Determine | This service operation enables to request ACR determination. | EEC |
| Eees_AppContextRelocation_Initiate | This service operation enables to request ACR initiation. | EEC |

#### 5.5.2.2	Eees_AppContextRelocation_Determine

##### 5.5.2.2.1	General

This service operation is used by an EEC to request ACR determination to the EES.

The following procedures are supported by the "Eees_AppContextRelocation_Determine" service operation:

-	ACR Determination.

##### 5.5.2.2.2	ACR Determination

In order to request ACR determination, the EEC shall send an HTTP POST request to the EES, with the request URI set to "{apiRoot}/eees-appctxtreloc/<apiVersion>/determine" and the request body including the ACRDetermReq data structure that shall contain the necessary information to enable the EES to carry out ACR determination as described in clause 6.5.5.2.2.

Upon receiving the HTTP POST message from the EEC, the EES shall:

a)	 process the ACR determination request;

b)	verify if the EEC is authorized to request ACR determination at the EES;

c)	if the EEC is authorized to request ACR determination with the EES, then:

1)	the S-EES determines the T-EES via the Discover T-EAS procedure and may notify the EEC with target information and/or ACR result notification;

Upon success, the EES responds with an HTTP "204 No Content" status code.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body.

#### 5.5.2.3	Eees_AppContextRelocation_Initiate

##### 5.5.2.3.1	General

This service operation is used by an EEC to request ACR initiation to the EES.

The following procedures are supported by the "Eees_AppContextRelocation_Initiate" service operation:

-	ACR Initiation.

##### 5.5.2.3.2	ACR Initiation

In order to request ACR initiation, the EEC shall send an HTTP POST request to the EES, with the request URI set to "{apiRoot}/eees-appctxtreloc/<apiVersion>/initiate" and the request body including the ACRInitReq data structure that shall contain the necessary information to enable the EES to carry out ACR initiation as described in clause 6.5.5.2.3.

Upon receiving the HTTP POST message from the EEC, the EES shall:

a)	process the ACR initiation request information;

b)	verify if the EEC is authorized to request ACR initiation at the EES and;

c)	if the EEC is authorized to request ACR initiation with the EES, then;

1)	if T-EAS routing information (i.e. N6 routing information) as specified in table 6.5.5.2.3-1 is included in HTTP POST message:

i)	the EES may apply AF traffic influence with the N6 routing information in the 3GPP Core Network as specified in clause 4.4.7 of 3GPP TS 29.522 [r29522];

2)	if EAS notification indication as specified in table 6.5.5.2.3-1 is included in the HTTP POST message, the EES shall notify the EAS to start the ACR towards the T-EAS;

3)	if EEC context relocation details as specified in table 6.5.5.2.3-1 is included in HTTP POST message, then:

i)	if the T-EES is different than the current EES, then the EES shall initiate EEC Context Push towards the T-EES as specified in clause 5.11 of 3GPP TS 29.558 [4]; or

ii)	if the EEC context ID and the S-EES Endpoint are included, then EES shall initiate EEC Context Pull (using EEC Context ID) towards the S-EES as specified in clause 5.10 of 3GPP TS 29.558 [4];

iii)	if Previous T-EAS Endpoint is included in HTTP POST message, then:

A)	if the previous EAS notification indication is included in the HTTP POST message, the EES shall notify the cancellation of the ACR to the EAS;

4)	the EES shall return the response message.

Upon success, the EES responds with an HTTP "204 No Content" status code.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body.

* * * End of Change * * * *