3GPP TSG-CT WG1 Meeting #150	C1-244224

# Maastricht, Netherlands, 19-23 August 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.526 | CR | 0281 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove the NOTE in an inappropriate place |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GProtoc19 |  |  |  |  |  | Date: |  |  | 2024-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In table 5.2.1, "NOTE 17: This connection capability identifier can be used for either a network connection with certain capabilities or a traffic category specified in GSMA PRD NG.135 [26]" is introduced by C1-237787 to clarify related usage for "IMS" and "Internet". Therefore, the NOTE 17 placed as follows in table 5.2.1 may be interpreted as NOTE 17 applied for all the connection capability, which is not true. Therefore, this NOTE 17 should be removed. "For "connection capabilities type”, the traffic descriptor component value field shall be encoded as a sequence of one octet for number of connection capabilities followed by one or more octets, each containing a connection capability identifier encoded as follows (NOTE 17)" |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove "NOTE 17" from the statement mentioned above. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A NOTE in an inappropriate place may be misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * 1st Change * * * *

5.2	Encoding of UE policy part type URSP

The UE policy part type URSP contains one or more URSP rules which may be included in the UE policy part contents as defined in annex D.6.2 of 3GPP TS 24.501 [11].

If the UE policy part contents includes one or more URSP rules (i.e. the UE policy part type field is set to "URSP"), the UE policy part contents including URSP rules is encoded as shown in figures 5.2.1 to 5.2.4A and table 5.2.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| URSP rule 1 |  |  |  |  |  |  |  | octet q+3 octet s |
| URSP rule 2 |  |  |  |  |  |  |  | octet (s+1)* octet t* |
| … |  |  |  |  |  |  |  | octet (t+1)* octet u* |
| URSP rule n |  |  |  |  |  |  |  | octet (u+1)* octet r* |

Figure 5.2.1: UE policy part contents including one or more URSP rules

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of URSP rule |  |  |  |  |  |  |  |  | octet v octet v+1 |  |
| Precedence value of URSP rule |  |  |  |  |  |  |  |  | octet v+2 |  |
| Length of traffic descriptor |  |  |  |  |  |  |  |  | octet v+3 octet v+4 |  |
| Traffic descriptor |  |  |  |  |  |  |  |  | octet v+5 octet w |  |
| Length of route selection descriptor list |  |  |  |  |  |  |  |  | octet w+1 octet w+2 |  |
| Route selection descriptor list |  |  |  |  |  |  |  |  | octet w+3 octet x |  |
| Additional indications |  |  |  |  |  |  |  |  | octet (x+1)* |  |

Figure 5.2.2: URSP rule

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Route selection descriptor 1 |  |  |  |  |  |  |  | octet w+3 octet y |
| Route selection descriptor 2 |  |  |  |  |  |  |  | octet (y+1)* octet z* |
| … |  |  |  |  |  |  |  | octet (z+1)* octet a* |
| Route selection descriptor m |  |  |  |  |  |  |  | octet (a+1)* octet x* |

Figure 5.2.3: Route selection descriptor list

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of route selection descriptor |  |  |  |  |  |  |  | octet b octet b+1 |
| Precedence value of route selection descriptor |  |  |  |  |  |  |  | octet b+2 |
| Length of route selection descriptor contents |  |  |  |  |  |  |  | octet b+3 octet b+4 |
| Route selection descriptor contents |  |  |  |  |  |  |  | octet b+5 octet c |

Figure 5.2.4: Route selection descriptor

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | URERI |  | octet x+1 |  |

Figure 5.2.4A: Additional indications

Table 5.2.1: UE policy part contents including a URSP rule

| Precedence value of URSP rule (octet v+2) The precedence value of URSP rule field is used to specify the precedence of the URSP rule among all URSP rules in the URSP. This field includes the binary encoded value of the precedence value in the range from 0 to 255 (decimal). The higher the value of the precedence value field, the lower the precedence of the URP rule is. Multiple URSP rules in the URSP shall not have the same precedence value (NOTE 10). |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic descriptor (octets v+5 to w) The traffic descriptor field is of variable size and contains a variable number (at least one) of traffic descriptor components. Each traffic descriptor component shall be encoded as a sequence of one octet traffic descriptor component type identifier and a traffic descriptor component value field. The traffic descriptor component type identifier shall be transmitted first. |  |  |  |  |  |  |  |  |  |  |
| Traffic descriptor component type identifier Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 1 Match-all type 0 0 0 0 1 0 0 0 OS Id + OS App Id type (NOTE 1)(NOTE 3) 0 0 0 1 0 0 0 0 IPv4 remote address type 0 0 1 0 0 0 0 1 IPv6 remote address/prefix length type 0 0 1 1 0 0 0 0 Protocol identifier/next header type 0 1 0 1 0 0 0 0 Single remote port type (NOTE 6) 0 1 0 1 0 0 0 1 Remote port range type (NOTE 6) 0 1 0 1 0 0 1 0 IP 3 tuple type 0 1 1 0 0 0 0 0 Security parameter index type 0 1 1 1 0 0 0 0 Type of service/traffic class type 1 0 0 0 0 0 0 0 Flow label type 1 0 0 0 0 0 0 1 Destination MAC address type (NOTE 7) 1 0 0 0 0 0 1 1 802.1Q C-TAG VID type (NOTE 4) 1 0 0 0 0 1 0 0 802.1Q S-TAG VID type (NOTE 4) 1 0 0 0 0 1 0 1 802.1Q C-TAG PCP/DEI type (NOTE 4) 1 0 0 0 0 1 1 0 802.1Q S-TAG PCP/DEI type (NOTE 4) 1 0 0 0 0 1 1 1 Ethertype type 1 0 0 0 1 0 0 0 DNN type (NOTE 3) 1 0 0 1 0 0 0 0 Connection capabilities type (NOTE 3) 1 0 0 1 0 0 0 1 Destination FQDN type 1 0 0 1 0 0 1 0 Regular expression type 1 0 1 0 0 0 0 0 OS App Id type (NOTE 3) 1 0 1 0 0 0 0 1 Destination MAC address range type (NOTE 7) 1 0 1 0 0 0 1 0 PIN ID type (NOTE 8) 1 0 1 0 0 0 1 1 Connectivity group ID type (NOTE 13) All other values are spare. If received they shall be interpreted as unknown. |  |  |  |  |  |  |  |  |  |  |
| For "match-all type", the traffic descriptor component shall not include the traffic descriptor component value field. The "match-all type" traffic descriptor component shall not appear more than once among all traffic descriptors of the whole URSP rules in the URSP. If the "match-all type" traffic descriptor component is included in a traffic descriptor, there shall be no traffic descriptor component with a type other than "match-all type" in the traffic descriptor. |  |  |  |  |  |  |  |  |  |  |
| For "OS Id + OS App Id type", the traffic descriptor component value field shall be encoded as a sequence of a sixteen octet OS Id field, a one octet OS App Id length field, and an OS App Id field. The OS Id field shall be transmitted first. The OS Id field contains a Universally Unique IDentifier (UUID) as specified in IETF RFC 4122 [16]. |  |  |  |  |  |  |  |  |  |  |
| For "IPv4 remote address type", the traffic descriptor component value field shall be encoded as a sequence of a four octet IPv4 address field and a four octet IPv4 address mask field. The IPv4 address field shall be transmitted first. |  |  |  |  |  |  |  |  |  |  |
| For "IPv6 remote address/prefix length type", the traffic descriptor component value field shall be encoded as a sequence of a sixteen octet IPv6 address field and one octet prefix length field. The IPv6 address field shall be transmitted first. |  |  |  |  |  |  |  |  |  |  |
| For "protocol identifier/next header type", the traffic descriptor component value field shall be encoded as one octet which specifies the IPv4 protocol identifier or IPv6 next header. |  |  |  |  |  |  |  |  |  |  |
| For "single remote port type", the traffic descriptor component value field shall be encoded as two octets which specify a port number. |  |  |  |  |  |  |  |  |  |  |
| For "remote port range type", the traffic descriptor component value field shall be encoded as a sequence of a two octet port range low limit field and a two octet port range high limit field. The port range low limit field shall be transmitted first. |  |  |  |  |  |  |  |  |  |  |
| For "IP 3 tuple type", the traffic descriptor component value field shall be encoded as a sequence of a one octet IP 3 tuple information bitmap field where: - bit 1 set to zero indicates that the IPv4 address field is absent; - bit 1 set to one indicates that the IPv4 address field is present; - bit 2 set to zero indicates that the IPv6 remote address/prefix length field is absent; - bit 2 set to one indicates that the IPv6 remote address/prefix length field is present; - bit 3 set to zero indicates that the protocol identifier/next header field is absent; - bit 3 set to one indicates that the protocol identifier/next header field is present; - bit 4 set to zero indicates that the single remote port field is absent; - bit 4 set to one indicates that the single remote port field is present; - bit 5 set to zero indicates that the remote port range field is absent; - bit 5 set to one indicates that the remote port range field is present; and - bits 6,7, and 8 are spare bits; followed by a four octet IPv4 address field and a four octet IPv4 address mask field, if the IPv4 address field is present; followed by a sixteen octet IPv6 address field and one octet prefix length field, if the IPv6 remote address/prefix length field is present; followed by one octet which specifies the IPv4 protocol identifier or IPv6 next header, if the protocol identifier/next header field is present; followed by two octets which specify a port number, if the single remote port field is present; followed by a two octet port range low limit field and a two octet port range high limit field, if the remote port range field is present. The IP 3 tuple information bitmap field shall be transmitted first. The traffic descriptor component value field shall not contain both the IPv4 address field and the IPv6 remote address/prefix length field. If the traffic descriptor component value field contains both the IPv4 address field and the IPv6 remote address/prefix length field, the receiving entity shall ignore the URSP rule. The traffic descriptor component value field shall not contain both the single remote port field and the remote port range field. If the traffic descriptor component value field contains both the single remote port field and the remote port range field, the receiving entity shall ignore the URSP rule. The traffic descriptor component value field shall contain at least one of the IPv4 address field, IPv6 remote address/prefix length field, the protocol identifier/next header field, the single remote port field and the remote port range field, otherwise the receiving entity shall ignore the URSP rule. |  |  |  |  |  |  |  |  |  |  |  |
| For "security parameter index type", the traffic descriptor component value field shall be encoded as four octets which specify the IPsec security parameter index. |  |  |  |  |  |  |  |  |  |  |
| For "type of service/traffic class type", the traffic descriptor component value field shall be encoded as a sequence of a one octet type-of-service/traffic class field and a one octet type-of-service/traffic class mask field. The type-of-service/traffic class field shall be transmitted first. |  |  |  |  |  |  |  |  |  |  |
| For "flow label type", the traffic descriptor component value field shall be encoded as three octets which specify the IPv6 flow label. The bits 8 through 5 of the first octet shall be spare whereas the remaining 20 bits shall contain the IPv6 flow label. |  |  |  |  |  |  |  |  |  |  |
| For "destination MAC address type", the traffic descriptor component value field shall be encoded as 6 octets which specify a MAC address. |  |  |  |  |  |  |  |  |  |  |
| For "802.1Q C-TAG VID type", the traffic descriptor component value field shall be encoded as two octets which specify the VID of the customer-VLAN tag (C-TAG) as specified in IEEE Std 802.1Q [20]. The bits 8 through 5 of the first octet shall be spare whereas the remaining 12 bits shall contain the VID. |  |  |  |  |  |  |  |  |  |  |
| For "802.1Q S-TAG VID type", the traffic descriptor component value field shall be encoded as two octets which specify the VID of the service-VLAN tag (S-TAG) as specified in IEEE Std 802.1Q [20]. The bits 8 through 5 of the first octet shall be spare whereas the remaining 12 bits shall contain the VID. |  |  |  |  |  |  |  |  |  |  |
| For "802.1Q C-TAG PCP/DEI type", the traffic descriptor component value field shall be encoded as one octet which specifies the 802.1Q C-TAG PCP and DEI as specified in IEEE Std 802.1Q [20]. The bits 8 through 5 of the octet shall be spare, and the bits 4 through 2 contain the PCP and bit 1 contains the DEI. |  |  |  |  |  |  |  |  |  |  |
| For "802.1Q S-TAG PCP/DEI type", the traffic descriptor component value field shall be encoded as one octet which specifies the 802.1Q S-TAG PCP as specified in IEEE Std 802.1Q [20]. The bits 8 through 5 of the octet shall be spare, and the bits 4 through 2 contain the PCP and bit 1 contains the DEI. |  |  |  |  |  |  |  |  |  |  |
| For "ethertype type", the traffic descriptor component value field shall be encoded as two octets which specify an ethertype. |  |  |  |  |  |  |  |  |  |  |
| For "DNN type", the traffic descriptor component value field shall be encoded as a sequence of a one octet DNN length field and a DNN value field of a variable size. The DNN value contains an APN as defined in 3GPP TS 23.003 [4]. |  |  |  |  |  |  |  |  |  |  |
| For "connection capabilities type”, the traffic descriptor component value field shall be encoded as a sequence of one octet for number of connection capabilities followed by one or more octets, each containing a connection capability identifier encoded as follows: |  |  |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |  |  |
| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | IMS (NOTE 17) |
| 0 |  | 0 | 0 | 0 | 0 | 0 | 1 | 0 |  | MMS |
| 0 |  | 0 | 0 | 0 | 0 | 1 | 0 | 0 |  | SUPL |
| 0 |  | 0 | 0 | 0 | 1 | 0 | 0 | 0 |  | Internet (NOTE 17) |
| 0 |  | 0 | 0 | 1 | 0 | 0 | 0 | 0 |  | LCS user plane positioning (NOTE 16) |  |
| 0 |  | 0 | 1 | 0 | 0 | 0 | 0 | 0 |  |  |
| to |  |  |  |  |  |  |  |  |  | Operator specific connection capabilities (NOTE 15) |
| 1 |  | 0 | 1 | 0 | 0 | 0 | 0 | 0 |  |  |
| 1 |  | 0 | 1 | 0 | 0 | 0 | 0 | 1 |  | IoT delay-tolerant (NOTE 14) |
| 1 |  | 0 | 1 | 0 | 0 | 0 | 1 | 0 |  | IoT non-delay-tolerant (NOTE 14) |
| 1 |  | 0 | 1 | 0 | 0 | 0 | 1 | 1 |  | Downlink streaming (NOTE 14) |
| 1 |  | 0 | 1 | 0 | 0 | 1 | 0 | 0 |  | Uplink streaming (NOTE 14) |
| 1 |  | 0 | 1 | 0 | 0 | 1 | 0 | 1 |  | Vehicular communications (NOTE 14) |
| 1 |  | 0 | 1 | 0 | 0 | 1 | 1 | 0 |  | Real time interactive (NOTE 14) |
| 1 |  | 0 | 1 | 0 | 0 | 1 | 1 | 1 |  | Unified communications (NOTE 14) |
| 1 |  | 0 | 1 | 0 | 1 | 0 | 0 | 0 |  | Background (NOTE 14) |
| 1 |  | 0 | 1 | 0 | 1 | 0 | 0 | 1 |  | Mission critical communications (NOTE 14) |
| 1 |  | 0 | 1 | 0 | 1 | 0 | 1 | 0 |  | Time critical communications (NOTE 14) |
| 1 |  | 0 | 1 | 0 | 1 | 0 | 1 | 1 |  | Low latency loss tolerant communications in un-acknowledged mode (NOTE 14) |
| All other values are spare. If received, they shall be interpreted as unknown. |  |  |  |  |  |  |  |  |  |  |
| For "destination FQDN type”, the traffic descriptor component value field shall be encoded as a sequence of one octet destination FQDN length field and a destination FQDN value of variable size. The destination FQDN value field shall be encoded as defined in clause 28.3.2.1 in 3GPP TS 23.003 [4]. For "regular expression type”, the traffic descriptor component value field shall be encoded as a sequence of one octet regular expression length field and a regular expression value of variable size. The regular expression value field shall take the form of Extended Regular Expressions (ERE) as defined in chapter 9 in IEEE 1003.1 [19]. |  |  |  |  |  |  |  |  |  |  |
| For "OS App Id type", the traffic descriptor component value field shall be encoded as a one octet OS App Id length field and an OS App Id field. For "destination MAC address range type", the traffic descriptor component value field shall be encoded as a sequence of a 6 octet destination MAC address range low limit field and a 6 octet destination MAC address range high limit field. The destination MAC address range low limit field shall be transmitted first. For "PIN ID type", the traffic descriptor component value field shall be encoded as a sequence of a one octet PIN ID value length field and a PIN ID value field of a variable size octet string. For "connectivity group ID type", the traffic descriptor component value field shall be encoded as a sequence of one octet connectivity group ID length field and a connectivity group ID value field of a variable size which represents a specific connectivity group configured in the 5G-RG. |  |  |  |  |  |  |  |  |  |  |
| Precedence value of route selection descriptor (octet b+2) The precedence value of route selection descriptor field is used to specify the precedence of the route selection descriptor among all route selection descriptors in the URSP rule. This field includes the binary encoded value of the precedence value in the range from 0 to 255 (decimal). The higher the value of the precedence value field, the lower the precedence of the route selection descriptor is. |  |  |  |  |  |  |  |  |  |  |
| Route selection descriptor contents (octets b+5 to c) The route selection descriptor contents field is of variable size and contains a variable number (at least one) of route selection descriptor components. Each route selection descriptor component shall be encoded as a sequence of a one octet route selection descriptor component type identifier and a route selection descriptor component value field. The route selection descriptor component type identifier shall be transmitted first. |  |  |  |  |  |  |  |  |  |  |
| Route selection descriptor component type identifier Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 1 SSC mode type 0 0 0 0 0 0 1 0 S-NSSAI type 0 0 0 0 0 1 0 0 DNN type 0 0 0 0 1 0 0 0 PDU session type type 0 0 0 1 0 0 0 0 Preferred access type type (NOTE 2) 0 0 0 1 0 0 0 1 Multi-access preference type (NOTE 2) 0 0 1 0 0 0 0 0 Non-seamless non-3GPP offload indication type (NOTE 9) 0 1 0 0 0 0 0 0 Location criteria type 1 0 0 0 0 0 0 0 Time window type 1 0 0 0 0 0 0 1 5G ProSe layer-3 UE-to-network relay offload indication type (NOTE 9) 1 0 0 0 0 0 1 0 PDU session pair ID type (NOTE 5, NOTE 9) 1 0 0 0 0 0 1 1 RSN type (NOTE 5, NOTE 9) 1 0 0 0 0 1 0 0 5G ProSe multi-path preference type (NOTE 9) All other values are spare. If received they shall be interpreted as unknown. |  |  |  |  |  |  |  |  |  |  |
| For "SSC mode type", the route selection descriptor component value field shall be encoded as a one octet SSC mode field. The bits 8 through 4 of the octet shall be spare, and the bits 3 through 1 shall be encoded as the value part of the SSC mode information element defined in clause 9.11.4.16 of 3GPP TS 24.501 [11]. The "SSC mode type" route selection descriptor component shall not appear more than once in the route selection descriptor. |  |  |  |  |  |  |  |  |  |  |
| For "S-NSSAI type", the route selection descriptor component value field shall be encoded as a sequence of a one octet S-NSSAI length field and an S-NSSAI value field of a variable size. The S-NSSAI value shall be encoded as the value part of the S-NSSAI information element defined in clause 9.11.2.8 of 3GPP TS 24.501 [11], without the mapped HPLMN SST field and without the mapped HPLMN SD field. |  |  |  |  |  |  |  |  |  |  |
| For "DNN type", the route selection descriptor component value field shall be encoded as a sequence of a one octet DNN length field and a DNN value field of a variable size. The DNN value contains an APN as defined in 3GPP TS 23.003 [4]. |  |  |  |  |  |  |  |  |  |  |
| For "PDU session type type", the route selection descriptor component value field shall be encoded as a one octet PDU session type field. The bits 8 through 4 of the octet shall be spare, and the bits 3 through 1 shall be encoded as the value part of the PDU session type information element defined in clause 9.11.4.11 of 3GPP TS 24.501 [11]. The "PDU session type type" route selection descriptor component shall not appear more than once in the route selection descriptor. |  |  |  |  |  |  |  |  |  |  |
| For "preferred access type type", the route selection descriptor component value field shall be encoded as a one octet preferred access type field. The bits 8 through 3 shall be spare, and the bits 2 and 1 shall be encoded as the value part of the access type information element defined in clause 9.11.2.1A of 3GPP TS 24.501 [11]. The "preferred access type type" route selection descriptor component shall not appear more than once in the route selection descriptor. |  |  |  |  |  |  |  |  |  |  |
| For "multi-access preference type", the route selection descriptor component value field shall be of zero length. The "multi-access preference type" route selection descriptor component shall not appear more than once in the route selection descriptor. The "multi-access preference type" route selection descriptor component in the route selection descriptor indicates the multi-access preference. |  |  |  |  |  |  |  |  |  |  |
| For "non-seamless non-3GPP offload indication type", the route selection descriptor component shall not include the route selection descriptor component value field. The "non-seamless non-3GPP offload indication type" route selection descriptor component shall not appear more than once in the route selection descriptor. If the "non-seamless non-3GPP offload indication type" route selection descriptor component is included in a route selection descriptor, there shall be no route selection descriptor component with a type other than "non-seamless non-3GPP offload indication type" in the route selection descriptor. |  |  |  |  |  |  |  |  |  |  |
| For "location criteria type", the route selection descriptor component value field may contain one or more types of location area and is encoded as shown in Figure 5.2.5 and Table 5.2.2. |  |  |  |  |  |  |  |  |  |  |  |
| For "time window type", the route selection descriptor component value field shall be encoded as a sequence of a Starttime field and a Stoptime field. The Starttime field is represented by the number of seconds since 00:00:00 on 1 January 1970 and is encoded as the 64-bit NTP timestamp format defined in IETF RFC 5905 [17], where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. The encoding of the Stoptime field is the same as the Starttime field. |  |  |  |  |  |  |  |  |  |  |  |
| For "5G ProSe layer-3 UE-to-network relay offload indication type", the route selection descriptor component shall not include the route selection descriptor component value field. The "5G ProSe layer-3 UE-to-network relay offload indication type" route selection descriptor component shall not appear more than once in the route selection descriptor. If the "5G ProSe layer-3 UE-to-network relay offload indication type" route selection descriptor component is included in a route selection descriptor, there shall be no route selection descriptor component with a type other than "5G ProSe layer-3 UE-to-network relay offload indication type" in the route selection descriptor. If neither "5G ProSe layer-3 UE-to-network relay offload indication type" nor "5G ProSe multi-path preference” is present, the traffic shall not be routed via a 5G ProSe layer-3 UE-to-network relay outside of a PDU Session. |  |  |  |  |  |  |  |  |  |  |  |
| For "PDU session pair ID type", the route selection descriptor component value field shall be encoded as a one octet PDU session pair ID field. The PDU session pair ID value shall be encoded as defined in clause 9.11.4.32 of 3GPP TS 24.501 [11]. |  |  |  |  |  |  |  |  |  |  |  |  |
| For "RSN type", the route selection descriptor component value field shall be encoded as a one octet RSN field. The RSN value shall be encoded as the value part of the RSN information element defined in clause 9.11.4.33 of 3GPP TS 24.501 [11]. For "5G ProSe multi-path preference type", the route selection descriptor component shall not include the route selection descriptor component value field. The "5G ProSe multi-path preference" route selection descriptor component shall not appear more than once in the route selection descriptor. If the "5G ProSe multi-path preference" route selection descriptor component is included in a route selection descriptor, there shall be no route selection descriptor component with type "5G ProSe layer-3 UE-to-network relay offload indication type" in the route selection descriptor. If neither "5G ProSe layer-3 UE-to-network relay offload indication type" nor "5G ProSe multi-path preference type” is present, the traffic shall not be routed via a 5G ProSe layer-3 UE-to-network relay outside of a PDU Session. Additional indications (octet x+1) (NOTE 11) Additional indications field is used to indicate additional indications related to the URSP rule, and is encoded as shown in Figure 5.2.4A. . URSP rule enforcement report indication (URERI) (bit 1 of octet x+1) URSP rule enforcement report indication indicates whether to report the URSP rule enforcement for the associated URSP rule according to clause 4.2.4 (NOTE 12). Bit 1 0 URSP rule enforcement report is not required 1 URSP rule enforcement report is required Bits 2 to 8 of octet x+1 are spare and shall be encoded as zero. If received they shall be interpreted as unknown. |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: For "OS Id + OS App Id type", the traffic descriptor component value field does not specify the OS version number or the version number of the application. NOTE 2: The PCF does not include both the "preferred access type type" and the "multi-access preference type" route selection descriptor components in a single route selection descriptor. If there are both "preferred access type type" and "multi-access preference type" route selection descriptor components in a single route selection descriptor, the UE ignores the "preferred access type type" route selection descriptor component. NOTE 3: The W-AGF acting on behalf of the FN-RG shall interpret the value as unknown. NOTE 4: The traffic descriptor of a URSP rule cannot include more than one instance of this traffic component type. NOTE 5: Redundant PDU session is not applicable over non-3GPP access. The UE ignores any route selection descriptor which includes "PDU session pair ID type" or "RSN type" route selection descriptor component and also includes a "preferred access type type" route selection descriptor component set to "non-3GPP access" or a "multi-access preference type" route selection descriptor component. NOTE 6: The traffic descriptor of a URSP rule shall not contain both the “Single remote port type” and the “Remote port range type” traffic descriptor components. If the traffic descriptor of a URSP rule contains both the “Single remote port type” and the “Remote port range type” traffic descriptor components, the receiving entity shall ignore the URSP rule. NOTE 7: The traffic descriptor of a URSP rule shall not contain both the “Destination MAC address type” and the “Destination MAC address range type” traffic descriptor components. If the traffic descriptor of a URSP rule contains both the “Destination MAC address type” and the “Destination MAC address range type” traffic descriptor components, the receiving entity shall ignore the URSP rule. NOTE 8: The traffic descriptor component type "PIN ID type" shall be mutually exclusive to the other traffic descriptor components, i.e., if the traffic descriptor of the URSP rule contains both PIN ID and the other traffic descriptor components, the UE shall ignore the other traffic descriptor component and shall use the PIN ID. The PIN ID uniquely identifies a PIN in the PLMN. The PIN ID is only applicable for the traffic of a PINE. NOTE 9: Not applicable for PIN traffic. NOTE 10: The precedence value of a URSP rule in a UE policy section associated with network descriptor entry type "one or more VPLMNs" in VPS URSP configuration should be lower than the precedence value of a URSP rule in a UE policy section associated with network descriptor entry type "one or more MCCs" or "any VPLMN" in VPS URSP configuration. NOTE 11: Additional indications field is included when the URSP rule includes an octet following octet x. NOTE 12: A URSP rule can contain this indication only if the URSP rule includes one or more connection capabilities in traffic descriptor component. NOTE 13: The traffic descriptor component type "connectivity group ID" can only be combined with IP descriptors, non-IP descriptors, or both, in the same URSP rule. if the traffic descriptor of the URSP rule contains connectivity group ID and any other traffic descriptor components other than IP descriptors or non-IP descriptors, the receiving entity shall ignore the other traffic descriptor component. NOTE 14: These connection capabilities are traffic categories specified in GSMA PRD NG.135 [26]. NOTE 15: The operator specific connection capability identifier can match against a operator specific traffic category as specified in 3GPP TS 23.503 [2]. NOTE 16: This indication is included in the connection capability as specified in 3GPP TS 23.273 [27]. NOTE 17: This connection capability identifier can be used for either a network connection with certain capabilities or a traffic category specified in GSMA PRD NG.135 [26]. |  |  |  |  |  |  |  |  |  |  |

* * * End of Changes * * * *