3GPP TSG-CT WG1 Meeting #154	C1-252519

# Wuhan, China, 07-11 April 2025																(was C1-252198)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 4349 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the WUS assistance information IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | SAES19 |  |  |  |  |  | Date: |  |  | 2025-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According the usage requirement for WUS assistance information, there is only one type of information used by single direction (UL or DL), so the WUS assistance information IE should only have one WUS assistance information type entry. So it proposes to delete redundant WUS assistance information type entries. In addition, the length of WUS assistance information IE in NAS message should be “3” rather than “3-n”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The maximum length of WUS assistance information IE is changed to 3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The IE coding does not align with usage requirement, which can add implementation or result in wrong implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1, 8.2.4.1, 8.2.26.1, 8.2.29.1, 9.9.3.62 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

8.2.1.1	Message definition

This message is sent by the network to the UE to indicate that the corresponding attach request has been accepted. See table 8.2.1.1.

Message type:	ATTACH ACCEPT

Significance:	dual

Direction:	network to UE

Table 8.2.1.1: ATTACH ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator | Protocol discriminator 9.2 | M | V | 1/2 |
|  | Security header type | Security header type 9.3.1 | M | V | 1/2 |
|  | Attach accept message identity | Message type 9.8 | M | V | 1 |
|  | EPS attach result | EPS attach result 9.9.3.10 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.9.2.9 | M | V | 1/2 |
|  | T3412 value | GPRS timer 9.9.3.16 | M | V | 1 |
|  | TAI list | Tracking area identity list 9.9.3.33 | M | LV | 7-97 |
|  | ESM message container | ESM message container 9.9.3.15 | M | LV-E | 5-n |
| 50 | GUTI | EPS mobile identity 9.9.3.12 | O | TLV | 13 |
| 13 | Location area identification | Location area identification 9.9.2.2 | O | TV | 6 |
| 23 | MS identity | Mobile identity 9.9.2.3 | O | TLV | 7-10 |
| 53 | EMM cause | EMM cause 9.9.3.9 | O | TV | 2 |
| 17 | T3402 value | GPRS timer 9.9.3.16 | O | TV | 2 |
| 59 | T3423 value | GPRS timer 9.9.3.16 | O | TV | 2 |
| 4A | Equivalent PLMNs | PLMN list 9.9.2.8 | O | TLV | 5-47 |
| 34 | Emergency number list | Emergency number list 9.9.3.37 | O | TLV | 5-50 |
| 64 | EPS network feature support | EPS network feature support 9.9.3.12A | O | TLV | 3-5 |
| F- | Additional update result | Additional update result 9.9.3.0A | O | TV | 1 |
| 5E | T3412 extended value | GPRS timer 3 9.9.3.16B | O | TLV | 3 |
| 6A | T3324 value | GPRS timer 2 9.9.3.16A | O | TLV | 3 |
| 6E | Extended DRX parameters | Extended DRX parameters 9.9.3.46 | O | TLV | 3 |
| 65 | DCN-ID | DCN-ID 9.9.3.48 | O | TLV | 4 |
| E- | SMS services status | SMS services status 9.9.3.4B | O | TV | 1 |
| D- | Non-3GPP NW provided policies | Non-3GPP NW provided policies 9.9.3.49 | O | TV | 1 |
| 6B | T3448 value | GPRS timer 2 9.9.3.16A | O | TLV | 3 |
| C- | Network policy | Network policy 9.9.3.52 | O | TV | 1 |
| 6C | T3447 value | GPRS timer 3 9.9.3.16B | O | TLV | 3 |
| 7A | Extended emergency number list | Extended emergency number list 9.9.3.37A | O | TLV-E | 7-65538 |
| 7C | Ciphering key data | Ciphering key data 9.9.3.56 | O | TLV-E | 35-2291 |
| 66 | UE radio capability ID | UE radio capability ID 9.9.3.60 | O | TLV | 3-n |
| B- | UE radio capability ID deletion indication | UE radio capability ID deletion indication 9.9.3.61 | O | TV | 1 |
| 35 | Negotiated WUS assistance information | WUS assistance information 9.9.3.62 | O | TLV | 3 |
| 36 | Negotiated DRX parameter in NB-S1 mode | NB-S1 DRX parameter 9.9.3.63 | O | TLV | 3 |
| 38 | Negotiated IMSI offset | IMSI offset 9.9.3.64 | O | TLV | 4 |
| 1D | Forbidden TAI(s) for the list of "forbidden tracking areas for roaming" | Tracking area identity list 9.9.3.33 | O | TLV | 8-98 |
| 1E | Forbidden TAI(s) for the list of "forbidden tracking areas for regional provision of service" | Tracking area identity list 9.9.3.33 | O | TLV | 8-98 |
| 1F | Unavailability configuration | Unavailability configuration 9.9.3.70 | O | TLV | 3-9 |
| 20 | Access technology utilization control | Access technology utilization control 9.9.3.3A | O | TLV | 4-5 |
| 21 | S&F satellite operation parameters | S&F satellite operation parameters 9.9.3.73 | O | TLV | 3-257 |

* * * Next Change * * *

8.2.4.1	Message definition

This message is sent by the UE to the network in order to perform an attach procedure. See table 8.2.4.1.

Message type:	ATTACH REQUEST

Significance:	dual

Direction:	UE to network

Table 8.2.4.1: ATTACH REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator | Protocol discriminator 9.2 | M | V | 1/2 |
|  | Security header type | Security header type 9.3.1 | M | V | 1/2 |
|  | Attach request message identity | Message type 9.8 | M | V | 1 |
|  | EPS attach type | EPS attach type 9.9.3.11 | M | V | 1/2 |
|  | NAS key set identifier | NAS key set identifier 9.9.3.21 | M | V | 1/2 |
|  | EPS mobile identity | EPS mobile identity 9.9.3.12 | M | LV | 5-12 |
|  | UE network capability | UE network capability 9.9.3.34 | M | LV | 3-14 |
|  | ESM message container | ESM message container 9.9.3.15 | M | LV-E | 5-n |
| 19 | Old P-TMSI signature | P-TMSI signature 9.9.3.26 | O | TV | 4 |
| 50 | Additional GUTI | EPS mobile identity 9.9.3.12 | O | TLV | 13 |
| 52 | Last visited registered TAI | Tracking area identity 9.9.3.32 | O | TV | 6 |
| 5C | DRX parameter | DRX parameter 9.9.3.8 | O | TV | 3 |
| 31 | MS network capability | MS network capability 9.9.3.20 | O | TLV | 4-10 |
| 13 | Old location area identification | Location area identification 9.9.2.2 | O | TV | 6 |
| 9- | TMSI status | TMSI status 9.9.3.31 | O | TV | 1 |
| 11 | Mobile station classmark 2 | Mobile station classmark 2 9.9.2.4 | O | TLV | 5 |
| 20 | Mobile station classmark 3 | Mobile station classmark 3 9.9.2.5 | O | TLV | 2-34 |
| 40 | Supported Codecs | Supported Codec List 9.9.2.10 | O | TLV | 5-n |
| F- | Additional update type | Additional update type 9.9.3.0B | O | TV | 1 |
| 5D | Voice domain preference and UE's usage setting | Voice domain preference and UE's usage setting 9.9.3.44 | O | TLV | 3 |
| D- | Device properties | Device properties 9.9.2.0A | O | TV | 1 |
| E- | Old GUTI type | GUTI type 9.9.3.45 | O | TV | 1 |
| C- | MS network feature support | MS network feature support 9.9.3.20A | O | TV | 1 |
| 10 | TMSI based NRI container | Network resource identifier container 9.9.3.24A | O | TLV | 4 |
| 6A | T3324 value | GPRS timer 2 9.9.3.16A | O | TLV | 3 |
| 5E | T3412 extended value | GPRS timer 3 9.9.3.16B | O | TLV | 3 |
| 6E | Extended DRX parameters | Extended DRX parameters 9.9.3.46 | O | TLV | 3 |
| 6F | UE additional security capability | UE additional security capability 9.9.3.53 | O | TLV | 6 |
| 6D | UE status | UE status 9.9.3.54 | O | TLV | 3 |
| 17 | Additional information requested | Additional information requested 9.9.3.55 | O | TV | 2 |
| 32 | N1 UE network capability | N1 UE network capability 9.9.3.57 | O | TLV | 3-15 |
| 34 | UE radio capability ID availability | UE radio capability ID availability 9.9.3.58 | O | TLV | 3 |
| 35 | Requested WUS assistance information | WUS assistance information 9.9.3.62 | O | TLV | 3 |
| 36 | DRX parameter in NB-S1 mode | NB-S1 DRX parameter 9.9.3.63 | O | TLV | 3 |
| 38 | Requested IMSI offset | IMSI offset 9.9.3.64 | O | TLV | 4 |

* * * Next Change * * *

8.2.26.1	Message definition

This message is sent by the network to the UE to provide the UE with EPS mobility management related data in response to a tracking area update request message. See table 8.2.26.1.

Message type:	TRACKING AREA UPDATE ACCEPT

Significance:	dual

Direction:	network to UE

Table 8.2.26.1: TRACKING AREA UPDATE ACCEPT message content

| IEI |  | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Protocol discriminator | Protocol discriminator 9.2 | M | V | 1/2 |
|  |  | Security header type | Security header type 9.3.1 | M | V | 1/2 |
|  |  | Tracking area update accept message identity | Message type 9.8 | M | V | 1 |
|  |  | EPS update result | EPS update result 9.9.3.13 | M | V | 1/2 |
|  |  | Spare half octet | Spare half octet 9.9.2.9 | M | V | 1/2 |
| 5A |  | T3412 value | GPRS timer 9.9.3.16 | O | TV | 2 |
| 50 |  | GUTI | EPS mobile identity 9.9.3.12 | O | TLV | 13 |
| 54 |  | TAI list | Tracking area identity list 9.9.3.33 | O | TLV | 8-98 |
| 57 |  | EPS bearer context status | EPS bearer context status 9.9.2.1 | O | TLV | 4 |
| 13 |  | Location area identification | Location area identification 9.9.2.2 | O | TV | 6 |
| 23 |  | MS identity | Mobile identity 9.9.2.3 | O | TLV | 7-10 |
| 53 |  | EMM cause | EMM cause 9.9.3.9 | O | TV | 2 |
| 17 |  | T3402 value | GPRS timer 9.9.3.16 | O | TV | 2 |
| 59 |  | T3423 value | GPRS timer 9.9.3.16 | O | TV | 2 |
| 4A |  | Equivalent PLMNs | PLMN list 9.9.2.8 | O | TLV | 5-47 |
| 34 |  | Emergency number list | Emergency number list 9.9.3.37 | O | TLV | 5-50 |
| 64 |  | EPS network feature support | EPS network feature support 9.9.3.12A | O | TLV | 3-5 |
| F- |  | Additional update result | Additional update result 9.9.3.0A | O | TV | 1 |
| 5E |  | T3412 extended value | GPRS timer 3 9.9.3.16B | O | TLV | 3 |
| 6A |  | T3324 value | GPRS timer 2 9.9.3.16A | O | TLV | 3 |
| 6E |  | Extended DRX parameters | Extended DRX parameters 9.9.3.46 | O | TLV | 3 |
| 68 |  | Header compression configuration status | Header compression configuration status 9.9.4.27 | O | TLV | 4 |
| 65 |  | DCN-ID | DCN-ID 9.9.3.48 | O | TLV | 4 |
| E- |  | SMS services status | SMS services status 9.9.3.4B | O | TV | 1 |
| D- |  | Non-3GPP NW policies | Non-3GPP NW provided policies 9.9.3.49 | O | TV | 1 |
| 6B |  | T3448 value | GPRS timer 2 9.9.3.16A | O | TLV | 3 |
| C- |  | Network policy | Network policy 9.9.3.52 | O | TV | 1 |
| 6C |  | T3447 value | GPRS timer 3 9.9.3.16B | O | TLV | 3 |
| 7A |  | Extended emergency number list | Extended emergency number list 9.9.3.37A | O | TLV-E | 7-65538 |
| 7C |  | Ciphering key data | Ciphering key data 9.9.3.56 | O | TLV-E | 35-2291 |
| 66 |  | UE radio capability ID | UE radio capability ID 9.9.3.60 | O | TLV | 3-n |
| B- |  | UE radio capability ID deletion indication | UE radio capability ID deletion indication 9.9.3.61 | O | TV | 1 |
| 35 |  | Negotiated WUS assistance information | WUS assistance information 9.9.3.62 | O | TLV | 3 |
| 36 |  | Negotiated DRX parameter in NB-S1 mode | NB-S1 DRX parameter 9.9.3.63 | O | TLV | 3 |
| 38 |  | Negotiated IMSI offset | IMSI offset 9.9.3.64 | O | TLV | 4 |
| 37 |  | EPS additional request result | EPS additional request result 9.9.3.67 | O | TLV | 3 |
| 1D |  | Forbidden TAI(s) for the list of "forbidden tracking areas for roaming" | Tracking area identity list 9.9.3.33 | O | TLV | 8-98 |
| 1E |  | Forbidden TAI(s) for the list of "forbidden tracking areas for regional provision of service " | Tracking area identity list 9.9.3.33 | O | TLV | 8-98 |
| 39 |  | Maximum time offset | GPRS timer 3 9.9.3.16B | O | TLV | 3 |
| 1F |  | Unavailability configuration | Unavailability configuration 9.9.3.70 | O | TLV | 3-9 |
| 20 | Access technology utilization control | Access technology utilization control 9.9.3.3A | O | TLV | 4-5 |

* * * Next Change * * *

8.2.29.1	Message definition

The purposes of sending the tracking area update request by the UE to the network are described in clause 5.5.3.1. See table 8.2.29.1.

Message type:	TRACKING AREA UPDATE REQUEST

Significance:	dual

Direction:	UE to network

Table 8.2.29.1: TRACKING AREA UPDATE REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator | Protocol discriminator 9.2 | M | V | 1/2 |
|  | Security header type | Security header type 9.3.1 | M | V | 1/2 |
|  | Tracking area update request message identity | Message type 9.8 | M | V | 1 |
|  | EPS update type | EPS update type 9.9.3.14 | M | V | 1/2 |
|  | NAS key set identifier | NAS key set identifier 9.9.3.21 | M | V | 1/2 |
|  | Old GUTI | EPS mobile identity 9.9.3.12 | M | LV | 12 |
| B- | Non-current native NAS key set identifier | NAS key set identifier 9.9.3.21 | O | TV | 1 |
| 8- | GPRS ciphering key sequence number | Ciphering key sequence number 9.9.3.4a | O | TV | 1 |
| 19 | Old P-TMSI signature | P-TMSI signature 9.9.3.26 | O | TV | 4 |
| 50 | Additional GUTI | EPS mobile identity 9.9.3.12 | O | TLV | 13 |
| 55 | NonceUE | Nonce 9.9.3.25 | O | TV | 5 |
| 58 | UE network capability | UE network capability 9.9.3.34 | O | TLV | 4-15 |
| 52 | Last visited registered TAI | Tracking area identity 9.9.3.32 | O | TV | 6 |
| 5C | DRX parameter | DRX parameter 9.9.3.8 | O | TV | 3 |
| A- | UE radio capability information update needed | UE radio capability information update needed 9.9.3.35 | O | TV | 1 |
| 57 | EPS bearer context status | EPS bearer context status 9.9.2.1 | O | TLV | 4 |
| 31 | MS network capability | MS network capability 9.9.3.20 | O | TLV | 4-10 |
| 13 | Old location area identification | Location area identification 9.9.2.2 | O | TV | 6 |
| 9- | TMSI status | TMSI status 9.9.3.31 | O | TV | 1 |
| 11 | Mobile station classmark 2 | Mobile station classmark 2 9.9.2.4 | O | TLV | 5 |
| 20 | Mobile station classmark 3 | Mobile station classmark 3 9.9.2.5 | O | TLV | 2-34 |
| 40 | Supported Codecs | Supported Codec List 9.9.2.10 | O | TLV | 5-n |
| F- | Additional update type | Additional update type 9.9.3.0B | O | TV | 1 |
| 5D | Voice domain preference and UE's usage setting | Voice domain preference and UE's usage setting 9.9.3.44 | O | TLV | 3 |
| E- | Old GUTI type | GUTI type 9.9.3.45 | O | TV | 1 |
| D- | Device properties | Device properties 9.9.2.0A | O | TV | 1 |
| C- | MS network feature support | MS network feature support 9.9.3.20A | O | TV | 1 |
| 10 | TMSI based NRI container | Network resource identifier container 9.9.3.24A | O | TLV | 4 |
| 6A | T3324 value | GPRS timer 2 9.9.3.16 | O | TLV | 3 |
| 5E | T3412 extended value | GPRS timer 3 9.9.3.16B | O | TLV | 3 |
| 6E | Extended DRX parameters | Extended DRX parameters 9.9.3.46 | O | TLV | 3 |
| 6F | UE additional security capability | UE additional security capability 9.9.3.53 | O | TLV | 6 |
| 6D | UE status | UE status 9.9.3.54 | O | TLV | 3 |
| 17 | Additional information requested | Additional information requested 9.9.3.55 | O | TV | 2 |
| 32 | N1 UE network capability | N1 UE network capability 9.9.3.57 | O | TLV | 3-15 |
| 34 | UE radio capability ID availability | UE radio capability ID availability 9.9.3.58 | O | TLV | 3 |
| 35 | Requested WUS assistance information | WUS assistance information 9.9.3.62 | O | TLV | 3 |
| 36 | DRX parameter in NB-S1 mode | NB-S1 DRX parameter 9.9.3.63 | O | TLV | 3 |
| 38 | Requested IMSI offset | IMSI offset 9.9.3.64 | O | TLV | 4 |
| 29 | UE request type | UE request type 9.9.3.65 | O | TLV | 3 |
| 28 | Paging restriction | Paging restriction 9.9.3.66 | O | TLV | 3-5 |
| 30 | Unavailability information | Unavailability information 9.9.3.69 | O | TLV | 3-9 |

* * * Next Change * * *

9.9.3.62	WUS assistance information

The purpose of the WUS assistance information information element is to transfer the required assistance information to determine the WUS group used when paging the UE.

The WUS assistance information information element is coded as shown in figure 9.9.3.62.1, figure 9.9.3.62.2 and table 9.9.3.62.1.

The WUS assistance information is a type 4 information element, with a length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WUS assistance information IEI |  |  |  |  |  |  |  | octet 1 |
| Length of WUS assistance information contents |  |  |  |  |  |  |  | octet 2 |
| WUS assistance information type |  |  |  |  |  |  |  | octet 3 |

Figure 9.9.3.62.1: WUS assistance information information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of information |  |  | UE paging probability information value |  |  |  |  | octet 1 |

Figure 9.9.3.62.2: WUS assistance information type –type of information= "000"

Table 9.9.3.62.1: WUS assistance information information element

| Value part of the WUS assistance information information element (octet 3) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The value part of the WUS assistance information information element consists of one type of WUS assistance information. |  |  |  |  |  |
| WUS assistance information type: |  |  |  |  |  |
| Type of information (octet 1) |  |  |  |  |  |
| Bits |  |  |  |  |  |
| 8 | 7 | 6 |  |  |  |
| 0 | 0 | 0 | UE paging probability information |  |  |
| All other values are reserved. |  |  |  |  |  |
| UE paging probability information value: |  |  |  |  |  |
| This field contains the value of UE paging probability information requested by the UE or negotiated by the network. It represents the probability of the UE receiving the paging. bit |  |  |  |  |  |
| 5 | 4 | 3 | 2 | 1 | UE paging probability information value |
| 0 | 0 | 0 | 0 | 0 | p00 (UE calculated paging probability is 0%) |
| 0 | 0 | 0 | 0 | 1 | p05 (UE calculated paging probability > 0% and <= 5%) |
| 0 | 0 | 0 | 1 | 0 | p10 (UE calculated paging probability > 5% and <= 10%) |
| 0 | 0 | 0 | 1 | 1 | p15 (UE calculated paging probability > 10% and <= 15%) |
| 0 | 0 | 1 | 0 | 0 | p20 (UE calculated paging probability > 15% and <= 20%) |
| 0 | 0 | 1 | 0 | 1 | p25 (UE calculated paging probability > 20% and <= 25%) |
| 0 | 0 | 1 | 1 | 0 | p30 (UE calculated paging probability > 25% and <= 30%) |
| 0 | 0 | 1 | 1 | 1 | p35 (UE calculated paging probability > 30% and <= 35%) |
| 0 | 1 | 0 | 0 | 0 | p40 (UE calculated paging probability > 35% and <= 40%) |
| 0 | 1 | 0 | 0 | 1 | p45 (UE calculated paging probability > 40% and <= 45%) |
| 0 | 1 | 0 | 1 | 0 | p50 (UE calculated paging probability > 45% and <= 50%) |
| 0 | 1 | 0 | 1 | 1 | p55 (UE calculated paging probability > 50% and <= 55%) |
| 0 | 1 | 1 | 0 | 0 | p60 (UE calculated paging probability > 55% and <= 60%) |
| 0 | 1 | 1 | 0 | 1 | p65 (UE calculated paging probability > 60% and <= 65%) |
| 0 | 1 | 1 | 1 | 0 | p70 (UE calculated paging probability > 65% and <= 70%) |
| 0 | 1 | 1 | 1 | 1 | p75 (UE calculated paging probability > 70% and <= 75%) |
| 1 | 0 | 0 | 0 | 0 | p80 (UE calculated paging probability > 75% and <= 80%) |
| 1 | 0 | 0 | 0 | 1 | p85 (UE calculated paging probability > 80% and <= 85%) |
| 1 | 0 | 0 | 1 | 0 | p90 (UE calculated paging probability > 85% and <= 90%) |
| 1 | 0 | 0 | 1 | 1 | p95 (UE calculated paging probability > 90% and <= 95%) |
| 1 | 0 | 1 | 0 | 0 | p100 (UE calculated paging probability > 95% and <= 100%) |
| All other values shall be interpreted as 10100 by this version of the protocol. |  |  |  |  |  |

* * * End of Change * * *